3GPP TSG-SA WG6 Meeting #47-e	S6-220289

# e-meeting, 14th – 22nd February 2022	                                                 (revision of S6-220165)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0092 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct QoS monitoring service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. The QoS monitoring measurement requirement data are not aligned in subs-notif model and req-resp model. 2. The presence conditions in Table 14.3.2.20-1, 14.3.2.22-1, 14.3.2.33-1 and 14.3.2.34-1 are incorrect. 3. In order to support PLR and traffic data rate monitoring, NWDAF analytic id “DN Performance Analytic” should be utilized by NRM server. 4. For a list of UEs, it is not specified how NRM server aggregates the monitoring report. 5. The default value for measurement requirements in QoS monitoring is not specified. 6. Data details are not specified with Frequency of reporting and termination of reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the examples of monitoring requirements. - Change presence condition for QoS monitoring target. - Add “DN Performance Analytic” for NRM server interaction with NEF. - Clarify NRM server aggegates the report for the list of UEs being requested. - Re-structure the measurement requirements and specify the default value. - Clarify the detailed data associated with Frequency of reporting and termination of reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and incomplete QoS monitoring function in SEAL NRM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.20, 14.3.2.22, 14.3.2.33, 14.3.2.34, 14.3.3.4.1.2, 14.3.3.4.2.2, 14.3.3.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 14.3.2.20	Unicast QoS monitoring subscription request

Table 14.3.2.20-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription request.

Table 14.3.2.20-1: Unicast QoS monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | O | It describes the requirements on the QoS measurements. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates a current time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| Reporting requirements | O | It describes the requirements for QoS monitoring reporting. |
| > Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event triggered (i.e. threshold reached). If absent in the request, default event triggered reporting is used. |
| > Reporting threshold | O | If the Frequency of reporting is event triggered (threshold reached), the reporting threshold corresponding to the measurement index shall be provided. |
| > Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| > Termination of reporting | O | It indicates when the reporting of QoS monitoring shall stop. It can be event triggered (i.e. number of reports reached or threshold reached), when reporting duration expires or when explicit termination is requested. If absent in the request, it defaults to event triggered reporting (threshold reached). |
| > Max number of reports | O | It indicates the maximum number of reports, it shall be provided when Termination of reporting is set to event triggered (number of reports reached). |
| > Termination threshold | O | It indicates the reporting termination threshold corresponding to the measurement index, it shall be provided when Termination of reporting is set to event triggered (threshold reached). |
| > Reporting duration | O | It indicates the reporting duration, it shall be provided when Termination of reporting is set to duration expiration. |
| NOTE: Only one of these information elements shall be present |  |  |

* * * Next Change * * * *

#### 14.3.2.22	Unicast QoS monitoring notification

Table 14.3.2.22-1 describes the information flow for unicast QoS monitoring notification from the network resource management server to the VAL server.

Table 14.3.2.22-1: Notify unicast QoS monitoring event

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UE IDs | O (see NOTE) | List of VAL UEs whose QoS monitoring data is provided. This list is either the list provided in the subscription request or a subset of that list. It can contain a single UE. |
| VAL group ID | O (see NOTE) | The group ID used for the VAL group for which QoS monitoring data is provided. This group ID shall be the same VAL group ID as the VAL group ID provided in the subscription request. |
| List of VAL stream IDs | O (see NOTE) | List of VAL stream IDs whose QoS monitoring data is provided. This list is either the list provided in the subscription request or a subset of that list. It can contain a single stream ID. |
| QoS monitoring data | M | QoS monitoring data is an aggregate of QoS measurements data obtained from the 5GS. |
| NOTE: Only one of these information elements shall be present. |  |  |

* * * Next Change * * * *

#### 14.3.2.33	Unicast QoS monitoring data request

Table 14.3.2.33-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring data request.

Table 14.3.2.33-1: Unicast QoS monitoring data request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | M | It describes the requirements on the QoS measurement data to be provided. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates either a current or past time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| NOTE: Only one of these information elements shall be present. |  |  |

* * * Next Change * * * *

#### 14.3.2.34	Unicast QoS monitoring data response

Table 14.3.2.34-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring data response.

Table 14.3.2.34-1: Unicast QoS monitoring data response

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UE IDs | O (see NOTE 1) | List of VAL UEs whose QoS monitoring data is provided. This list is either the list provided in the request or a subset of that list. It can contain a single UE. |
| VAL group ID | O (see NOTE 1) | The group ID used for the VAL group for which QoS monitoring data is provided. This group ID shall be the same VAL group ID as the VAL group ID provided in the request. |
| List of VAL stream IDs | O (see NOTE 1) | List of VAL stream IDs whose QoS monitoring data is provided. This list is either the list provided in the request or a subset of that list. It can contain a single stream ID. |
| Result | M | Indicates the success or failure for the operation |
| QoS monitoring data | M (see NOTE 2) | QoS monitoring data is an aggregate of QoS measurements data obtained from the 5GS. |
| Cause | O | Provides reason for the failure (e.g. when data for the requested time period is not available). |
| NOTE 1: Only one of these information elements shall be present. NOTE 2: If the Result information element indicates failure then the value of the QoS monitoring data information element has no meaning. |  |  |

* * * Next Change * * * *

###### 14.3.3.4.1.2	Procedure

Figure 14.3.3.4.1.2-1 illustrates the high level procedure for unicast QoS monitoring subscription.

Pre-conditions:

-	The VAL UE has an established connection in the 5GS

Figure 14.3.3.4.1.2-1: Unicast QoS monitoring subscription

1.	The VAL server sends a unicast QoS monitoring subscription request to the NRM server either in conjunction with a request for unicast network resources requiring QoS or when the unicast QoS connection is already established. The NRM server checks if the VAL server is authorized to initiate the unicast QoS monitoring request for the requested target VAL UEs, VAL group ID, or VAL stream IDs.

NOTE:	It is left for stage 3 to decide whether to combine the QoS monitoring subscription request with the request for unicast resources.

2.	The NRM server interacts with the NEF to establish relevant QoS monitoring subscriptions. The NRM server uses the NEF procedures for the AFsessionWithQoS described in clause 5.2.6.9 of 3GPP TS 23.502 [11] and the NEF procedures for the AnalyticsExposure described in clause 5.2.6.16 of 3GPP TS 23.502 [11] and in particular the UE Communication Analytics described in clause 6.7.3 of 3GPP TS 23.288 [34] and DN Performance Analytics described in clause 6.14 of 3GPP TS 23.288 [34]. Based on the input received from the VAL server in step 1, the NRM server determines the relevant NEF subscription procedures and the parameters for these subscriptions, such as the QoS parameters to be measured (e.g. packet delay, data rate, traffic volume), the frequency of reporting etc. For the frequency of reporting which can be event triggered, periodic, or when the PDU Session is released, the NRM server determines the following:

a)	if the reporting frequency is event triggered:

i)	the corresponding reporting threshold to each QoS parameter;

ii)	minimum waiting time between subsequent reports;

b)	if the reporting frequency is periodic, the reporting period.

3.	The NRM server responds with a unicast QoS monitoring subscription response indicating the subscription status.

* * * Next Change * * * *

###### 14.3.3.4.2.2	Procedure

Figure 14.3.3.4.2.2-1 illustrates the high level procedure of unicast QoS monitoring notification event.

Pre-conditions:

-	The VAL server has an active unicast QoS monitoring subscription with the NRM server

Figure 14.3.3.4.2.2-1: Unicast QoS monitoring notification procedure

1.	The NRM server receives QoS monitoring data by means of notifications provided by the NEF. The NRM server coordinates and combines the information from the NEF notifications and determines whether to send a notification to the VAL server based on the VAL server subscription’s frequency of reporting. For a VAL group or a list of VAL UEs, the NRM server aggregates QoS monitoring data for each UE belonging to the group or the list; for a VAL stream, the NRM server aggregates the QoS monitoring data for the stream. The NRM server stores the QoS monitoring data as needed for later retrieval.

2.	The NRM server sends a unicast QoS monitoring notification including the measured QoS data to the VAL server.

* * * Next Change * * * *

###### 14.3.3.4.4.2	Procedure

Figure 14.3.3.4.4.2-1 illustrates the high level procedure of unicast QoS monitoring data retrieval.

Figure 14.3.3.4.4.2-1: Retrieval of unicast QoS monitoring data

1.	The VAL server sends a unicast QoS monitoring data request to the NRM server specifying the details of the requested data and the time period of interest. The NRM server checks if the VAL server is authorized to initiate the unicast QoS monitoring data request for the requested target VAL UEs, VAL group ID, or VAL stream IDs.

2.	The NRM server determines if it has the requested data stored internally or whether it needs to interact with the NEF to fetch the data using the AnalyticsExposure described in clause 5.2.6.16 of 3GPP TS 23.502 [11] and in particular the UE Communication Analytics described in clause 6.7.3 of 3GPP TS 23.288 [34] and DN Performance Analytics described in clause 6.14 of 3GPP TS 23.288 [34]. The NRM server collects and processes the collected data to match the measurement data requirement provided in the VAL server request.

3.	The NRM server responds with a unicast QoS monitoring data response with the requested data or with a failure indication, and optionally the cause of the failure, in case the requested data is not available.

* * * End of Changes * * * *