3GPP TSG-SA WG6 Meeting #49-Bis-e	S6-221665

E-meeting, 22nd June – 1st July 2022	(revision of S6-22xxxx)

Source:	CATT

Title:	Pseudo-CR on solution#4 update

Spec:	3GPP TR 23.700-96 v0.6.0

Agenda item:	9.7

Document for:	Approval

Contact:	Chunshan Xiong, chunshan.xiong@cictmobile.com

1. Introduction

This contribution updates solution#4.

2. Reason for Change

The FLS can get target UE location information from multiple location resources, e.g. from the FLC, SEAL LMS, and 3rd party location server. The FLC only needs to identify its location capabilities to provide the UE location information to the FLS via an IP connection.

* * * First Change * * * *

7.4	Solution #4: Location service registration

7.4.1	Solution description

This solution addresses key issue #5: Initialization and configuration for fused location service.

The procedure for location service registration is illustrated in figure below. This procedure is based on fused location architecture of solution#1. The purpose of this procedure is for the fused location client to register to the location services available at the fused location server while ensuring the privacy of the user.

If the Multi-USIM is supported by the fused location client, the fused location client performs the registration procedure for each PLMN of the Multi-USIM PLMNs.

Figure 7.4.1-1: Location service registration procedure

1.	The fused location client of a target UE sends location service registration request to the fused location server via the FLS-1 reference point, carrying the identifier of the UE (e.g. GPSI,  UUID, etc.) and  location capabilites (e.g. the available access types, the location methods, the location accuracy and latency of the access types).

To access the FLS via the FLS-1 reference point, the fused location client can use any available IP connection to send the location service registration request.

The fused location server can be pre-configured in the UE or is discovered via the DNS query.

2.	The fused location server checks authorization for the UE's request. If the fused location server supports privacy checking it also performs or assists with e.g. 5GC on privacy check.

3.	The fused location server, upon successful authorization and privacy check (if any), responds to the fused location client with registration result and stores the UE idnetifier information and location capabilities into the UE location context.

If the Multi-USIM is supported by the fused location client, the fused location client performs the steps 1 to 3 to register its identifier (e.g. MSISDN) associated with each PLMN of the Multi-USIM PLMNs .

7.4.2	Solution evaluation

This solution is based on the architecture proposed in Solution #1. the fused location client can provide its UE IDs and the location capabilities of the available IP connection to the FLS, and the FLS can generate and store these inforation as the UE location context. The FLS can use these UE location context in the following location procedures.