3GPP TSG-SA WG6 Meeting #53	S6-230934

Athens, Greece 27th February – 3rd March 2023	(revision of S6-230719)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0367 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on storing of MC service IDs mapping in Primary system during migration service authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | IRail |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MC service ID assigned by the migrated MC system for the Migrated MC service user is returned in the private call redirection response If the private call originated towards a migrated MC service user from the MC service user who belongs to the migrated MC service user’s home MC system (i.e. primary MC system). The storing of MC service IDs mapping information is not specified in the migration service authorization procedure but only the primary MC service server marking the MC service user as having migrated, and recording the partner MC system as the migrated MC system is available. This CR is trying to provide the information on storing of MC service IDs mapping infromation with any other additional information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification in step 7 that mapping of MC service IDs, migration status and any other information is added. Storing of additional info, i.e. migration status is added in the step 11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only marking of the MC service user as having migrated, and recording of the partner MC system as the migrated MC system is stored in the primary MC service server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - Storing of migration status is removed from the proposal, which is redundant information in step 7. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.6.3.3	Procedure

The procedure for service authorization for migration to a partner MC system is shown in figure 10.6.3.3-1.

Pre-conditions

-	The MC service user wishes to migrate to the partner MC system.

-	The MC service client has been configured with an MC service user profile that contains the necessary parameters needed for connectivity with the partner MC system.

-	A user authentication process has taken place which has supplied the necessary credentials to the MC service client to permit service authorization to take place in the partner system.

Figure 10.6.3.3-1	Service authorization for migration to partner MC system

1.	The MC service client requests migration service authorization with the partner MC service server indicating the selected MC service user profile to be used during migrated MC service. The MC service client provides both the MC service ID provided during user authentication in the partner MC system, and the MC service ID of the MC service user in the primary MC system of the MC service user.

NOTE 1:	The migrating MC service client also provides authentication credentials which are specified in 3GPP TS 33.180 [25].

2.	The partner MC service server performs an initial authorization check to verify that the MC service user is permitted to migrate to the partner MC system from the primary MC system of the MC service user.

NOTE 2:	The criteria for the initial authorization check is outside the scope of the present document, but for example could be based on a pre-configured list of users who are expected to request migrated service authorization.

3.	The partner MC service server identifies the primary MC system of the MC service user of the MC service client by use of the MC service ID of the MC service user in the primary MC system of the MC service user, which was presented by the MC service client in step 1, and sends a migration service authorization request to the gateway server in the partner MC system.

4.	The partner MC system gateway server identifies the primary MC system of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to the gateway server of the primary MC system.

5.	The gateway server in the primary MC system of the MC service user identifies the primary MC service server of the MC service user from the MC service ID presented in step 3, and forwards the migration service authorization request to that MC service server.

6.	The primary MC service server of the MC service user performs an authorization check, to verify that migration is permitted to that partner MC system by this MC service user using the indicated MC service user profile.

7.	The primary MC service server marks the MC service user as having migrated, and records the partner MC system as the migrated MC system. The primary MC service server also stores other necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system).

8.	The primary MC service server sends a migration service authorization response to the gateway server in the primary MC system.

9.	The gateway server in the primary MC system sends the migration service authorization response to the gateway server in the partner MC system.

10.	The gateway server in the partner MC system sends the migration service authorization response to the partner MC service server.

11.	The partner MC service server stores the necessary information related to the migrated MC service user (e.g., MC service ID of the migrated MC service user provided by the primary MC system mapped to the MC service ID of the migrated MC service user provided by partner MC system and the migration status of the MC service user). The migration status of the MC service user allows proper communication redirection back to the primary MC system once the migrated MC service user is no longer migrated on this partner MC system.

Editor’s note:	How the partner MC system gets informed that the MC service client no longer receives MC services from that partner MC system is FFS.

12.	The partner MC service server sends the migration service authorization response to the MC service client, confirming that successful migration and service authorization has taken place.

NOTE 3:	If topology hiding is not used, the migration service authorization request and migration service authorization response messages are sent between the MC service servers in the primary and partner MC system without the need to be sent via an MC gateway server.

* * * End of Changes * * * *