3GPP TSG-SA WG6 Meeting #57	S6-232801

Xiamen, China 9th – 13th October 2023

3GPP TSG CT WG3 Meeting 129	C3-233741

# Goteborg, Sweden, 21st August – 25th August 2023

Title:	LS on the Traffic specification for the time-synchronized TSC steam

Response to:

Release:	Rel-18

Work Item:	SEAL_Ph3

Source:	CT3

To:	SA6

Cc:

Contact person:	Igor Pastushok

igor.pastushok@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT3 is discussing the implementation of the TSC_Stream_Creation service operation defined in clause 14.3.7.3 of 3GPP TS 23.434.

The step 3 defined in clause 14.3.7.3 of 3GPP TS 23.434 states:

Based on the Traffic specification (from the TSC stream creation request in step 1), the SEAL NRM server determines whether time synchronization needs to be activated for the TSC stream on the DS-TTs. If the DS-TTs are time synchronized, then the NRM does not activate the time synchronization for the corresponding DS-TT.

The Traffic specification information element is specified as the following in the TSC stream creation request:

| Traffic specification | M | It includes MaxLatency, MaxFrameInterval, MaxFrameSize, MaxIntervalFrames, etc. (e.g. as described in IEEE 802.1Qcc [35] in clause 46.2). |
| --- | --- | --- |

The Traffic specification described in clause 46.2 of IEEE 802.1Qcc contains two groups of elements:

- required elements: Interval, MaxFramesPerInterval, MaxFrameSize, and TransmissionSelection; and

- optional TSpecTimeAware: EarliestTransmitOffset, LatestTransmitOffset, Jitter.

However, the step 3 defined in clause 14.3.7.3 of 3GPP TS 23.434 and TSC stream creation request do not specify the optional TSpecTimeAware information elements (e.g., EarliestTransmitOffset, LatestTransmitOffset, Jitter) that may also be needed to determine whether time synchronization needs to be activated for the TSC stream on the DS-TTs.

CT3 has the following question:

Question: What information elements should be provided in the Traffic specification (from the TSC stream creation request defined in clause 14.3.2.25 of 3GPP TS 23.434) to determine whether the time synchronization needs to be activated for the TSC stream on the DS-TTs?

# 2	Actions

To SA6 group:

ACTION: 	CT3 kindly requests SA6 to answer the question(s) above and update their specifications accordingly, if considered necessary.

# 3	Dates of next TSG CT WG 3 meetings

CT3#130                 9th October – 13th October 2023	Xiamen, China

CT3#131                 13th November – 17th November 2023 	Chicago, US