3GPP TSG-SA WG6 Meeting #57	S6-232941

Xiamen, China 9th Oct – 13th Oct 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0132 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Temporary change actions for token invalidation in RNAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure to revert to the default value for handling cases where tokens become invalidated due to expiration or revocation is not considered. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure is added about default value. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The revocation procedure lacks about restoring default value. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Exsisting clauses: 8.23.3, 8.23.4 New clauses: 8.23.X, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.23.X	Procedure for CAPIF to revert to the default value when token is invalidated in the case AEF is steteful

Figure 8.23.X-1 illustrates the procedure for reverting to the default value when token is invalidated.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

3.	Authorization details of the AEF are available with the CAPIF core function.

4.   The AEF can check the NF state at first use of token and store this state in a AEF’s repository.

Figure 8.23.X-1: Procedure for revoking API invoker authorization initiated by CAPIF core function

Procedure

1.	The AEF triggers the revocation of the API invoker authorization.

2.	The AEF sends revoke API invoker authorization request to the CAPIF core function with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CAPIF core function invalidates the API invoker authorization corresponding to the service API.

4.	The CAPIF core function sends a revoke API invoker authorization response to the AEF.

5.	Upon successful revocation of API invoker authorization corresponding to the service API at the CAPIF core function, the AEF invalidates the API invoker authorization corresponding to the service API.

5a  AEF checks repository to determine the state it was in when the initial token was used and reverts to the default value.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * Next Change * * * *

### 8.23.X.2	Procedure for CAPIF to revert to the default value when token is invalidated in the case AEF is steteless

Figure 8.23.X-2 illustrates the procedure for reverting to the default value when token is invalidated.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

3.	Authorization details of the AEF are available with the CAPIF core function.

4.   A separate function is required to track valid tokens and trigger state reversal.

Figure 8.23.X.2-1: Procedure for revoking API invoker authorization initiated by CAPIF core function

Procedure

1.	The AEF triggers the revocation of the API invoker authorization.

2.	The AEF sends revoke API invoker authorization request to the CAPIF core function with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CAPIF core function invalidates the API invoker authorization corresponding to the service API.

4.	The CAPIF core function sends a revoke API invoker authorization response to the AEF.

5.	Upon successful revocation of API invoker authorization corresponding to the service API at the CAPIF core function, the AEF invalidates the API invoker authorization corresponding to the service API.

5a  A separate function induce AEF to reset NF or call NF directly to reverse the state to the given default.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

* * * End of Change * * * *