3GPP TSG-SA WG6 Meeting #58	S6-233454

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

Source:	Apple

Title:	Pseudo-CR on EASProfile and AppInfo alignment

Spec:	3GPP TR 23.958 V1.1.1

Agenda item:	7.9

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

The mandatory IEs of ETSI MEC’s AppInfo data type have been incorrectly stated.

2. Reason for Change

The mandatory IEs of AppInfo, captured in ETSI GS MEC 011 V3.1.1, are highlighted below:

#### 7.1.2.6	Type: AppInfo

This type represents the information provided by the MEC application instance as part of the "application registration request" and "application registration update" messages.

The attributes of the AppInfo shall follow the indications provided in table 7.1.2.6-1.

Table 7.1.2.6-1: Attributes of AppInfo

| Attribute name | Data type | Cardinality | Description |
| --- | --- | --- | --- |
| appName | String | 1 | Name of the application. It shall be consistent with the appName in the AppD, if an AppD is available. |
| appProvider | String | 0..1 | Provider of the application. It shall be consistent with the appProvider in the AppD, if an AppD is available. See note 1. |
| appCategory | CategoryRef | 0..1 | Category of the application. |
| appDId | String | 0..1 | The application descriptor identifier. It is managed by the application provider to identify the application descriptor in a globally unique way. Shall be present if the application instance is instantiated by the MEC Management. |
| appInstanceId | String | 0..1 | Identifier of the application instance. Shall be present if the application instance is instantiated by the MEC Management. |
| endpoint | EndPointInfo | 0..1 | Endpoint information (e.g. URI, FQDN, IP address) of the application server, which is part of the application functionalities. Shall be present when isInsByMec is FALSE. See note 2. |
| appServiceRequired | ServiceDependency | 0..N | Describes services a MEC application requires to run. ServiceDependency is defined in ETSI GS MEC 010-2 [4]. It shall not be provided if an AppD is available. |
| appServiceOptional | ServiceDependency | 0..N | Describes services a MEC application may use if available. ServiceDependency is defined in ETSI GS MEC 010-2 [4]. It shall not be provided if an AppD is available. |
| appFeatureRequired | FeatureDependency | 0..N | Describes features a MEC application requires to run. FeatureDependency is defined in ETSI GS MEC 010-2 [4]. It shall not be provided if an AppD is available. |
| appFeatureOptional | FeatureDependency | 0..N | Describes features a MEC application may use if available. FeatureDependency is defined in ETSI GS MEC 010-2 [4]. It shall not be provided if an AppD is available. |
| isInsByMec | Boolean | 0..1 | Indicate whether the application instance is instantiated by the MEC Management. Default to FALSE if absent. |
| appProfile | AppProfile | 0..1 | Can be mapped to EAS profile as defined in ETSI TS 129 558 [19]. More information can be found in the informative Annex C. See note 1 and note 2. |
| NOTE 1: If appProfile is present, appProvider shall be consistent with provId provided in EAS profile data type, i.e. the same. NOTE 2: If appProfile is present, endpoint shall refer to the same end point as endPt provided in EAS profile data type. |  |  |  |

3. Conclusions

It is proposed to provide clarifcation to clause 5.3.1 EASProfile and AppInfo on the mandatory IEs of AppInfo and the assocaited alignment with EASProfile. Formating changes are also proposed for table 5.3.1-1.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958 V1.1.1.

* * * First Change * * *

## 5.3	EDGE-3 and Mp1 reference points

### 5.3.1 	EASProfile and AppInfo

Both EDGEAPP and ETSI MEC supports registration of EAS and MEC application instance with EES and MEC platform respectively. In order to support MEC application instance registration on EES, it is required that registration request includes at least the mandatory IEs that are required for EAS registration, i.e. EAS ID and EAS endpoint. On the other hand, according to ETSI GS MEC 011 [5] the application registration request must include appName.

AppInfo is the data type describing the information exchanged by a MEC application instance at registration to a MEC platform. It is defined in clause 7.1.2.6 of ETSI GS MEC 011 [5] and includes appName as a mandatory IE and endpoint as an optional IE. However, endpoint is mandatory when the IE isInsByMec is FALSE and, as stated above, would have to be provided for MEC application instance registration to an EES. The isInsByMec IE, with type Boolean, indicates whether the application instance is instantiated by a MEC management system. The IE appName can be considered equivalent to EAS ID. Furthermore endpoint can be directly mapped to EAS endpoint and that mapping is explicitly stated in note 2 of Table 7.1.2.6-1 of ETSI GS MEC 011 [5].

Table 5.3.1-1 provides a mapping of MEC attributes to those in the EAS Profile for MEC application registration with EES.

Table 5.3.1-1: Mapping with EASProfile for MEC application registration with EES

| Information element | Status/ Cardinality [2] | Description | Mapped with |
| --- | --- | --- | --- |
| EASID | M | The identifier of the EAS | AppInfo >appName |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. | AppInfo >endpoint |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS | Not available in case of MEC application registration |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. | AppInfo >appProvider |
| EAS Type | O | The category or type of EAS (e.g. V2X) | AppInfo >appCategory |
| EAS description | O | Human-readable description of the EAS | AppD >appDescriptor |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) | Not available in case of MEC application registration |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. | Not available in case of MEC application registration |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. | Not available in case of MEC application registration |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 | Not available in case of MEC application registration |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS | Not available in case of MEC application registration |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS | Not available in case of MEC application registration |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. | Not available in case of MEC application registration |
| General context holding time duration | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. | Not available in case of MEC application registration |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. | Not available in case of MEC application registration |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. | Not available in case of MEC application registration |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. | Not available in case of MEC application registration |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) | Not available in case of MEC application registration |

* * * End of Changes * * *