3GPP TSG-SA WG6 Meeting #58	S6-233516

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

Source:	Samsung

Title:	Pseudo-CR on new KI for enablement support for AI/ML operation spliting

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

Contact:	Sapan Shah (sapan.shah@samsung.com)

1. Introduction

This pCR proposes new key issue to study enablement stupport for AI/ML operation splitting between AI/ML endpoints.

2. Reason for Change

As specificed in TS 22.261, AI/ML operation splitting between AI/ML endpoints is one of the three types of AI/ML operation.

-	AI/ML operation splitting between AI/ML endpoints

The AI/ML operation/model is split into multiple parts according to the current task and environment. The intention is to offload the computation-intensive, energy-intensive parts to network endpoints, whereas leave the privacy-sensitive and delay-sensitive parts at the end device. The device executes the operation/model up to a specific part/layer and then sends the intermediate data to the network endpoint. The network endpoint executes the remaining parts/layers and feeds the inference results back to the device.

At enabler layer, SA6 can provide support to negotiate split inference and also to discover nodes to perform computation-intensive, energy-intensive parts of AI/ML operations.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[r22261]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

* * * Next Change * * * *

## 5.x	Key issue #x: Enablement support for AI/ML operation spliting

As specified in 3GPP TS 22.261 [r22261], AI/ML operation splitting between AI/ML endpoints is one of the three types of AI/ML operation. The scheme of split AI/ML inference can be depicted in Figure x-1. The AI/ML operation/model is split into multiple parts according to the current task and environment (such as communications data rate, device resource, and server workload). The intention is to offload the computation-intensive, energy-intensive parts to network endpoints (cloud or edge server), whereas leave the privacy-sensitive and delay- sensitive parts at the end device. The device executes the operation/model up to a specific part/layer and sends the intermediate data to the network endpoint. The network endpoint executes the remaining parts/layers and feeds the inference results back to the device.

Figure x-1. Example of split AI/ML inference (from Annex B.1 of TR 22.874)

At enabler layer, a support can be provided to negotiate splitting of the AI/ML operation and also to discover required nodes to perform computation-intensive, energy-intensive parts of AI/ML operations.

Open issues:

1)	Whether and how to discover requried nodes to perform computation-intensive, energy-intensive parts of AI/ML operations?

* * * End of Change * * * *