3GPP TSG-SA WG6 Meeting #58	S6-233540

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0338 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to modification of ad hoc group data communication participants procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Any authorised user can modify ad hoc group data communication participants. However, the existing procedure allows only the initiator of the data communication to modify the ad hoc group data communication participants. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The modification of the ad hoc group data communication participants procedure title is updated to indicate that it is applicable for authorized users and the pre-conditions are updated to indicate that the ad hoc group data communication is already established and initiated by one of the ad hoc group participants with additional changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only the initiator of the data communication will be able to modify ad hoc group data communication participants and no way for an authorised user (e.g. Dispatcher) to modify. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.17.3.1.4	Modification of ad hoc group data communication participants by the initiator

Figure 7.17.3.1.4-1 below illustrates the modification of ad hoc group data communication participants procedure.

Pre-conditions:

1.	An MCData ad hoc group data communication is already in progress the participants list provided by the originating MCData user while initiating the MCData ad hoc group data communication.

2.	The participants of the MCData ad hoc group call belongs to the single MCData system.

3.	The MCData users on MCData client 1, MCData client 3 to MCData client n are on an ongoing ad hoc group data communication.

4.	The MCData user at MCData client 1 determines to remove the user of MCData client 3 from the ad hoc group data communication and add user of MCData client 2 into the on-going ad hoc group data communication.

Figure 7.17.3.1.4-1: Modification of ad hoc group data communication participants by the initiator

1.	The MCData client 1 sends the modify ad hoc group data session participants request to the MCData server in order to remove MCData client 3 from the ongoing ad hoc group data communication and add MCData client 2 into it.

2.	The MCData server verifies whether the MCData client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group data communication.

3.	The MCData server sends modify ad hoc group data session participants response to the MCData client 1.

4.	The MCData server sends the ad hoc group data session leave request to the MCData client 3 in order to remove it from the on-going ad hoc group data communication.

5.	The MCData client 3 notifies the user of the ad hoc group data session leave request.

6.	The MCData client 3 sends the ad hoc group data session leave response to the MCData server.

7.	The MCData server removes the MCData client 3 from the ongoing ad hoc group data communication.

8.	The MCData server sends the ad hoc group data session request towards MCData client 2.

NOTE:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCData client 2 notifies the user about the incoming ad hoc group data communication.

10.	The MCData client 2 accepts the ad hoc group data session request and send ad hoc group data session responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

11.	The MCData server may notify the initiating MCData user of all the users who are added to the on-going ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the ad hoc group data communication.

12.	The MCData server may notify the participants about the change in the participants list of on-going ad hoc group data communication.

* * * * End of changes * * * *