3GPP TSG-SA WG6 Meeting #58	S6-233543

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0205 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to modification of ad hoc group call criteria by the initiator procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Any authorised user can modify criteria of an ongoing ad hoc group call. However, the existing procedure allows only the initiator of the call to modify criteria of an ongoing ad hoc group call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Included Criteria for determining the participants in the Ad hoc group call request (MCVideo server – MCVideo client). - Modify ad hoc group call criteria request and Modify ad hoc group call criteria response – Information flow is updated to indicate that the MCVideo ID included is of the user requesting for modification of ad hoc group call criteria. - The modification of the ad hoc group call criteria procedure title is updated to indicate that it is applicable for authorized users and the pre-conditions are updated to indicate that the ad hoc group call is already established and initiated by one of the ad hoc group participants with additional changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only the initiator of the call will be able to modify criteria of an ongoing ad hoc group call and no way for an authorised user (e.g. Dispatcher) to modify. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.2.4, 7.19.2.20, 7.19.2.21, 7.19.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 7.19.2.4	Ad hoc group call request (MCVideo server – MCVideo client)

Table 7.19.2.4-1 describes the information flow ad hoc group call request from the MCVideo server to the MCVideo client.

Table 7.19.2.4-1 Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | M | The identity of the MCVideo user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ad hoc group ID | M | The MCVideo group ID to be associated with the ad hoc group call |
| SDP offer | M | Media parameters of MCVideo server |
| Broadcast indicator (NOTE 1) | O | Indicates that the ad hoc group call request is for a broadcast ad hoc group call |
| Imminent peril indicator (NOTE 1) | O | Indicates that the ad hoc group call request is an MCVideo imminent peril call |
| Emergency Indicator (NOTE 1) | O | Indicates that the ad hoc group call request is an MCVideo emergency call |
| Preconfigured ad hoc group identity | O | Group identity whose configuration is to be applied for this ad hoc group call. |
| Criteria for determining the participants (see NOTE 2) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |
| NOTE 1: If used, only one of these information elements is present. NOTE 2: This information element is included for participants authorised to modify criteria of an ongoing ad hoc group call. |  |  |

* * * Next Change * * * *

#### 7.19.2.20	Modify ad hoc group call criteria request (MCVideo client – MCVideo server)

Table 7.19.2.20-1 describes the information flow Modify ad hoc group call criteria request from the MCVideo client to the MCVideo server.

Table 7.19.2.20-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the user who is requesting for modification |
| Functional alias | O | The functional alias of the user who is requesting for modification |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 7.19.2.21	Modify ad hoc group call criteria response (MCVideo server – MCVideo client)

Table 7.19.2.21-1 describes the information flow Modify ad hoc group call criteria response from the MCVideo server to the MCVideo client.

Table 7.19.2.21-1 Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the user who is requesting for modification |
| Functional alias | O | The functional alias of the user who is requesting for modification |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |

* * * Next Change * * * *

##### 7.19.3.1.6	Modification of ad hoc group call criteria by an authorized user

Figure 7.19.3.1.6-1 below illustrates the modification of ad hoc group call criteria procedure.

Pre-conditions:

1.	An ad hoc group call is already in progress and the criteria provided by the originating MCVideo user while initiating the MCVideo ad hoc group call.

2.	The participants of the MCVideo ad hoc group call belongs to the single MCVideo system.

3.	The MCVideo users on MCVideo client 1, MCVideo client 3 to MCVideo client n are on an ongoing ad hoc group call.

4.	MCPTT user at MCVideo client 1 determines to modify the criteria for determining the list of participants. MCPTT client 3 does not meet the modified criteria while MCPTT client 2 meets the modified criteria.

Figure 7.19.3.1.6-1: Modification of ad hoc group call criteria by an authorized user

1.	The MCVideo client 1 sends the modify ad hoc group call criteria request to the MCVideo server which contains an updated criteria to determine the list of participants.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group video call. The MCVideo server determines the list of ad hoc group call participants based on the criteria provided and identifies that MCVideo client 3 is to be removed from and MCVideo client 2 is to be added to the group.

3.	The MCVideo server sends modify ad hoc group call criteria response to the MCVideo client 1.

4.	The MCVideo server sends the ad hoc group call leave request to the MCVideo client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCVideo client 3 notifies the user of the ad hoc group call leave request.

6.	The MCVideo client 3 sends the ad hoc group call leave response to the MCVideo server.

7.	The MCVideo server removes the MCVideo client 3 from the ongoing ad hoc group call.

8.	The MCVideo server sends the ad hoc group call request towards MCVideo client 2.

NOTE 1:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCVideo client 2 notifies the user about the incoming ad hoc group call.

10.	The MCVideo client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server.

11.	The MCVideo server may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

12.	The MCVideo server may notify the participants about the change in the participants list of on-going ad hoc group call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

* * * * End of changes * * * *