3GPP TSG-SA WG6 Meeting #58	S6-233583

Chicago, IL, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0342 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Change list of participants when criteria was used to setup the ad hoc group call (MCData) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Ad hoc can be formed by a list of participants or by criteria provided by an authorized user. The list of participants created by the criteria may not be complete or some necessary users may not be in the group. An authorized user should be able to override the list of participants if needed by providing a distinct list of participants also when criteria was used to setup the ad hoc group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifies that the critieria is not longer in use when an authorised user overrides the criteria by modifying the list of participants using a list of MCData IDs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

##### 7.17.3.1.4	Modification of ad hoc group data communication participants by the initiator

Figure 7.17.3.1.4-1 below illustrates the modification of ad hoc group data communication participants procedure by the initiator of the ad hoc group data communication.

Pre-conditions:

1.	The MCData client 1 is the initiator of the ad hoc group data communication.

2.	The MCData users on MCData client 1, MCData client 3 to MCData client n are on an ongoing ad hoc group data communication.

3.	The MCData user 1 requests to remove the user of MCData client 3 from the ad hoc group data communication and add user of MCData client 2 into the on-going ad hoc group data communication.

Figure 7.17.3.1.4-1: Modification of ad hoc group data communication participants by the initiator

1.	The MCData client 1 sends the modify ad hoc group data session participants request to the MCData server in order to remove MCData client 3 from the ongoing ad hoc group data communication and add MCData client 2 into it.

2.	The MCData server verifies whether the MCData client 1 is authorized to add or remove (modify) the participants of the on-going ad hoc group data communication.

3.	The MCData server sends modify ad hoc group data session participants response to the MCData client 1. If the ad hoc group was formed by using criteria, this request overrides the criteria and the MCPTT server stops checking whether MCData clients meet of no longer meet the criteria.

4.	The MCData server sends the ad hoc group data session leave request to the MCData client 3 in order to remove it from the on-going ad hoc group data communication.

5.	The MCData client 3 notifies the user of the ad hoc group data session leave request.

6.	The MCData client 3 sends the ad hoc group data session leave response to the MCData server.

7.	The MCData server removes the MCData client 3 from the ongoing ad hoc group data communication.

8.	The MCData server sends the ad hoc group data session request towards MCData client 2.

NOTE:	Steps 8 to 10 can occur at any time following step 3.

9.	The receiving MCData client 2 notifies the user about the incoming ad hoc group data communication.

10.	The MCData client 2 accepts the ad hoc group data session request and send ad hoc group data session responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

11.	The MCData server may notify the initiating MCData user of all the users who are added to the on-going ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the ad hoc group data communication.

12.	The MCData server may notify the participants about the change in the participants list of on-going ad hoc group data communication.

# * * * * End of changes * * * *