3GPP TSG-SA WG6 Meeting #58	S6-233617

Chicago, USA, 13th – 17th November 2023	(revision of S6-233355)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0081 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-triggered protection zone configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Added Procedure on UE-triggered dynamic VRU zone configuration from TR 23.700-64 to TS 23.286. The procedure allows VAE client to request the creation of VRU zones if authorized. Implementing this functionality requires VAE involvement because it involves signaling affecting VAE performance. In addition, in order to provide detection of vulnerable users in relationship to the initiating UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure on UE-triggered dynamic VRU zone configuration from TR 23.700-64 is proposed for addition to TS 23.286. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 23.286 does not provide support for UE-triggered dynamic VRU zone configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.21.1; New: 9.21.2.x1; 9.21.2.x2; 9.21.2.x3; 9.21.x4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

The terms and definitions as specified in 3GPP TS 23.286 [6] apply.

VRU high-risk zone: Refers to a geographic area with Vulnerable Road Users (VRUs) recognition mechanisms.

Protection zone: Refers to a geographic area surrounding a UE in which functionality such as ITS V2X, V2P, V2V is employed for the immediate purpose of protection of the UE and of the area VRUs.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSM	Basic Safety Message

CAM	Cooperative Awareness Message

CPM	Collective Perception Message

DENM	Decentralized Environmental Notification Message

V2P	Vehicle-to-Pedestrian

V2X	Vehicle-to-Everything

VAE	V2X Application Enabler

VAM	VRU awareness message

VASS	V2X Application Specific Server

VRU	Vulnerable Road Users

VRUP	Vulnerable Road User Protection

* * * Next Change * * * *

### 9.21.1	General

This capability supports the configuration and provisioning of the VRU high risk zone at the VAE layer based on a request from the V2X application specific server (VASS). ). UE-initiated triggering of configuration and provisioning of protection zones is also supported at the VAE layer.

NOTE: Protection zones in the service layer are specific to the protected UE and are mapped by VASS and VAE  to one or more dynamic VRU high-risk zones. UE triggering of protection zones and their configuration via the VAE layer are limited via policies and subject to authorization.

Editor’s Note: It is FFS whether the “protection zone” terminology is to be changed for better alignment with ITS terminology e.g., “ dynamic VRU high-risk zone”.

* * * Next Change * * * *

* * * Next Change * * * *

### 9.21.x4	 Procedure on UE/VRU-triggered protection zone configuration

Protection zones are defined specific to a VRU and are based on UE/VRU-initiated requests and VASS configurations. Based on this information, the one or more dynamic VRU high-risk zones may be configured, providing jointly the protection zone functionality. The procedure in figure 9.21.x4-1 complements the VASS Server-triggered VRU high-risk zone configuration procedure in clause 9.21.3.

NOTE: UE triggering of protection zones and their configuration via the VAE layer are limited via policies and subject to authorization.

Pre-conditions:

- V2X application specific server or client has subscribed to VAE layer to provide support for V2P communication.

- The VAE server and the VAE client are preconfigured with policies allowing support of UE protection zone configuration and support of UE/VRU-initiated requests.

Figure 9.21.x4-1: UE-triggered protection zone configuration

- a.  Optionally, an application client on the UE makes a request to the VAE client to create a UE/VRU protection zone.

b.  The VAE client sends a protection zone management request to a VAE server to create a protection zone. The request includes the information elements specified in Table 9.21.2.x3-1.

2.	The VAE server determines the validity of the request depending on whether UE protection zone configurations and UE/VRU-initiated requests for protection zone configurations are supported. If the request is valid, the VAE server sends a protection zone management notification to a VASS for the creation of the protection zone for the UE and includes the information provided in the request.

3.	The VASS authorizes the request and determines whether a protection zone based on information received from the VAE server is to be created. The VASS provides the VAE server with the protection zone and V2P communication configuration.

4.	The VASS determines one or more dynamic VRU high-risk zone configurations corresponding to the protection zone request and configures the VAE server accordingly. Steps 1-3 of clause 9.21.3 are executed for each of the dynamic VRU high-risk zones determined.

5.	The VAE server configures CN and SEAL LMS subscriptions to obtain UE location from the 5G system for the protected UE and for vehicular UEs around the protected zone. Sources of UE mobility information may come from SEAL LMS servers, EESs, analytics functions (e.g. from ADAES or NWDAF), and the 5G core network. For V2X UE location obtained from the location management server, the procedures specified in 3GPP TS 23.434 (clause 9.3.12) apply.

- a. The VAE Server sends a protection zone management response to the VAE client that the protection zone has been created.

b.  Optionally, the VAE client sends a response to the triggering application client on the protected UE, confirming the creation of the protection zone.

7.	The VAE server uses notifications from the 5G network and SEAL LMS (e.g. updated location information of the protected UE) to update the protection zone configuration, which may result in updates to one or more of the component dynamic VRU high-risk zones.

8.	The VAE server sends notification messages to other VRUs (pedestrians or V2X-UEs at similar risk) within each of the component dynamic VRU high-risk zones. The VAE server also sends vehicular UEs within these zones V2P communication parameters to enable communication between the vehicular UEs and the protected UE.

9. a. The VAE server alerts the VAE client when V2X UEs posing risk to the protected UE are expected to move, in a given time horizon, into the dynamic VRU high-risk zones composing the protection zone.

b. The VAE server alerts VASS when V2X UEs posing risk to the protected UE are expected to move into each of the dynamic VRU high-risk zones composing the protection zone.

* * * End of Changes * * * *