3GPP TSG-SA WG6 Meeting #58	S6-233629

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0538 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add common EAS endpoint in EAS selection response |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In common EAS scenario, if EEC informs EES about the selected EAS, EES uses this information to interact with ECS-ER as described in cl.8.15.2.2: - If there is ECS-ER, the EEC selected EAS is used in interaction between the EES and ECS-ER, and EEC receives common EAS in the EAS information provisioning response. If ECS-ER returns a different EAS (which is common EAS) to EES, the EAS information provisioning response does not have the support for this EAS information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add common EAS endpoint and common EES endpoint in EAS information provisioning response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete function. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.15.3.3	EAS information provisioning response

Table 8.15.3.2-1 describes the information elements for EAS information provisioning response from the EES to the EEC.

Table 8.15.3.3-1: EAS information provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE 1) | O | Indicates that the request was successful. |
| > Selected ACR scenario list (NOTE 2) | O | The list of ACR scenarios (or the list of ACR scenarios for EAS bundles) selected by the EES |
| > Instantiated EAS Information (NOTE 3) | O | Instantiated EAS information. Each element includes the information described below. |
| >> EAS Profile | M | The profile of the instantiated EAS.Each element is described in clause 8.2.4. |
| >> EES Endpoint | O | The endpoint address (e.g. URI, IP address) of the EES |
| >> Lifetime | O | Time interval or duration during which the information elements in the EAS profile is valid and supposed to be cached in the EEC (e.g. time-to-live value for an EAS Endpoint) |
| > Common EAS endpoint (NOTE 4) | O | This IE includes common EAS endpoint. |
| > Common EES endpoint (NOTE 4) | O | This IE includes common EES endpoint. It shall be provided when common EAS endpoint is provided. |
| Failure response (NOTE 1) | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE 1: One of these IEs shall be present in the message. NOTE 2: Only if request type is “ACR scenario selection request”. NOTE 3: Only if request does not include selected EAS endpoint. NOTE 4: Only if request type is “EAS selection”. |  |  |

* * * END of Change * * * *