3GPP TSG-SA WG6 Meeting #58	S6-233651

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0477 | rev |  | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Connection authorization corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, before the MC service user using the N3GPP device to exchange data with the MC servers, the connection authorization is performed. However, what is being authorized is not clear. In clause 11.5.1 and 11.5.1.2.1 the following descriptions indicates the “connection” between the N3GPP device and the gateway UE is being authorized. The connection of non-3GPP devices via an MC gateway UE require authorisation verification by the MC system. Two different types of non-3GPP devices are supported, those which can host MC service client and those which cannot host MC service clients. “The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC gateway client is authorized and controlled by the MC server. ” The benefit of the connection authorization is to enforce the traffic control of the MC service user ‘s traffic via the MC gateway UE, i.e., allow traffic of authorized user and prevent traffic of unauthorized user at the MC gateway UE rather than at the MC servers. After the MC service user is authenticated and the service is authorized, the MC server can configure the traffic filter to the MC gateway UE to ‘open the gate’ for certain traffic per connection request from the client from the N3GPP gateway. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the connection authorization descriptions to clarify that the connection authorization is used for the access control at the MC gateway UE of certain traffic. - Clarify that the connection authorization is triggered by the MC gateway client at Non-3GPP device to request to open the gate for the specific traffic of the authorized user. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The connection authorization feature is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7, 15.5.1.1, 11.5.1.2.1, 11.5.1.2.2, , 11.5.1.2.2.1, 11.5.1.2.2.2,11.5.1.2.3, 11.5.1.3.1, 11.5.1.3.2,11.5.1.3.2.1, 11.5.1.3.2.2, 11.5.1.3.3 New {11.5.1.2a, 11.5.1.2a.1, 11.5.1.2a.2, 11.5.1.2a.3, 11.5.1.2a.4} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

8.1.7	GW MC service ID

The GW MC service ID is used for identity the MC service user from the non-3GPP device within the MC gateway UE. The MC gateway UE allocates the GW MC service ID to the MC service user upon receiving the successful connection authorization from the MC server. The GW MC service ID is used in the signalling messages exchange between the MC gateway client and the MC gateway UE as described in clause 11.5.2 and 11.5.3.

# /************************ Next change ******************************/

11.5.1.1	General

The connection of non-3GPP devices via an MC gateway UE require authorisation verification by the MC system, in order to enforce routing control of the service traffic  (e.g., affiliation, call reqest, floor control/transmission control, media with the corresponding MC server) from a MC service user at the MC gateway UE. Two different types of non-3GPP devices are supported, those which can host MC service client and those which cannot host MC service clients.

For the identity authentication and service authorization traffic, the routing control at the MC gateway UE is not required.

11.5.1.2a	Information flows

11.5.1.2a.1	Connection authorization request (MC gateway client - MC gateway UE server)

Table 11.5.1.2a.1-1 describes the information flow connection authorization request sent from the MC gateway client, which resides on a non-3GPP device, to the MC gateway UE.

Table 11.5.1.2a.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |

11.5.1.2a.2	Connection authorization request (MC gateway UE server - MC gateway UE function)

Table 11.5.1.2a.2-1 describes the information flow connection authorization request sent from the MC gateway UE server to the MC gateway UE function.

Table 11.5.1.2a.2-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| MC service ID | M | The MC service ID of the MC gateway UE |

11.5.1.2a.3	Connection authorization response (MC gateway UE function - MC gateway UE server)

Table 11.5.1.2a.3-1 describes the information flow connection authorization response sent from the MC gateway UE function to the MC gateway UE server.

Table 11.5.1.2a.3-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC service user |
| Response | M | Result of the connection authorization request, service feasibility, and connection evaluation. |

11.5.1.2a.4	Connection authorization response (MC gateway UE server - MC gateway client)

Table 11.5.1.2a.4-1 describes the information flow connection authorization response sent from the MC gateway UE function to the MC gateway UE server.

Table 11.5.1. 2a.4-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID allocated to the requesting MC service user by the MC gateway UE. |
| Response | M | Result of the connection authorization request, service feasibility, and connection evaluation. |

# /************************ Next change ******************************/

11.5.1.2	Connection authorisation for non-3GPP devices that host an MC client

11.5.1.2.1	General

The clause is applied to non-3GPP devices which can host an MC client. Before request the MC gateway UE to enable the service traffic routing (e.g., affiliation, call request, floor control/transmission control, media with the corresponding MC server), the MC service user has to complete the identity authentication and service authorization with the corresponding MC server. The MC service user uses the MC gateway client to initiate the connection authorization. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC gateway client is authorized and controlled by the MC server. The MC gateway client informs MC clients about the connection status.

The MC server maintains the assignment between MC clients to the MC gateway UE until the disconnection..

11.5.1.2.2	Void

11.5.1.2.2.1	Void

11.5.1.2.2.2	Void

11.5.1.2.3	Connection authorisation for MC client host on Non-3GPP device procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 11.5.1.2.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	 The Non-3GPP device already establish the secure non-3GPP connection (e.g., WiFi) with the MC gateway UE

-	The MC gateway client has been configured with the necessary parameters needed for connectivity with the MC gateway UE e.g., the identity of the MC gateway UE used at the GW-local reference point (e.g., MC service ID of the MC gateway UE).

-

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system as described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282 [13].

-	The MC gateway client has selected an MC gateway UE or alternatively, the MC gateway client has performed a selection by internal criteria.

-	The MC client already complete the identity authentication and service authorization with the corresponding MC server.

NOTE:	The internal criteria are outside the scope of the present document.

Figure 11.5.1.2.3-1: Connection authorisation with an MC server via an MC gateway UE

1.	The MC gateway client requests connection authorization via the MC gateway UE with an MC server. The MC gateway client provides the MC service ID of the MC service user.

2.	The MC gateway UE checks whether the requested MC service, as indicated by the MC service ID of the MC service user, is supported. The MC gateway UE may also check whether sufficient resources are available or if any other local criteria are met. If the MC service is supported, the procedure continues with step 3, otherwise the procedure proceeds with step 7.

NOTE:	Further information to the MC gateway UE selection is in Annex D.

3.	The MC gateway UE sends the connection authorization request to the MC server. The corresponding MC service ID of the MC gateway UE is provided along with the MC service ID of the requesting MC service user for connection authroization.

4.	The MC server performs an authorization check, to verify that access via the MC gateway UE is permitted. An MC server shall reject the connection authorization when the MC server receives connection authorization from a MC service user for a particular MC service for which the connection already exists with the same or different MC gateway UE.

5.	The MC server sends the connection authorization response to the MC gateway UE. The allowed traffic filter information is included to instruct the MC gateway UE to route the traffic between the MC client and the MC server.

6.	The MC gateway UE stores the allowed traffic filter information for further traffic routing control. The MC gateway UE allocates a GW MC service ID to the MC service user for the following signalling between the MC gateway client and the MC gateway UE via the GW-local reference point.

7.	The MC gateway UE sends the connection authorization response to the MC gateway client with the GW MC service ID, the connection authorization result.

After successful connection with the MC gateway UE, the MC clients are able to exchange the service related traffic e.g., affiliation, call request, floor control/transmission control, media with the corresponding MC server via the MC gateway UE.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

11.5.1.3	Connection authorisation for non-3GPP devices cannot host an MC client

11.5.1.3.1	General

The clause is applied to non-3GPP devices which cannot host an MC client. The MC server performs authorization for the use of the MC gateway UE by the MC gateway client, i.e. the binding between the MC gateway UE and the MC service user is authorized and controlled by the MC server.

NOTE:	The interworking between the MC gateway client hosted at the MC gateway UE and an MC service user is out of scope of the present document, nevertheless, the connection authorisation performed by the MC gateway UE shall enable the non-3GPP devices to get the access to MC services requested by the MC service user.

11.5.1.3.2	Void

11.5.1.3.2.1	Void

11.5.1.3.2.2	Void

11.5.1.3.3	Connection authorisation for MC client host on MC gateway UE procedure

The procedure for connection authorisation of an MC gateway client hosted by the MC gateway UE towards an MC server is shown in figure 11.5.1.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device, where the MC gateway client and MC clients are hosted by the MC gateway UE.

-	The MC client already complete the identity authentication and service authorization with the corresponding MC server.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system as described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282 [13]].

Figure 11.5.1.3.3-1: Connection authorisation of an MC gateway client hosted by an MC gateway UE

1.	The MC gateway client, hosted by the MC gateway UE, requests connection authorization with an MC server by providing the MC service ID of the MC service user and the corresponding MC service ID of the MC gateway UE obtained from the service authorization. The MC gateway UE sends the connection authorization request to the MC server.

2.	The MC server performs an authorization check, to verify that access using the MC gateway UE is permitted. An MC server shall reject the connection authorization when the MC server receives connection authorization from a MC service user for a particular MC service for which the connection already exists with the same or different MC gateway UE. The allowed traffic filter information is included to instruct the MC gateway UE to route the traffic between the MC client and the MC server.

3.	The MC server sends the connection authorization response to the MC gateway client residing on the MC gateway UE. The MC gateway UE stores the allowed traffic filter information for further traffic routing control

After successful connection with the MC gateway UE, the MC clients are able to exchange the service related traffic e.g., affiliation, call request, floor control/transmission control, media with the corresponding MC server via the MC gateway UE.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.