3GPP TSG-SA WG6 Meeting #58	S6-233666

Chicago, USA, 13th - 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0268 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area failure cause handling at NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#160, the CR addressing the MBS service area update issue was agreed in S2-2311901(R17 CR) and S2-2311276 (R18 mirror CR) In the CR, SA2 concluded that: - Case #1- TMGI allocation: NEF reject the request if cannot satisfy the service area request. - Case #2 - MBS Session Creation without TMGI: NEF reject the request if cannot satisfy the service area request. - Case #3 - MBS Session Creation with TMGI: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition; - Case #4 - MBS Session Update: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition. So the behaviour of the NRM server to well handle the above response from the network needs to be defined in order to better make use of the MBS feature: Case #1, #2 – The NRM sever needs to modify the service area, i.e., providing smaller service area, or using unicast delivery. Case #3, #4 – The NRM server may try another new MBS session for the non-supported MBS service area. If still fails, use the unicast delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the clause to describe how the NRM server behave when the MBS service area update with requested MBS service area is failed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NRM server can not handle the MBS service area update failure correctly, and the MBS may not be used as much as impossible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new)14.3.4A.3.x, 14.3.2.42, 14.3.2.43 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

14.3.4A.3.x	MBS service area update failure handling

The MBS service area requested by the MC service server in the MBS session procedure towards the 5G core network may be rejected or partially accepted, the MC service server may take different actions as follows:

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation without TMGI procedure as specified in 3GPP TS 23.247 [15] is rejected, the MC service server may recheck the requested MBS service area and retry with a correct MBS service area. The MC service server may also determine to use the unicast delivery for the group communication in that area.

-	If the MBS service area requested in MBS Session Creation with TMGI or MBS Session Update procedure as specified 3GPP TS 23.247 [15] is partially accepted due to the requested MBS service area is beyond the service area of the serving MB-SMF, or the MBS is not deployed at the updated MBS service area, the MC service server, based on the non-supported MBS service area and the indication provided by the 5GC, may try another new MBS session with the non-supported MBS service area. If it still fails, the MC service server utilizes unicast PDU session for the group communication in that area.

The figure 14.3.4A.3.x-1 provides a high level procedure of the NRM server determines to create an extra MBS Session with the non-supported MBS service area indicated by the 5GC due to the partially accept of MBS service area update of the ongoing MBS session.

NRM client

3GPP

NRM server

VAL server

VAL client

UE

Created MBS session with initial MBS service area

1a. MBS session update request with new service area

1b. New service area based on UE location report

2. MBS session update with new service area fails

3a. MBS session creation with new service area, and obtain new MBS session ID and ingress address

3. Based on the return information, try to create a new MBS session

4. Annouce the new MBS session, UE join the MBS session if multicast type, UE report listening status.

5. MBS session update response

6. User plane delivery mode

Figure 14.3.4A.3.x-1: MBS service area update failure handling

1.	The NRM server determines to update service area of the existing MBS session. This may be triggered by the MBS session update request with a new service area from the VAL server (step 1a), or the NRM server monitors the UE’s mobility and generates the new service area (step 1b) based on e.g., SEAL LMS mechanism.

2. The NRM servers performs the MBS session update with updated service area towards the 5GC as described in 3GPP TS 23.247 [39]. The MBS session update is partically accepted and a non-supported MBS service area and a indication are returned as described in 3GPP TS 23.247 [39].

3.	Upon receiving the partially accept response, the NRM determines to create a new MBS session with the non-supported service area.

4.	The NRM server performs an MBS session creation procedure with the new MBS service area (i.e., the 5GC non-supported service area) as described in 3GPP 23.247 [39].

4a.	If MBS session creation is successful, the new MBS session ID and new ingress address are returned. The procedure proceeds with step 5 to step 7.

4b.	If the MBS session creation is failed again, the procedure proceeds with step 7.

5.	The NRM server obtains the new MBS session ID and new ingress address of the new MBS session. The NRM server announces the new session information to the corresponding UEs. The UE may join the MBS session if it is multicast type. The UE may report the MBS listening status to the NRM server.

6.	 The NRM server returns the MBS session update response to the VAL server. The new MBS session ID are included. The ingress address of the new MBS session may be also included.

7.	The NRM server also indicates the VAL server to deliver the data over the new MBS session. If the MBS session creation is failed in step 4, the NRM server indicates the VAL server to use unicast delivery.

# /******************** Next Changes ********************/

14.3.2.42	Multicast/broadcast resource update request

Table 14.3.2.42-1 describes the information flow for the multicast/broadcast resource update request from VAL server to NRM server.

Table 14.3.2.42-1: Multicast/broadcast resource update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Identity of the multicast/broadcast resource | M | Identity of the multicast/broadcast resource to be updated |
| Service requirement description | O (NOTE 3) | Indicates the service characteristics, e.g., Media type, Media format, bandwidth requirements, flow description, Application Identifier, Priority indicator, emergency indicator Application service provider. |
| Multicast/Broadcast area information | O(NOTE 3) | Indicate the area where the multicast/broadcast resource is requested for |
| Local MBMS information (NOTE 1, NOTE 2) | O(NOTE 3) |  |
| > MB2-U information | O | IP address, UDP port number of the MB2-U interface |
| > xMB-U information | O | IP address, UDP port number of the xMB-U interface |
| > M1 interface information | O | M1 interface information for local MBMS |
| Local MBMS activation indication (NOTE 1, NOTE 2) | O | Indicates whether to request the NRM server to use Local MBMS information. |
| NOTE 1: The VAL server may provide either the Local MBMS information or the Local MBMS activation indication. This IE is present when the local MBMS is required for VAL services like V2X service. NOTE 2: This information element is only applicable when EPS is specified in the network system indicator IE. NOTE 3: At least one of those IEs shall be present. |  |  |

14.3.2.43	Multicast/broadcast resource update response

Table 14.3.2.43-1 describes the information flow for the Multicast/broadcast resource update response from NRM server to VAL server.

Table 14.3.2.43-1: Multicast/broadcast resource update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the multicast/broadcast resource update request operation. |
| Identity of the multicast/broadcast resource | M (NOTE 1) | The identity of the multicast/broadcast MBS session, or/and the identity of the MBMS bearer to be updated. |
| New Identity of the multicast/broadcast resource | O | The identity of the new multicast/broadcast MBS session created during update. |
| Session description information | O (NOTE 2) | Indicates Multicast/broadcast related configuration information as defined in 3GPP TS 26.346 [28] (e.g. radio frequency and MBMS Service Area Identities) or/and 3GPP TS 23.247 [39] (e.g. MBS FSA ID(s), Area Session ID, service requirements) |
| NOTE 1: This IE may not be required if the session announcement mode indicates that the request is sent by the NRM server. NOTE 2: If the Result Information element indicates failure then the values of the other information elements shall not present. |  |  |