3GPP TSG-SA WG6 Meeting #58	S6-233668

Chicago, US, 13th – 17th November, 2023

Source:	Ericsson

Title:	Pseudo-CR on Key Issue for the Support of policies in Application Layer AI/ML Services

Spec:	3GPP TS 23.700-82 V0.1.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

The AI/ML Services select the AI/ML member(s) to perform the AI/ML operations (regular, federative, disrtributed, etc. learining). In order to assist the AI/ML operations, the AI/ML Sevices require the information about:

-	AI/ML member selection policies, e.g., the AI/ML member can be selected to participate in the AI/ML operation if its throughput is greater than 17 Mbps and/or PLR is less than 10-7.

-	Individual AI/ML member participation policies, the AI/ML member is available to participate an AI/ML operation if:

-	battery level is greater than 80%;

-	it is non-working hours (i.e., from 17:00 to 00:00 on Mon-Fri); and/or

-	at home.

2. Reason for Change

The provisioning and management of the AI/ML policies is an essential functionality to organize the AI/ML operations across multiple heterogeneous AI/ML members and operations. Thus, the KI is needed to study the AI/ML policies provisioning, management, and applicability.

3. Conclusions

This pCR proposes to study the provisioning, management, and applicability of the AI/ML policies in the AIMLAPP layer.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.1.0.

# *** First Change ***

## 5.A	Key issue #A: Support of policies in Application Layer AI/ML Services

AI/ML Services enable the Distributed/Federative Learning over 5G system. The AI/ML Services need have information about conditions when AI/ML entities (e.g., VAL server, AI/ML Enabler Client, AI/ML Enabler Server) are available or not available to participate in the AI/ML operations (e.g., ML model training). In order to assist and organize AI/ML operations (e.g., ML model training), this study needs to determine the required policies provided by AI/ML entities.

This key issue will study the solutions on the below aspect(s) for AI/ML policies provisioning, management, and applicability:

1.	How to support the AI/ML member selection policies.

2.		How to support the AI/ML member participation policies.

3.	How to utilize the AI/ML policies in the AI/ML Application layer Services.

NOTE:	The user consent on the permission to participate the AI/ML operations (e.g., ML model training) in application layer is within SA3 scope.

# *** End of Changes ***