3GPP TSG-SA WG6 Meeting #58	S6-233669

Chicago, US, 13th – 17th November, 2023

Source:	Ericsson

Title:	Pseudo-CR on The AI/ML member participation policies provisioning and management

Spec:	3GPP TS 23.700-82 V0.1.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

AI/ML member participation policies (i.e., the conditions when an AI/ML member is available or not available to participate in the AI/ML operations) are essential for AI/ML Services for member selection in regular, distributed, and federated learning scenarios. These policies can include:

-	Energy policies, e.g., the AI/ML member is able to participate in the AI/LM operations if the battery level is greater than 80%;

-	Time-schedule policies, e.g., the AI/ML member is not able to participate in the AI/LM operations during working hours (i.e., from 9:00 to 17:00 on Mon-Fri); and

-	Location-based policies, e.g., the AI/ML member is able to participate in the AI/LM operations at home only represented by civic address.

-	AI/ML traffic charging policies, e.g., NSWO usage for the AI/ML traffic.

2. Reason for Change

The provisioning and management of the AI/ML member participation policies are essential for the AI/ML Services in regular, distributed, and federated learning scenarios.

3. Conclusions

This pCR proposes the provisioning and management of the AI/ML member participation policies in the AIMLAPP layer.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.1.0.

# *** First Change ***

## 8.A	Solution #A: AI/ML member participation policies provisioning and management

### 8.A.1	General

The following clauses specify procedures, information flows and APIs for Key issue# A to provision the AI/ML member participation policies for the AI/ML Enabler services.

Assumptions:

-	AI/ML member (e.g. AI/ML Enabler Client) is registered on the AI/ML Enabler Server to participate in the AI/ML operations (e.g., ML model training).

NOTE:	The AI/ML member policies provisioning service operation can be a part of the ML client config provisioning procedure defined in clause 8.3.2.

### 8.A.2	Procedures

#### 8.A.2.1	AI/ML member participation policies provisioning and management

Figure 8.3.2-1: AI/ML member participation policies provisioning procedure

1.	The AI/ML member (e.g. AI/ML Client) sends an AI/ML member participation policies provisioning and management request as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enabler server performs an authorization check of the AI/ML member.

3.	If the AI/ML member is authorised, the AI/ML Enabler Server stores the AI/ML member polices and applies the provisioned policies to ongoing and further the AI/ML operations (e.g., ML model training).

4.	AI/ML Enabler Server provides the response to the AI/ML member with a status via AI/ML member participation policies provisioning and management response defined in clause 8.A.3.2.

NOTE:	The service operation defined in clause 8.A.2.1 can be utilized for AI/ML member participation policies management (i.e., update/delete) functionality.

### 8.A.3	Information flows

#### 8.A.3.1	AI/ML member participation policies provisioning and management request

Table 8.A.3.1-1 describes the information flow from the AI/ML member to the AI/ML Enabler server as a request for the AI/ML member participation policies provisioning and management.

Table 8.A.3.1-1: AI/ML member participation policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (NOTE) | The identity of the AI/ML member performing the request. |
| List of policies | M | Identify the list of policies. |
| > List of VAL service ID(s) | M | Identify the list of VAL service IDs for which the policies are applicable. |
| > List of energy-related policies | O | Provides the list of energy-related polices as defined in table 8.A.3.1-2. |
| > List of time-schedule policies | O | Provides the list of time-schedule policies, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |
| > List of location-based policies | O | Provides the list of location-based policies, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| > List of AI/ML traffic connectivity policies | O | Provides the list of the AI/ML traffic connectivity policies, e.g., NSWO usage for the AI/ML traffic. |
| > List of compute-utilization based policy | O | Provides the AI/ML member participation availability based on list of compute (GPU, CPU, AIPU, NPU) utilization based policy, e.g. the AI/ML member may participate the AI/ML operation if the GPU load is less than the given percent. |
| >Mode of member participation | O | Indicates the AI/ML member availability for the participation based on the modes like training (e.g., FL, DML, ML, and/or Split Learning), aggregator, coordinator and/or inference. |
| NOTE: This information element shall not be updated in the AI/ML member participation policies management request. |  |  |

Table 8.A.3.1-2: Energy-related policy

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M (NOTE) | The policy identifier. |
| List of policy conditions | M | Represents the list of policy conditions. |
| > Availability indication | M | Indicates the AI/ML availability status (e.g., available or not available). |
| > Threshold value | M | Represents the threshold value (e.g., battery level or energy consumption). |
| > Threshold matching direction | M | Indicates the threshold matching direction (e.g., above, below, cross). |
| NOTE: This information element shall not be updated in the AI/ML member participation policies management request. |  |  |

#### 8.A.3.2	AI/ML member participation policies provisioning and management response

Table 8.A.3.2-1 describes the information flow from the AI/ML Enabler server to the AI/ML member as a response for the AI/ML member participation policies provisioning and management.

Table 12.3.2.2-1: AI/ML member participation policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |

# *** End of Changes ***