3GPP TSG-SA WG6 Meeting #58	S6-233670

Chicago, US, 13th – 17th November, 2023

Source:	Ericsson

Title:	Pseudo-CR on The AI/ML member selection policies provisioning and management

Spec:	3GPP TS 23.700-82 V0.1.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

Any AI/ML operarion (regular, federative, disrtributed, etc. learining) require to provision the AI/ML member selection policies in order to select the proper AI/ML member that is suitable for the AI/ML operation.

The AI/ML operarion member selection policies may include the following aspects:

-	AI/ML traffic QoS policies, e.g., the AI/ML member can be selected to participate in the AI/ML operation if its throughput is greater than 17 Mbps and/or PLR is less than 10-7.

-	AI/ML capability policies, e.g., the AI/ML member can be selected to participate in the AI/ML operation if it has iOS version 17.0.3 and higher and the hardware models from iPhone 12 with the given software packages installed.

Comment to rapporter:

The reference to table 8.B.3.1-1 in table 8.A.3.1-1 (this pCR) is impemented in pCR S6-233669.

2. Reason for Change

The provisioning and management of the The AI/ML member selection policies are essential for the AI/ML Services in regular, distributed, and federated learning scenarios.

3. Conclusions

This pCR proposes the provisioning and management of the AI/ML member selection policies in the AIMLAPP layer.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.1.0.

# *** First Change ***

## 8.A	Solution #A: AI/ML member selection policies provisioning and management

### 8.A.1	General

The following clauses specify procedures, information flows and APIs for Key issue# A to provision and manage the AI/ML member selection policies for the AI/ML Enabler services.

### 8.A.2	Procedures

#### 8.A.2.1	AI/ML member selection policies provisioning and management

Figure 8.3.2-1: AI/ML member selection policies provisioning procedure

1.	The VAL server sends an AI/ML member selection policies provisioning and management request as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enabler server performs an authorization check of the VAL server.

3.	If the VAL server is authorised, the AI/ML Enabler Server stores the AI/ML member selection policies and applies the provisioned policies to ongoing and further the AI/ML Enabler operations (e.g., ML model training).

4.	AI/ML Enabler Server provides the response to the VAL server with a status via AI/ML member selection policies provisioning and management response defined in clause 8.A.3.2.

NOTE:	The service operation defined in clause 8.A.2.1 can be utilized for AI/ML member selection policies management (i.e., update/delete) functionality.

### 8.A.3	Information flows

#### 8.A.3.1	AI/ML member selection policies provisioning and management request

Table 8.A.3.1-1 describes the information flow from the VAL server to the AI/ML Enabler server as a request for the AI/ML member selection policies provisioning and management.

Table 8.A.3.1-1: AI/ML member selection policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (NOTE 1) | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of AI/ML traffic QoS policies | O | Provides the list of AI/ML traffic QoS policies, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s). |
| List of AI/ML compute capability policies | O | Provides the list of the AI/ML compute capability policies, e.g., the OS, OS version, hardware capabilities, software capabilities. |
| List of AI/ML member mobility policies | O | Provides the list of mobility policies for the AI/ML member, e.g., low mobility, high mobility. |
| List of AI/ML member participation policies | O | Provides the list of the AI/ML member participation policies as defined in table 8.B.3.1-1. (NOTE 2) |
| NOTE 1: This information element shall not be updated in the AI/ML member participation policies management request. NOTE 2: The member participation policies represent the requestor conditions for the member selection based on the policies provided in table 8.B.3.1-1 to be selected to participate into the AI/ML operations. |  |  |

#### 8.A.3.2	AI/ML member selection policies provisioning and management response

Table 8.A.3.2-1 describes the information flow from the AI/ML Enabler server to the VAL server as a response for the AI/ML member selection policies provisioning and management.

Table 12.3.2.2-1: AI/ML member selection policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

# *** End of Changes ***