3GPP TSG-SA WG6 Meeting #58	S6-233726

Chicago, USA 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0486 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Connection/disconnection authorisation procedures (option B) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes to update the current (dis-)connection authorisation procedures in clause 11.5.1 and 11.5.4. The existing procedure is shortened to be MC service and resources check done by the MC gateway UE. The criteria for the resources check done is left to implementation. The binding between the MC client and MC gateway UE is done and controlled by the MC service server, and can be done during the MC user authentication and user service authorisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - update the procedures done in clauses 11.5.2 and 11.5.4.2 - voiding clauses 11.5.1.3 and 11.5.4.3 -editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Avoid confusion in the existing connection authorisation procedure as it depends on the type of non-3GPP connection, and the binding is done at the MC service server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1.1, 11.5.1.2, 11.5.1.3, 11.5.4.1, 11.5.4.2 and 11.5.4.3, 11.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 11.5.1	Connection authorisation mechanisms

#### 11.5.1.1	General

The connection of non-3GPP devices via an MC gateway UE requires connection authorisation verification by the MC gateway UE to perform MC service and resources check and to evaluate whether it is capable of supporting the new request. .

Once the connection authorisation check is done, and the MC gateway UE is capable of supporting the new request, the MC client (either hosted in the non-3GPP device or instantiated at the MC gateway UE) may proceed with MC user authentication and user service authorisation towards the MC service server, where the latter mark the new binding, i.e., the MC client to MC gateway UE association.

#### 11.5.1.2	Connection authorisation for non-3GPP devices that host an MC client

##### 11.5.1.2.1	General

The check is applied to non-3GPP devices which can host an MC client. The MC gateway UE performs a connection authorisation check for the support of the MC gateway client. The binding between the MC gateway UE and the MC client is authorised and controlled by the MC server during the MC user authentication and user service authorisation. The MC gateway client informs MC clients about the connection status.

For the period of association between MC server, MC client and MC gateway UE, the MC server maintains the assignment between MC clients to the MC gateway UE used. This assignment is cancelled again with the disconnection.

##### 11.5.1.2.2	Information flows

###### 11.5.1.2.2.1	Connection authorisation request

Table 11.5.1.2.2.1-1 describes the information flow connection authorisation request sent from the MC gateway client, which resides on a non-3GPP device, to the MC gateway UE.

Table 11.5.1.2.2.1-1: Connection authorisation request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be authorised. |  |  |

###### 11.5.1.2.2.2	Connection authorisation response

Table 11.5.1.2.2.2-1 describes the information flow connection authorisation response sent from the MC gateway UE to the MC gateway client residing on a non-3GPP device.

Table 11.5.1.2.2.2-1: Connection authorisation response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Response | M | Result of the connection authorisation request, service feasibility, and connection evaluation. |

##### 11.5.1.2.3	Connection authorisation procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 11.5.1.2.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The MC gateway client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC gateway client hosted at the non-3GPP device has been provided with an appropriate GW MC service ID.

-	The MC gateway UE has performed service authorisation for one or more MC services with the MC system as described in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12], and 3GPP TS 23.282 [13].

-	The MC gateway client has selected an MC gateway UE or alternatively, the MC gateway client has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

Figure 11.5.1.2.3-1: Connection authorisation check with an MC gateway UE

1.	The MC gateway client requests connection authorisation check towards the MC gateway UE.. The MC gateway client provides the GW MC service ID associated with the requested MC service.

2.	The MC gateway UE checks whether the requested MC service, as indicated by the GW MC service ID, is supported. The MC gateway UE may also check whether sufficient resources are available or if any other local criteria are met.

NOTE:	Further information to the MC gateway UE selection is in Annex D.

3.	If the MC service is supported by the MC gateway UE, the MC gateway UE marks the requested GW MC service ID and proceed with the next step.

4.	The MC gateway UE sends the connection authorisation response to the MC gateway client.

After successful connection with the MC gateway UE, the MC clients may continue with MC user authentication and user service authorisation.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

#### 11.5.1.3	Void

# * * * * Second change * * * *

### 11.5.4	Disconnection mechanism

#### 11.5.4.1	General

A connection using an MC gateway UE by the corresponding MC gateway client can be cancelled over time or re-established using same or another MC gateway UE. The connection/disconnection mechanism allows the MC gateway client to disconnect the use of the corresponding MC gateway UE.

Under certain circumstances, the connection with the corresponding MC gateway UE can change or has to be adjusted. The various reasons are detailed in the informative Annex D. For this purpose, the MC gateway UE can send a notification to the corresponding MC gateway client hosted on a non-3GPP device.

#### 11.5.4.2	Disconnection for non-3GPP devices that host an MC client

##### 11.5.4.2.1	General

The clause is applied to non-3GPP devices which can host an MC client..

##### 11.5.4.2.2	Information flows

###### 11.5.4.2.2.1	Disconnection request

Table 11.5.4.2.2.1-1 describes the information flow disconnection request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE.

Table 11.5.4.2.2.1-1: Disconnection request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |

###### 11.5.4.2.2.2	Disconnection response

Table 11.5.4.2.2.2-1 describes the information flow disconnection response sent from the MC gateway UE to the MC client residing on a non-3GPP device.

Table 11.5.1.2.2.2-1: Disconnection response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Response | M | Result of the disconnection request. |

###### 11.5.4.2.2.3	Connection status notification

Table 11.5.4.2.2.3-1 describes the information flow connection status notification sent from the MC gateway UE to the MC client, which resides on a non-3GPP device.

Table 11.5.4.2.2.3-1: Connection status notification

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the associated MC client. (see NOTE 1) |
| Status information | M | This information element provides connection status. (see NOTE 2). |
| NOTE 1: The GW MC service ID indicates for which MC service the connection is to be disconnected. NOTE 2: Information about the connection status are further detailed in Annex D. |  |  |

##### 11.5.4.2.3	Disconnection procedure

The procedure for disconnection from an MC gateway UE  is shown in figure 11.5.4.2.3-1.

Pre-conditions

-	The MC clients have no communication ongoing, e.g. group communication.

-	The MC gateway client of the MC service user on a non-3GPP device wishes to disconnect an existing binding towards an MC gateway UE.

Figure 11.5.4.2.3-1: Disconnection with an MC gateway UE

1.	The MC gateway client requests disconnection with the MC gateway UE. The MC gateway client of the MC service user provides the GW MC service ID.

2.	The MC gateway UE updates MC gateway client connection status as disconnected.

3.	The MC gateway UE sends the disconnection response to the MC gateway client.

##### 11.5.4.2.4	Connection status notification

The procedure for connection status notification initiated by an MC gateway UE towards an MC gateway client is shown in figure 11.5.4.2.4-1 informs about the status of connection status that may result into a disconnection.

Pre-conditions

-	The use of the MC gateway UE by the non-3GPP device is marked at the MC service server, and the MC client in the non-3GPP device receives MC services via the corresponding MC gateway UE.

-	The MC gateway UE is no longer able to provide the requested service depending on reasons further detailed in Annex D.

Figure 11.5.4.2.4-1: Connection status notification to an authorised MC gateway client

1.	The MC gateway UE wants to disconnect the connection with the corresponding MC gateway client. The MC gateway UE sends connection status notification to the MC gateway client using the corresponding GW MC gateway ID.

2.	The connection status may result that the MC gateway client wants to disconnect the connection with the MC gateway UE (see disconnection in clause 11.5.4.2.3).

#### 11.5.4.3	Void

# * * * * End of changes * * * *