3GPP TSG-SA WG6 Meeting #58	S6-233777

Chicago, USA, 13th – 17th November 2023	(revision of S6-233485)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0466 | rev | 1 | Current version: | 19.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ACM Group configuration management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS, Netherlands Police |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCShAC |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The study TR 23.700-38 has provided a solution on group configuration change between interconnected MC systems where group membership update by authorized user from a partner MC system is described. Introducing this procedure to chapter 10.17 will add a means on this subject to MC architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR adds a clause to chapter 10.17 specifying the procedure of group configuration change between interconnected MC systems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no clarification on how group memberships for interconnected MC service groups can be updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.17.X.4 (new), 10.17.X.5 (new) 10.17.X.6 (new), 10.17.Z (new), 10.17.Z.1 (new), 10.17.Z.1.1 (new), 10.17.Z.1.2 (new), 10.17.Z.1.2.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after CR “Common ACM procdure” |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.17.X.4	Group membership update request

Table 10.17.X.4-1 describes the information flow group membership update request from ACM client to ACM server in the primary MC system, from ACM server of primary MC system to ACM server of partner MC system and from ACM server to ACM client in partner MC system.

Table 10.17.X.4-1: Group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user at the primary MC system who triggers the information update |
| Functional alias | O | The functional alias of the MC service user at the primary MC system who triggers the information update |
| MC service ID | O (see NOTE) | The identity of the MC service user at the partner MC system towards the information update is sent |
| Functional alias | O (see NOTE) | The functional alias of the MC service user at the partner MC system towards the information update is sent |
| MC service group ID | M | Identity of the MC service group |
| MC service ID list | M | List of identities of the MC service users that are affected by this operation |
| Operations | M | Add to or delete from the group |
| NOTE : Either the MC service ID or the functional alias must be present. |  |  |

#### 10.17.X.5	Group membership update response

Table 10.17.X.5-1 describes the information flow group membership update response from ACM client to ACM server in partner MC system, from ACM server in partner MC system to ACM server in primary MC system and from ACM server to ACM client in primary MC system.

Table 10.17.X.5-1: Group membership update response

| Information element | Status | Description |
| --- | --- | --- |
| MC service group ID | M | Identity of the MC service group |
| Result | M | Indicates the success or failure for the operation |

#### 10.17.X.6	Process indication

Table 10.17.X.6-1 describes the information flow of Process indication from ACM server in partner MC system to ACM server in primary MC system and from ACM server to ACM client in primary MC system.

Table 10.17.X.6-1: Process indication

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user requesting for Group membership update and any other valid ACM request sent to partner MC system |
| Acknowledgement | M | ACM request has been received, stored and processing is in progress |

### 10.17.Z	ACM Group configuration management

#### 10.17.Z.1	ACM Group membership configuration

##### 10.17.Z.1.1	General

This clause address the following aspects:

-	The second precondition in 3GPP TS 23.280, clause 10.2.7.2: One or more MC service group members are defined in the partner MC system.

-	ACM entities can be used to manage/change memberships of interconnected MC service groups in partner MC systems.

##### 10.17.Z.1.2	Procedure

###### 10.17.Z.1.2.1	Group membership update by authorized user from a partner MC system

The procedure for an authorized MC service user in a partner MC system to request the primary MC system of the MC service group to modify group membership to an interconnection group is shown in figure 10.17.Z.1.2.1-1.

Pre-conditions

-	MC system A and MC system B are interconnected

-	MC system A and MC system B have implemented an ACMS

-	MC system B is the primary MC system of an interconnection group

-	The authorized MC service user of MC system A (the partner MC system) wants to modify the membership of MC users of system A in the interconnection group for which MC system B is the primary MC system.

-	When a functional alias is used as the target of the request by an ACM client in the MC system A, it is required that the ACM server of MC system A has subscribed to the MC service functional alias controlling server within the MC system A, and that the ACM server of MC system B has subscribed to the MC service functional alias controlling server within the MC system B.

Figure 10.17.Z.1.2.1-1: Group membership update by authorized user from a primary MC system

1.	The ACM client in primary MC system sends a group membership update request to the ACM server in his MC system, requesting to modify the membership of users from primary MC system an interconnection group for which partner MC system is the primary MC system.

2.	The ACM server of  the primary MC system checks whether the MC service user at the ACM client is authorized for the request.

3.	The ACM server of the primary MC system sends the group membership update request to the ACM server of the partner MC system, the primary MC system of the interconnection group.

4. 	The ACMS of the partner MC system sends a process indication to primary MC system, and stores, verifies and assesses the incoming request.

4a.	Based on local policy, the ACMS of partner MC system may automatically handle the request as described in clause 10.17.Y Common ACM procedure and sends a response to the ACMS of primary MC system (i.e. perform step 8).

4b.	Otherwise, the ACM server requests verification by an ACM client, following the pending request procedure in clause 10.17.Y Common ACM procedure if the user is not logged on, and steps 5, 6 and 7. If the request targets a functional alias the ACM server resolves the functional alias (based on the current state of the functional alias) and proceeds as follows:

-	if the functional alias is active for one ACM client, the request is sent to the ACM client;

-	if the functional alias is active for more than one ACM client, the ACM server sends the request to one of these ACM clients based on the local policy;

-	if the functional alias is not currently active, the ACM server stores the request.

5.	The ACM server of partner MC system sends the stored group membership update request to the ACM client of partner MC system.

6.	The authorized MC service user of partner MC system (manually) screens the content of the group membership update request and decides to approve it or not. The authorized user in the partner MC system takes whatever actions are needed to apply the group membership update. The actions within the partner MC system to apply this configurations are outside the scope of the 3GPP specifications.

7.	The ACM client of partner MC system sends a group membership update response to the ACM server of partner MC system.

8.	The ACM server of partner MC system sends the group membership update response to the ACM server of primary MC system.

9.	The ACM server of primary MC system stores the group membership update response. If the ACM client of primary MC system is not logged on the ACMS of primary MC system it will follow the pending request procedure as described in clause 10.17.Y. Common ACM procedure.

10.	The ACM server of primary MC system sends the group membership update response to the ACM client of MC system A.

* * * End of Change * * * *