3GPP TSG-SA WG6 Meeting #58	S6-233906

Chicago, USA, 13th – 17th November 2023	(revision of S6-233514)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0526 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Continuity for ENS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Service continuity for ENS is not supported yet in the EDGEAPP archtiecture. As per the objective of the WID, it is required to support the feature. Basic service continuity scenarios are already supported in the TS 23.558. Only difference between basic ACR scenarios and ACR for ENS case is that in basic scenarios, S-EAS (from where AC is taking service) is directly connected/registered to S-EES and all interections are happening over EDGE-3 interface. However, in case of ENS, the S-EAS is registered to EES (of partner ECSP) over EDGE-3 and S-EES interects with EES (of partner ECSP) over EDGE-9 interace. So, we need to enable all communication from S-EAS to S-EES via EES (of partner ECSP). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new clauses to describe reuse of existing procedures with the enhancements required to support service continuity for the ENS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service continuity for ENS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.18.2.x (NEW), 8.18.2.x.1 (NEW), 8.18.2.x.2 (NEW), 8.18.2.x.3 (NEW), 8.18.2.x.4 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.18.2.x	Service continuity procedures for ENS via leading ECSP

##### 8.18.2.x.1	ACR launching procedure

The S-EES uses procedure as defined in clause 8.8.3.4 to forward the ACR request with ACR action indicating ACR initiation from EEC towards partner EES (P-EES) over EDGE-9 interface. The request includes ECSP ID of the lead ECSP for partner ECSP.

##### 8.18.2.x.2	Selected T-EAS declaration

Upon receiving the Selected target EAS declaration request message, the EES ueses the clause 8.8.3.7 to send the request to the EES of the lead ECSP over EDGE-9 interface including partner ECSP ID.

##### 8.18.2.x.3	ACR status update procedure

Upon receving ACR status update request from the EAS, if the EAS is serving the UE of the lead ECSP (i.e. edge node sharing case), the EES uses the procedure as defined in clause 8.8.3.8 to send the request to the EES of the lead ECSP over EDGE-9 interface including partner ECSP ID. Upon receiving the response from EES of the lead ECSP, the EES sends the response back to the EAS.

##### 8.18.2.x.4	EAS Information provisioning

Upon receving EAS Information provisioning request from the EEC, if the request contains selected EAS ID and selected EAS Endpoint and if the selected EAS is registered to the EES of the partner ECSP (i.e. Edge Node Sharing scenario), then the EES uses the procedure as defined in clause 8.15.2.2 to send the EAS Information provisioning request to the partner EES over EDGE-9 interface to indicate about selection of the EAS for the service consumption. The request includes EESID and EES endpoint address, and lead ECSP ID along with the information shared by the EEC.

* * * End of Change * * * *