3GPP TSG-SA WG6 Meeting #58	S6-233984

Chicago, USA, 13th – 17th November 2023	(revision of S6-233619)

Source:	Convida Wireless LLC

Title:	KI#3 solution F/AIML client registration

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

Quang Ly, ly.quang@convidawireless.com

1. Introduction

This contribution provides a solution on F/AIML client registration for KI #3 support for federated learning

2. Reason for Change

Proposes a solution for KI #3 support for federated learning. The F/AIML notation corresponds to a new (SA6 #58) architectural enhancement proposal which distinguishes between AI/ML functionality provided via enahncements to existing enablers (e.g. MTME-enhanced ADAE) and a stand-aline F/AIML enabler.

This solution is different than solution #3 in that F/AIML clients may have temporal and spatial bounds that ML clients may not. Updates are made to solution #3 to refer to MTME-enabled ADAE clients rather than ML clients.

In ML processing, datasets are stored and maintained in a central location and controlled by data scientists or owners. Data is processed centrally with a complete view of the features and characteristics of the data. This scenario lends itself to implementation via enahncements to existing enablers, e.g. MTME-enhanced ADAE.

In federated learning, data are collected, processed, and used for training or inferencing by individual F/AIML clients. Data scientists do not have visibility into the type of data an F/AIML client has, such as a dataset’s features, format, size, and other characteristics that may be important for a particular ML application. Hence, federated learning requires more management operations such as F/AIML client registration, discovery, selection, and monitoring.

In addition, data are being collected and processed by F/AIML clients to preserve data privacy and hence information about dataset availability, capabilities, sizes, format, age, etc. need to be taken into consideration for a particular ML/FL application. A VAL server or F/AIML client wanting to configure federated training or inferencing needs to be able to discover F/AIML clients that are able to provide the data needed for federated operations. Without this information, federated training or inferencing will generate invalid models or inferencing results.

This contribution also proposes changes to the exiting solution #3 to distinguich MTME-enhanced ADAE functionality from F/AIML functonality.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82.

* * * First Change * * * *

# 8	Solutions

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #3 |  | X |  |  |
| Sol #X |  |  | X |  |

* * * Next Change * * * *

## 8.3	Solution #3: Provision of MTME-enhanced ADAE clients to support AI/ML at the application layer

### 8.3.1	General

The following clauses specify procedures, information flows and APIs for Key issue 2 to provision the MTME-enhanced ADAE clients for the AIML Enabler services.

Assumptions:

1.	The proposed solution is based on the ADAE functional architecture enahncements to support AIML services. The service producer can be collocated with SEAL entity like an ADAE server or part of the ADAE server. The service consumers could be VAL servers, VAL UEs, or any other entities that consume MTME-enhanced ADAE services.

### 8.3.2	Procedures

Figure 8.3.2-1: MTME-enhanced ADAE client config provisioning procedure

1.	The MTME-enhanced ADAE service consumer (e.g. VAL server or VAL UE) sends a MTME-enhanced ADAE client config provisioning request to the MTME-enhanced ADAE service producer (e.g. ADAE server). The request may contain UE ML client profile e.g., UE compute capability, ML capability, UE QoS.

Editor's Note 1: How ML application QoS requirements and UE client profile will be used is FFS.

Editor's Note 2: The contents of the request are high-level and can be changed and is FFS.

2.	Upon receiving the request, the MTME-enhanced ADAE service producer performs an authorization check. If authorization is successful, the MTME-enhanced ADAE service producer registers the client and provides the response to the MTME-enhanced ADAE service consumer with a registration ID.

Editor's Note 3: The solution is subject to change based on the architecture and is FFS.

Editor's Note 4: The contents of the response can be changed and is FFS.

* * * Next Change * * * *

## 8.X 	Solution #X: AIML enablement client registration

### 8.X.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client registration.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The AIML enablement client has been pre-configured or has discovered the address (e.g. URI) of the AIML enablement server.

3.  The AIML enablement client has been pre-configured with an AIML enablement policy.

Figure 8.x.1-1: AIML enablement client registration procedure

- The AIML enablement client sends a registration request to the AIML enablement server, the registration request includes a AIML enablement client identifier, security credentials, and an AIML enablement policy including: application identifier, AIML enablement role, supported ML application types (e.g. supervised, unsupervised learning) and models/algorithms (e.g. neural networks, gradient boost), AIML enablement client consent and availability, AIML enablement client capabilities (e.g. computing and storage capabilities), and dataset characteristics (e.g. available datasets, dataset size, data format, dataset age).

Editor’s Note: Other IEs in the request are FFS.

2.	The AIML enablement server validates the registration request and verifies the security credentials. The AIML enablement server further performs an authentication and authorization check to determine if the AIML enablement client is permitted to register to the F/AIML server and participate in federated learning operations. The AIML enablement server assigns an identifier for the AIML enablement policy to associate it with the AIML enablement client.

3.	The AIML enablement server returns an AIML enablement registration response to the AIML enablement client with the status of the registration request and the identifier associated with the AIML enablement policy.