3GPP TSG-SA WG6 Meeting #58	S6-233985

Chicago, USA, 13th – 17th November 2023	(revision of S6-233621)

Source:	Convida Wireless LLC

Title:	KI#3 solution F/AIML client discovery

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

Quang Ly, ly.quang@convidawireless.com

1. Introduction

This contribution provides a solution on F/AIML client discovery for KI #3 support for federated learning

2. Reason for Change

Proposes a solution for KI #3 support for federated learning. The F/AIML notation corresponbds to a new (SA6 #58) architectural enhancement proposal.

In federated learning, a data scientist does not have visibility into available datasets provided by F/AIML clients and hence needs to first discover F/AIML clients with certain FL capabilities, including having the right types of data required for the ML application. As a result, the data scientist (e.g. via a VAL server) needs to discover F/AIML clients that have federated learning capabilities required for the ML model as well as the availability of the required dataset.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82.

* * * First Change * * * *

# 8	Solutions

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #Y |  |  | X |  |

* * * Second Change * * * *

## 8.Y 	Solution #Y: AIML enablement client discovery

### 8.Y.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client discovery.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The AIML enablement server has a repository of AIML enablement clients and corresponding information of federated learning capabilities and datasets.

Figure 8.y.1-1: AIML client discovery procedure

1.	An AIML consumer sends a request to an AIML enablement server to discover AIML enablement clients that are available to participate in federated learning operations, e.g. is available and have required data to train an ML model. The AIML enablement client discovery request includes client/server identifier, security credentials, and discovery criteria that include application identifier, AIML role, ML application type (e.g. supervised, unsupervised learning) and models/algorithm (e.g. neural networks, gradient boost), target location, AIML enablement client capabilities (e.g. computing and storage capabilities), and dataset characteristics (e.g. available datasets, dataset size, data format, dataset age).

Editor’s Note: Other IEs in the request are FFS.

2.	The AIML enablement server validates the discovery request and verifies the security credentials provided in the discovery request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to discover AIML enablement clients. If authorized, the AIML enablement server determines a list of AIML enablement clients that meets the discovery criteria.

NOTE: The AIML enablement server may perform Member UE selection assistance procedure from TS 23.502, section 4.15.13 to obtain a list of UEs capable of AIML enablement operations, e.g. federated learning. The AIML enablement server then determines a list of UEs that fulfils the discovery criteria.

3.	The AIML enablement server sends an AIML enablement client discovery response that includes the status of the discovery request and the list of AIML enablement clients matching the discovery criteria.