3GPP TSG-SA WG6 Meeting #58	S6-234037

Chicago, USA, 13th – 17th November 2023	(revision of S6-233559, 3977)

Source:	Lenovo

Title:	Analysis of AI/ML support in 3GPP

Spec:	3GPP TR 23.700-82 v0.1.0

Agenda item:	8.3

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper covers the stage 1 and stage 2 work in 3GPP which is related to AI/ML support.

2. Reason for Change

Clause 4 will provide the analysis of existing work in 3GPP on AI/ML. This paper provides the general description of the stage 1 and stage 2 as well as the required analysis which motivates the investigaiton of enablement services related to AI/ML support.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.1.0.

* * * First Change * * * *

# 4	Analysis of AI/ML support in 3GPP

## 4.1	AI/ML related use cases and requirements in 3GPP

### 4.1.1	Description

3GPP SA1 has identified use cases and requirements (in TS 22.261[10]) for the support of AI/ML model distribution, transfer, training for various applications (e.g., video/speech recognition, robot control, automotive), also covering the support for Distributed AI training/inference based on direct device connection.

The use cases provided in TS 22.261 are covering:

-	AI/ML operation splitting between AI/ML endpoints: The AI/ML operation/model is split into multiple parts according to the current task and environment.

-	AI/ML model/data distribution and sharing over 5G system: multi-functional mobile terminals might need to switch the AI/ML model in response to task and environment variations. Hence, 5GS needs to support the distribution and sharing of the model.

-	Distributed/Federated Learning over 5G system: The cloud server trains a global model by aggregating local models partially trained by each end devices.

Based on these use cases, there are Stage 1 requirements which are relevant for application enablement..

### 4.1.2	Analysis

Editor’s Note: The analysis of stage 1 requirements which are relevant to SA6 is FFS, based on the progress of the Key Issues.

## 4.2	AI/ML related stage 2 work in 3GPP

### 4.2.1	Description

#### 4.2.1.1 3GPP SA2

3GPP SA2 started from Rel-16 to investigate the AI/ML support focusing on the main two aspects.

1) AI-ML support in NWDAF

Network analytics and AI/ML is deployed in the 5G core network via the introducing of NWDAF consider the support of various analytics types that can be distinguished using different Analytics IDs, e.g., “UE Mobility”, “NF Load”, etc. as elaborated in TS 23.288 [2]. Each NWDAF may support one or more Analytics IDs and may have the role of: (i) AI/ML inference called NWDAF AnLF, or (ii) AI/ML training called NWDAF MTLF or (iii) both.

Figure 4.2.1-1: NWDAF architecture for analytics generation based on trained models

Figure 4.2.1-1 illustrates the various NWDAF deployment flavours and their respective input data sources and the potential consumer for analytics output results. Specifically, NWDAF relies on various sources of data input including data from 5G core NFs, AFs, 5G core repositories, e.g., NRF, UDM, etc., and OAM data, including performance measurements (PMs), KPIs, configuration management data and alarms. An NWDAF may provide in turn analytics output results to 5G core NF, AFs, and OAM. Optionally, DCCF and MFAF may be involved to distribute and collect repeated data towards or from various data sources.

For FL use cases, in Release 18, 3GPP defined federated learning amongst different NWDAF MTLFs where ML model training is running in multiple local MTLFs.

2) Network assistance for AI-ML services

As part of AIMLSys, enhancements to 5GC (as specified in TS 23.501 [5] clause 5.46) are specified for assisting the AI/ML operations in the application layer (between one or more AI/ML users and AI/ML server). In this direction, NEF may assist the AI/ML application server in scheduling available UE(s) to participate in the AI/ML operation (e.g. Federated Learning). Also, 5GC may assist the selection of UEs to serve as FL clients, by providing a list of target member UE(s), then subscribing to the NEF to be notified about the subset list of UE(s) (i.e. list of candidate UE(s)) that fulfil certain filtering criteria.

#### 4.2.1.2 3GPP SA4

SA4 has an ongoing study on AI and ML for Media (TR 26.927 [11). This study aims to identify relevant interoperability requirements and implementation constraints of AI/ML in 5G media services. This study includes media-based AI/ML use cases and architecture considerations related to media services.

#### 4.2.1.2 3GPP SA5

SA5 in Rel-17 has provided a work on AI/ML management (TS 28.105 [9]) focusing on AI/ML capabilities (e.g. ML training MnS/MF) at OAM side. TS 28.105 specifies the AI/ML management capabilities and services for 5GS where AI/ML is used, including management and orchestration (e.g. MDA, see 3GPP TS 28.104 [3]). In this work, an ML training Function (MLT) playing the role of ML training MnS producer is introduced which may consume various data from 5GS for ML training purpose and provides the training outputs to other management functions in OAM.

### 4.2.2	Analysis

Editor’s Note: The analysis of stage 2 work which is relevant to SA6 is FFS.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[9]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[10]	3GPP TS 22.261: "Service requirements for the 5G system".

[11]	3GPP TR 26.927: “Study on Artificial Intelligence and Machine learning in 5G media services”.