3GPP TSG-SA WG6 Meeting #58	S6-234049

Chicago, USA, 13th – 17th November 2023	(revision of 3913 was 3571)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0146 | rev | 2 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service API update for CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eCAPIF |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The interactions among CCFs in a CAPIF interconnection scenario are incomplete. In the current version of the specification it is only possible for a CCF to publish service API to another CCF. It is needed to allow the update operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Information flow and information elements for service API update in a CAPIF interconnection are introduced - Include CCF as a known consumer of the Update_Service_API operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missalignment with stage 3 Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.2.X (New), 8.25.2.Y (New), 8.25.3.Z (New), 10.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 includes eCAPIF in the list of Work item codes and inclusion of missing information in Table 8.25.2.X-1 Revision 2: removed the reason IE in Table 8.25.2.X |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 8.25.2.X	Interconnection update service API request

Table 8.25.2.X-1 describes the information flow interconnection update service API request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.X-1: Interconnection update service API request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CCF requesting the update operation. It may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional and applicable only on CAPIF-6 interface), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, Service KPIs (optional). |
| Service API category | O (see NOTE 1) | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |
| NOTE 1: At least one of the Service API information or Service API category shall be present. |  |  |

* * * * Next Change * * * *

#### 8.25.2.Y	Interconnection update service API response

Table 8.25.2.Y-1 describes the information flow interconnection update service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.Y-1: Interconnection update service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

* * * * Next Change * * * *

### 8.25.3.Z	Update service APIs for CAPIF interconnection

Figure 8.25.3.Z-1 illustrates the procedure for updating the published service APIs information by a CAPIF core function.

Pre-conditions:

1.	CCF-A and CCF-B connect to each other, and either belong to the single trust domain of the same CAPIF provider or trust domains of different CAPIF providers. When CCF-A and CCF-B belong to trust domains of different CAPIF providers, the two CAPIF providers have business agreement for service API sharing.

2.	CCF-B is configured as the designated CAPIF core function in the trust domain of CAPIF provider A.

Figure 8.25.3.Z-1: Update service APIs for CAPIF interconnection

1.	The CCF-A sends an interconnection update service API request to the CCF-B, including the information in table 8.25.2.X-1.

2.	Upon receiving the interconnection update service API request, the CCF-B checks whether the CCF-A is authorized to update the published service APIs information. If the check is successful, the service API information provided by the CCF-A is updated at the CCF-B (API registry).

3.	The CCF-B provides an interconnection update service API response to the CCF-A containing the result of the update operation and triggers notifications to subscribed API invokers (if any) as described in subclause 8.8.4.

* * * * Next Change * * * *

### 10.3.4	Update_Service_API operation

API operation name: Update_Service_API

Description: Update the published service API information.

Known Consumers: API publishing function, CAPIF core function.

Inputs: Refer subclause 8.6.2.1, 8.25.2.X.

Outputs: Refer subclause 8.6.2.2, 8.25.2.Y.

See subclause 8.6.3 and 8.25.3.Z for the details of usage of this API operation.

* * * * End of Changes * * * *