3GPP TSG-SA WG6 Meeting #58	S6-240081

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

Source:	Vodafone

Title:	Discussion of instigating ACR at the edge enabler server (EES)

Agenda Item:	9.5

Contact:	peter.dawes@vodafone.com

# Introduction

TS 23.558 includes APIs and procedures to enable context relocation e.g., if a UE moves or if an application server needs to load balance. A trigger event that can be detected by the detection entity is needed to instigate such re-location.

Although TS 23.558 describes the EES as an entity that can play the detection role, no existing procedure allows the EES to instigate ACR, for example by monitoring UE location. This discussion paper summarizes TS 23.558 text related to the ACR detection role and proposes a new ACR management type that enables an EES to autonomously manage ACR.

# TS 23.558 Roles and Procedures

Detection Entity

The EEC, EES and EAS can potentially perform the detection role, as per the description of detection entity in TS 23.558 clause 8.8.1.1:

A detection entity detects the probable need for ACR by monitoring various aspects, such as UE's location or predicted/expected UE location and indicates to the decision-making entity to determine if the ACR is required. The EEC, EES and EAS can potentially perform the detection role:

Typically UE location is used as the trigger event. Although the EES is listed as a potential detection entity, e.g. in clause 8.8.2.5 S-EES executed ACR, TS 23.558 does not include any procedure steps that enable an EES to detect a trigger event such as UE location. Although many procedures in TS 23.558 include the EES passing UE location requests to the 5G core, the EES is not specified to subscribe to UE location requests itself (also, it is not listed as a consumer of the UELocationAPI in Table 8.6.2.4.1-1).

User Plane Path Change as the Detection Event

TS 23.558 clause 8.6.3.2.2	Subscribe in clause 8.6.3 ACR management events includes the following text:

The EES subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE(s) as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]

And user plane path management events are listed in TS 23.502 clause 4.3.6.3 as below.

(extract from TS 23.502)

#### 4.3.6.3	Notification of User Plane Management Events

The SMF may send a notification to the AF if the AF had subscribed to user plane management event notifications as described in clause 4.3.6.2 and in clause 5.6.7 of TS 23.501 [2]. The following are the examples of such events:

-	A PDU Session Anchor identified in the AF subscription request has been established or released.

-	A DNAI has changed.

-	The SMF has received a request for AF notification and the on-going PDU Session meets the conditions to notify the AF.

-	Ethernet PDU Session Anchor Relocation as defined in clause 4.3.5.8.

-	Candidate DNAI(s) has changed.

-	A common EAS has changed.

The SMF uses notification reporting information received from PCF to issue the notification either via an NEF (2a, 2b and 4a, 4b) or directly to the AF (2c and 4c).

(end of extract)

These user plane management events occur due to a request from an external entity, e.g. an external PDU session modification request that changes the endpoint (DNAI) of a session. An EES alone will not detect the need for ACR if it relies on user plane management events to initiate context relocation.

UE location as an ACR Trigger

An EEC (UE) can monitor its location and request PDU session modification (or can be triggered by an EAS to do so) based on knowledge of UE location and EAS service area. Such a modification can cause a user plane management event, which can trigger further procedures. In the case of an EES, TS 23.558 does not require the EES to monitor UE location and therefore an EES will not trigger a PDU session modification.

EES Execution of Context Relocation

TS 23.558 clause 8.8.2.5 S-EES executed ACR describes the S-EES detecting, deciding and executing ACR from the S-EAS to the T-EAS. Step 2 describes detection but does not indicate what the EES detects. Step 1 includes the text "The S-EAS may initiate EELManagedACR with S-EES as specified in clause 8.8.3.6" but clause 8.8.3.6 simply refers back to clause 8.8.2.5 in the text "3. The S-EES determines the EELManagedACR operations to be executed as specified in clause 8.8.2.5".

Therefore the procedure in 8.8.2.5 does not enable the EES to perform ACR alone but relies on the S-EAS to instigate the process (by sending an EELManagedACR request).

# Proposal

It is proposed to add a new ACR management event called "ACR control" that enables an EES to autonomously instigate application context relocation. Instead of the EES detecting user plane path change using API/event of TrafficInfluence/UP_PATH_CHANGE, an "ACR control" indication to the EES causes the EES to monitor UE location using API/Event of MonitoringEvent/AREA_OF_INTEREST. The AREA_OF_INTEREST is the service area information provided by an EAS when that EAS registers with the EES. By monitoring AREA_OF_INTEREST, the EES removes from the UE and EAS the responsibility of monitoring UE location.

A related CR is submitted to this meeting.