3GPP TSG-SA WG6 Meeting #61	S6-242052

Jeju Island, South Korea, 20th – 24th May 2024

Source:	InterDigital Inc.

Title:	Pseudo-CR on Solution 25 update

Spec:	3GPP TR 23.700-82

Agenda item:	8.3

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

The pCR provides an update to Solution 25.

2. Reason for Change

Solution 25 on AIML model distribution has been proviously approved.

The following changes are proposed for completeness:

- Clause 8.25.4: Added missing corresponding APIs

- Clause 8.25.5: Added missing solution evaluation

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.4.0.

* * * First Change * * * *

### 8.25.1	Solution description

This solution is for Key Issue #5 and addresses AIML model distribution (e.g., transfer) management and configuration; the solution also addresses Key Issue #3 (open issue #5) on distribution of the model information to FL members.

A UE may need to switch an AIML model adaptively based on task and environment variations [10]. Since AIML models are diverse and the UE has limited storage (e.g., cannot store all possible AIML models), it can be determined that a UE may benefit from AIML model distribution from a network endpoint. In other words. an AIML model can be distributed from a NW endpoint to a UE when the UE needs it to adapt to the changed AIML tasks and environments.

The AIML enablement layer can provide AIML distribution capabilities as described in this solution

* * * Next Change * * * *

### 8.25.4	Corresponding APIs

#### 8.25.4.1	API Overview

This clause provides a summary on the corresponding API for solution #25.

-	Discover AIML model profiles API (subscribe/notify model: API provider: AIMLE Server, known consumers: AIMLE client, corresponding to step 1, 4 and 5 of clause 8.25.2.1).

-	Discover AIML models API (subscribe/notify model: API provider: AIMLE Client, known consumers: VAL client, corresponding to step 0 and 6 of clause 8.25.2.1).

-	Discover AIML models profiles API (subscribe/notify model: API provider: Model Repository, known consumers: AIMLE server, corresponding to step 2 and 3 of clause 8.25.2.1).

* * * Next Change * * * *

### 8.25.5	Solution evaluation

The proposed solution addresses Key Issue #5 for AIML model distribution. The solution introduces the subscribe/notify procedures for AIML model discovery.

The solution allows a VAL client to discover, be notified ans obtain AIML model(s) that may be provided by the AIMLE server; the discovered models may be used for performing inference or training.

The solution is based on the AIML enablement layer architecture, including an AIML enablement client, an AIML enablement server and a Model Repository.

* * * End of Changes * * * *