3GPP TSG-SA WG6 Meeting #61	S6-242072

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Convida Wireless

Title:	XRApp update solution 05 policy use alignment

Spec:	TR 23.700-23 v0.3.0

Agenda item:	8.5

Document for:	Approval

Contact:	Mladin.Catalina@convidawireless.com

1. Introduction

The solution for  SEALDD Policy Configuration for multi-modal flows describes configuration of a Multi-modal SEALDD policy, which includes burst and synchronization policy. In discussions around the policy solution, it was determined that alignment of policy use with other solutions is recommended.

2. Reason for Change

Align the SEALDD Policy Configuration with solution #6 in the use of policy parameters.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v0.3.0

* * * First Change * * * *

## 7.6	Solution #6: Multi-modal flows alignment and monitoring

### 7.6.1	Architecture Impacts

This solution requires a mechanism for data storage to cache the traffic flows.

### 7.6.2	Solution description

#### 7.6.2.1	General

Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that have not arrived yet.

To avoid the downlink traffic flow delay difference introduced from (R)AN and Internet, the SEALDD/XRApp client could do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred to application client at specific time.

As shown in figure 7.6.2.1-1, flow#1 and flow#2 are associated flows. They are sent out from VAL server at the same time, while flow#1 arrives at SEALDD/XRApp client earlier than flow#2 due to different routing path. To align those associated flows, the SEALDD/XRApp client holds and buffers the flow#1 for a while. Until all the associated flows arrived(i.e., flow#2), the SEALDD/XRApp client sends out those associated flows to application client at the same time.

Editor’s note: Whether the implementation entity is XRApp or SELADD depends on the Application enablement architecture.

Figure 7.6.2.1-1: DL traffic flow alignment

#### 7.6.2.2	Procedure of flow alignment

Figure 7.6.2.2-1illustrates the procedures of multi-modal flows alignment.

Precondition:

1.	The VAL server can discover and select the XRApp server by CAPIF functions.

2.	The connection has been established among the UE, XRApp server and VAL server and a multi-modal XR policy has been configured, including burst and synchronization policies.

SEALDD/XRApp client

5GC

SEALDD/XRApp server

1. multi-modal flows coordination request

VAL server

5. flows coordination

4. multi-modal flows coordination response

connection established

2. multi-modal flows coordination request

3. multi-modal flows coordination response

Application client

Figure 7.6.2.2-1: multi-modal flows alignment

- VAL server sends the multi-modal flows alignment request with VAL ID, UE ID(s), flows description (e.g., Multi-modal Service ID, packet filter description, etc.), flows transmission requirement, flows alignment assistance information. Flows transmission requirement including the delay requirement. Flows alignment assistance information may include the timestamp (e.g., timestamp in the RTP header), number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server sends the multi-modal flows alignment request to the SEALDD/XRApp client identified by the UE ID(s), along with the information received in step1.

If the maximum acceptable duration for traffic flow alignment is not provided, then the XRApp server would determine the maximum acceptable duration for traffic flow alignment based on flows transmission requirement. The server may translate the flows description into SEALDD flow ID.

- The SEALDD/XRApp client sends the multi-modal flows alignment response to the SEALDD/XRApp server.

- The SEALDD/XRApp server sends the multi-modal flows alignment response to the VAL server indicating the result of multi-modal flows alignment.

- When the traffic flow arrived, the SEALDD/XRApp client initiate the multi-modal flows alignment based on the received information. The flows needing to be aligned are detected based on  VAL ID, flows description and the associated synchronization policy. If the number of associated flows is provided, the number of associated flows is used to determine whether all the associated flows arrived. If the timestamp is provided, the flows that are sent at specific time will be aligned. After all associated flows arrived, the SEALDD/XRApp client sends the flows to the application client. If the maximum acceptable time duration is provided, once the maximum acceptable time is reached, the SEALDD/XRApp client will no longer wait for the associated flows even if the associated flow has  not arrived yet.

If a burst policy has been provided, packets in a burst are assigned the same PDU set before forwarding the messages to the Core Network, enabling burst handling via the PDU set feature in the Core Network.

NOTE 1: Step3 and step5 can be executed in parallel.

NOTE 2: In addition to burst handling via the 5GC PDU set feature, it is assumed that store-and-forwarding based synchronization of messages is performed  in a burst aware manner so that all the messages in a burst which span across different flows arrive at the destination in a synchronized fashion.

#### 7.6.2.3	Procedure of flow alignment monitoring

Figure 7.6.2.3-1illustrates the procedure of multi-modal flows alignment monitoring.

Precondition:

SEALDD/XRApp client

5GC

SEALDD/XRApp server

1. multi-modal flows coordination monitoring request

VAL server

5. flows coordination monitoring

7. multi-modal flows coordination monitoring notification

multi-modal flows alignment

2. multi-modal flows coordination monitoring request

4. multi-modal flows coordination monitoring response

6. multi-modal flows coordination monitoring notification

3. multi-modal flows coordination monitoring response

-	The multi-modal flows alignment has been requested and triggered.

Figure 7.6.2.3-1: traffic flow alignment monitoring

- VAL server sends the multi-modal flows alignment monitoring request. VAL ID, UE ID, flows description, monitoring period are provided in the request.

- Upon receiving the request, the SEALDD/XRApp server performs an authorization check. If authorization is successful, the SEALDD/XRApp server sends the multi-modal flows alignment monitoring request to the SEALDD/XRApp client.

- The SEALDD/XRApp client sends the multi-modal flows alignment monitoring response to the SEALDD/XRApp server.

4.	The SEALDD/XRApp server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

5.	The XRApp client calculates the end-to-end delay difference based on the arrival time and timestamp(e.g., timestamp in the RTP header) among the flows.

Editor’s note: The detail of the end-to-end delay difference calculation is FFS.

6-7.XRApp client sends the multi-modal flows alignment monitoring result to the XRApp server, then to the VAL server.

### 7.6.3	Solution evaluation

This solution addresses KI#2 and KI#8 to support alignment of flows within a multi-modal flow. The solution describes how the service layer enables the based its configuration and policies, and how enables use of existing CN functionality for this purpose.

* * * End Changes * * *