3GPP TSG-SA WG6 Meeting #61	S6-242103

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

Source:	Huawei, Hisilicon

Title:	Solution on Satellite edge computing considering EAS onboard

Spec:	3GPP TR 23.700-01 v0.3.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes a solution on Satellite edge computing.

2. Reason for Change

In the Key issue #2, there is an open issue "When/how could EAS discovery by the EEC in EDGEAPP be optimized?"

Due to the EAS on MEO and LEO satellites, the EAS(s) are mobile as the satellites are moving relative to the Earth's surface. In this case, if the ephemeris information of the EAS is not considered for the discovery, frequent interruption, ACR, and/or service degradation may be experienced when there is a mismatch between the movement of the UE and the movement of the EAS on board satellite.

Thus, the EAS discovery procedure should be optimized.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v0.3.0.

* * * First Change * * * *

7.X.1	Solution #1: Satellite edge computing considering EAS onboard

7.X.1.1	Solution description

7.X.1.1.1	General

Figure 7.2.1.1.1-1: EAS onboard satellite deployment

In this deployment option, ECS and EES are deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The UPF to access EAS is deployed on satellite. RAN (e.g. gNodeB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF. The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. UPF can be deployed on ground to access the ECS and EES, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space), and EEC (in UE) can reach the EES by EDGE-1 (on ground).

Considering that EAS(s) deployed in multiple MEO or LEO satellites, so that the EAS(s) are mobile as the satellites are moving relative to the Earth's surface. In this case, if the ephemeris information of the EAS is not considered for the discovery, frequent interruption may be experienced when there is a mismatch between the movement of the UE and the movement of the EAS on board satellite.

As for the EAS on MEO and LEO satellites however the EES is on the ground, in the EAS discovery procedure, the EES can determine a list of EAS based on the UE (predicted/expected) location, Prediction expiration time, EAS Satellite ID and EAS ephemeris information. Upon receiving a list of EAS information, the UE can dertermine the EAS based on the UE (predicted/expected) location, Prediction expiration time, EAS Satellite ID and EAS ephemeris information, and UE may sequentially connect to corresponding EAS based on ephemeris information of the EASs. When an EAS is about to leave the UE’s location, the UE may connect to a next EAS in advance to obtain an edge computing service, and does not need to perform EAS discovery again. In doing so, the service interruption caused by periodic EAS discovery can be avoided.

* * * NEXT Change * * * *

7.X.1.2	Corresponding APIs

7.X.1.2.1 	EAS Profile enhancement for EAS onboard

According to clause 7.X.1.2 EAS discovery for EAS onboard satellite the following change is needed in TS 23.558.

- TS 23.558 Clause 8.2.4 EAS Profile EAS mobility on board LEO or MEO Satellite

TS 23.558 Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X, UAV, application enabler) |
| EAS description | O | Human-readable description of the EAS |
| … | … | … |
| EAS SAT ID | O | The Satellite ID corresponding to the EAS. |
| > ephemeris information | O | The ephemeris information of the satellite corresponding to the EAS |
| … NOTE 1: The assignment of the trajectory ID for EAS(s) can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculed for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |