3GPP TSG-SA6 Meeting #61	S6-242107

# Jeju, Korea (Republic Of), 20th May 2024 - 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0642 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | API requester identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Trading |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a lack of consistency regarding inclusion of the API requester identifier in API calls. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add Requester ID as an information element in request data types that don’t currently include it. Of note, UE location subscribe request includes the requestor identifier whereas the UE location request does not. However, this does not appear to be a subscription specific inclusion since other similar requests include it, e.g., Session with QoS create request and application traffic influence trigger from EAS request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent approach API definition resulting in an extra burden on the API producer in identifying the API consumer. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.3.2, 8.6.5.3.2 & 8.8.4.15. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change 1

##### 8.6.2.3.2	UE location request

Table 8.6.2.3.2-1: UE location request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| UE ID | M | Identifies the UE for which location will be reported (e.g. GPSI) |
| Security credentials | M | Security credentials of the EAS |
| Location granularity | O | Indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the EAS |
| Location QoS | O | Indicates the location quality of service as described in clause 4.1b of 3GPP TS 23.273 [9]. |

Change 2

##### 8.6.5.3.2	UE Identifier API request

Table 8.6.5.3.2-1: UE Identifier API request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Identifier of the requestor (i.e. EASID or EECID). |
| User information (NOTE 1) (NOTE 3) | O | Information about the User or UE available in the EAS or EEC, e.g. IP address. |
| UE ID (NOTE 2) (NOTE 3) | O | UE ID in the form of GPSI as per clause 7.2.6. |
| EAS ID list (NOTE 4) | O | Identifier of the EAS(s) for which the UE IDs are requested for by EAS or EEC given the User information (e.g. IP address). |
| EAS Provider ID | O | Identifier of the ASP that provides the EAS. |
| Application Port ID (NOTE 5) | O | Application Port ID, as defined in 3GPP TS 23.502 [3], associated with the EAS. |
| Security Credentials | M | Security credentials of the EAS or EEC. |
| NOTE 1: This IE is Mandatory when EAS invoke the UE ID API. When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. When EAS invokes the API, it may recognize the UE IP address is a public IP address different from the actual UE IP address (private IP address), i.e., the UE is behind a NAT, and should therefore include the Port Number and associated IP address as part of the User information. NOTE 2: This IE is used when invoked by the EEC and if the EEC have the UE ID already in a form not desired to be shared with the EAS. NOTE 3: At least one of them shall be present. NOTE 4: This IE is Mandatory when EAS invoke the UE ID API. NOTE 5: This IE may only be present when EAS invoke the UE ID API. |  |  |

Editor's note: [SA3] It is FFS whether the IP address (i.e. User information) provided in the request can be verified that is indeed the UE’s and whether the UE provided information (IP Address) can be trusted.

Change 3

#### 8.8.4.15	EELManagedACR service request

Table 8.8.4.15-1 describes information elements for the EELManagedACR service request from the Edge Application Server to the Edge Enabler Server.

Table 8.8.4.15-1: EELManagedACR service request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| UE identifier | M | Identifies the UE for which ACR is requested |
| EAS characteristics for ACR | M | Set of characteristics to determine required Edge Application Servers, as detailed in Table 8.5.3.2-2. |
| Security credentials | M | Security credentials of the source Edge Application Server |
| Address for Application Context storage | O | The address information from where the Application Context can be accessed for ACT. |

End of changes