3GPP TSG-SA WG6 Meeting #61	S6-242119

Jeju, South Korea 20th – 24th May 2024	(revision of S6-xxxxxx)

Source:	NTT DOCOMO

Title:	Overview of service API access procedure for API invoker for TR 23.946

Spec:	3GPP TR 23.946 v0.4.0

Agenda item:	9.7

Document for:	Approval

Contact:	kouhei.nozaki.rv@nttdocomo.com

1. Introduction

This pCR proposes service API access procedure for API invoker for CAPIF_EXT (3GPP TR 23.946 v0.4.0).

2. Reason for Change

The procedures of CAPIF are extensive and could be difficult for developers to understand immediately. In particular, in order for an API invoker to invoke a service API, several steps must be taken, such as onboarding, discovering the service API, and obtaining authorization. Therefore, this pCR focuses on the API invoker, demonstrating an overview of the procedures for an API invoker to invoke service APIs. This will help developers understand the steps for service API invocation.

This pCR will be helpful for the developers in understanding these procedures.

3. Proposal

It is proposed to approve this pCR for 3GPP TR 23.946 v0.4.0.

* * * First Change * * * *

4.X	Overview of service API access procedure for API invoker

This subclause shows the overview of service API access for API invoker. The detail of the CAPIF overall operations is shown in Annex A in TS 23.222.

4.X.1	Service API access procedure for API invoker without resource owner

Figure 4.X.1-1 describes the overview of service API access procedure for API invoker without resource owner.

The API invoker accesses the service API following steps:

1.	API invoker onboards to the CAPIF core function, as specified in subclause 8.1 in TS 23.222.

2.	The API invoker authenticates with the CAPIF core function, as specified in subclause 8.10 in TS 23.222.

3.	The API invoker discovers the service API, as specified in subclause 8.7 in TS 23.222.

4.	To access the service API, the API invoker obtains authorization with the CAPIF core function, as specified in subclause 8.11 in TS 23.222.

5.	The API invoker authenticates with the AEF to access the service API prior to invoking the service API as specified in subclause 8.14 or upon the invocation as specified in subclause 8.15.

6.	The API invoker requests service API invocation, as specified in subclause 8.16 in TS 23.222.

Figure 4.X.1-1: Overview of service API access procedure for API invoker without resource owner

4.X.2	Service API access procedure for API invoker with resource owner

Figure 4.X.1-2 describes the overview of service API access procedure for API invoker with resource owner.

The API invoker accesses the service API following steps:

1.	API invoker onboards to the CAPIF core function, as specified in subclause 8.1 in TS 23.222.

2.	The API invoker authenticates with the CAPIF core function, as specified in subclause 8.10 in TS 23.222.

3.	The API invoker discovers the service API, as specified in subclause 8.7 in TS 23.222.

4.	The API invoker authenticates with the AEF to access the service API prior to invoking the service API as specified in subclause 8.14 or upon the invocation as specified in subclause 8.15.

5.	To access the service API, the API invoker obtains authorization with the CAPIF core function, and then requests service API invocation as specified in subclause 8.31 in TS 23.222.

Figure 4.X.1-2: Overview of service API access procedure for API invoker with resource owner

* * * End of Changes * * * *