3GPP TSG-SA WG6 Meeting #51	S6-242301

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0658 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Address EN for triggering service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Address EN on configuration of triggering service Editor's note: It is FFS whether and how the ECS and EES determine the parameters (e.g., port ID) necessary for EEC triggering service or if configuration is sufficient Several usecases need to be provided for, including: - UEs with multiple EECs, each EEC may use a different port ID - EECs communicating with multiple ECSs or EESs (e.g. for different ECSPs) communications with each ECSP may use a different port ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 8.16.1 deleted the EN and added clarifications Editor's note: It is FFS whether and how the ECS and EES determine the parameters (e.g., port ID) necessary for EEC triggering service or if configuration is sufficient In 8.3.3.3.4 and 8.5.3.4 added clarifications that the EEC triggering request IE includes the port number for receiving the trigger. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Usecases described above are not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 8.3.3.3.4; 8.5.3.4; 8.16.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedure for the 5G System (5GS); Stage 2".

[4]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[5]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[8]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[9]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[10]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[11]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[12]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.434: "Service enabler architecture layer for verticals; Functional architecture and information flows; Stage 2".

[14]	3GPP TS 23.286: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows ".

[15]	ETSI ISG MEC ETSI GS MEC 003 V2.1.1 (2019-01), "Multi-access Edge Computing (MEC); Framework and Reference Architecture"

[16]	Void

[17]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[19]	GSMA PRD OPG.02 Operator Platform Telco Edge Requirements  version 4.0, available at https://www.gsma.com/futurenetworks/resources/operator-platform-telco-edge-requirements/

[20]	3GPP TS 23.548: "5G System Enhancements for Edge Computing".

[21]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[22]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[23]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications".

[24]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[25]	3GPP TS 32.257: "Telecommunication management;Charging management;Edge computing domain charging".

[26]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[27]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations".

[28]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[x]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)”.

* * * Next Change * * * *

##### 8.3.3.3.4	Service provisioning subscription request

Table 8.3.3.3.4-1 describes the information elements for service provisioning subscription request from the EEC to the ECS.

Table 8.3.3.3.4-1: Service provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address (NOTE 1, NOTE 2) | O | The Notification Target Address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| AC profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| ECSP identifiers | O | The list of EEC preferred ECSPs that provide the EES. |
| EEC Triggering request (NOTE 2) (NOTE 3) | O | Indicates that EEC Triggering is requested and the trigger destination, i.e. UE port number for receiving triggers for the specific EEC. |
| NOTE 1: When SEAL NMS is used this IE is same as Callback URL in SEAL notification management service. NOTE 2: One of them may be provided. NOTE 3: The UE port number for receiving EDGE-1 and / or EDGE-4 triggers is pre-configured with each EEC in accordance with 3GPP TS 23.040 [x]. How the triggering port number is pre-configured is out of scope of the present specification. |  |  |

* * * Next Change * * * *

#### 8.5.3.4	EAS discovery subscription request

Table 8.5.3.4-1 describes the information elements for EAS discovery subscription request from the EEC to the EES.

Table 8.5.3.4-1: EAS discovery subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Event ID | M | Event ID: - EAS availability change - EAS dynamic information change |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Notification Target Address (NOTE 1) | O | The Notification target address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| EAS discovery filters | O | Set of characteristics to determine matching EASs (as detailed in Table 8.5.3.2-2). Applicable for "EAS availability change" event |
| EAS dynamic information filters | O | List of dynamic information changes (as detailed in Table 8.5.3.4-2) about EAS, the EEC is interested in. Applicable for "EAS dynamic information change" event |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not. The IE also indicates which ACR scenarios are supported by the EEC. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| EAS Instantiation Triggering Indication | O | Indicates to the EES that EAS instantiation triggering may be performed for the current request if needed. |
| EAS Instantiation Triggering Indication (NOTE 2) | O | Indicates to the EES that EAS instantiation triggering may be performed for the current request if needed. |
| EAS Instantiation Triggering Suppress (NOTE 2) | O | Indicates to the EES that EAS instantiation triggering should not be performed for the current request, and Instantiable EAS Information (e.g. instantiated, instantiable but not be instantiated yet) is to be provided in the notification. |
| EEC Triggering request (NOTE 1) (NOTE 3) | O | Indicates that EEC Triggering is requested and the port number for receiving the trigger. |
| NOTE 1: One of them may be included in the request message. NOTE 2: One of them may be included in the request message. NOTE 3: The triggering port number is pre-configured with the EEC in accordance with 3GPP TS 23.040 [x]. How the triggering port number is pre-configured is out of scope of the present specification. |  |  |

* * * Next Change * * * *

## 8.16	EEC triggering service to initiate procedures over EDGE-1 or EDGE-4

### 8.16.1	General

The functional entities of the Edge Enabler Layer can utilize the Application Triggering services (or Device Triggering) provided by the 3GPP core network via NEF or SCEF (see clause 4.4.5 in 3GPP TS 23.501 [2], clause 4.13.2, 5.2.6.5.2 in TS 23.502 [3], and clause 4.5.1 TS 23.682 [17]) to enable for ECS or EES to perform EEC Triggering, i.e., to trigger EEC to perform the procedures over EDGE-1 or EDGE-4 (e.g., service provisioning or EAS discovery).

If application triggering is supported and required by the EEC, the EEC may indicate to the ECS or EES that the EEC triggering service is needed.

The triggering mechanism is used by the ECS and EES for messaging to EEC by invoking application triggering service as specified for Nnef_Trigger_Delivery service in clause 4.13.2 of TS 23.502 [3] or the device triggering procedure via T8 in clause 5.17 of TS 23.682 [17]. The ECS can use it to trigger the EEC to initiate service provisioning request (as per clause 8.3.3.2.2) when the EDN configuration information in the ECS is updated. The EES can use it to trigger the EEC to initiate EAS discovery request (as per clause 8.5.2.2) when the EAS profile in the EES is updated.

.

After receiving the EEC triggering request in the Service Provisioning subscription or EAS discovery subscription request as described in clause 8.3.3 and 8.5.2, when ECS or EES performs EEC triggering service by invoking the application triggering service, the ECS or EES includes, in the Trigger Payload, Triggering Entity Information (identifier and endpoint address of the triggering EES or ECS) and optionally Trigger Description, which indicates the procedure (e.g., service provisioning or EAS discovery) to be initiated by the EEC. The Nnef_Trigger_Delivery procedure uses the receiving port information provided by EEC in the EEC Triggering request, or pre-agreed and pre-provisioned information, according to local policies. Upon receiving the Trigger Payload, the EEC selects which procedure needs to be performed according to the received Trigger Description and initiates the selected procedure towards either EES or ECS indicated by the Triggering Entity Information in the Trigger Payload.

NOTE:	If the Trigger Description is not given, application port information can be used by the EEC to select the procedure to be triggered.

* * * End of Changes * * * *