3GPP TSG-SA WG6 Meeting #61	S6-242314

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0185 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to clause 6.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S6 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | SNAAPP |  |  |  |  |  | Date: |  |  | 2024-05-12 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following statement in clause 6.2.3 is incorrect: “The API exposing function (e.g. NEF, SCEF) acts as a resource owner consent enforcement point as specified in 3GPP TS 33.501 [8] and interacts with the authorization function in the CAPIF core function via CAPIF-3.” In CAPIF, CCF acts as the resource owner consent enforcement point. In non-CAPIF mode, the AEF acts as resource owner consent enforcement point. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the statement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CAPIF specification cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.2.3	Functional model description to support RNAA

Figure 6.2.3-1 shows the architectural model for the RNAA which allows the resource owner to provide authorization to the API invocation.

Figure 6.2.3-1: High level functional architecture for CAPIF supporting RNAA

The authorization function is an internal entity of the CAPIF core function.

The resource owner function interacts with the authorization function in the CAPIF core function via CAPIF-8. The resource owner function communicates with the authorization function in the CAPIF core function to manage resource owner consent.

The CCF (Authorization Function) (e.g. NEF, SCEF) acts as a resource owner consent enforcement point. The API exposing function can retrieve the resource owner consent parameters from the authorization function.

NOTE 1:	RNAA is supported for both 4G and 5G network.

The API invoker interacts with authorization function in the CAPIF core function via CAPIF-1/CAPIF-1e.

NOTE 2:	In the current release, 3rd party API providers (i.e., API providers outside the PLMN trust domain) are not supported for RNAA.

NOTE 3:	The interaction between resource owner function and CCF over CAPIF-8 is not specified in the current release of the specification.

The security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [12].