3GPP TSG-SA WG6 Meeting #61	S6-242362

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242334)

Source:	Ericsson

Title:	CAPIF onboarding improvement

Spec:	3GPP TR 23.700-22 v0.2.0

Agenda item:	8.7

Document for:	Approval

Contact:	Wenliang Xu (wenliang.xu@ericsson.com)

1. Introduction

This pCR provides update for existing solution for API invoker onboarding.

2. Reason for Change

The CAPIF onboarding procedure was enhancement with conditional onboarding. When a onboarding criteria is not met, the CCF trigges offboarding notification to the previously onboarded API invoker, expecting the API invoker to further refresh or even cancel its onboarding.

Sometimes, the problem about condition not met is just temporary and can be solved soon (e.g. an AEF or API is not available temporarily). In this case, it is not necessary for the API invoker to compromise its onboarding criteria or even request to cancel its onboarding.

This paper clarified the missing suspension status for API invoker and introduces a more flexible handling with offboarding graceful period. In addition, the CCF can utilize ADAE analytics service for API analytics (cl.8.6 of TS 23.436) in order to make decision for triggering notifications to the API invoker.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-22.

* * * First Change * * * *

## 6.6	Solution #5: Enhancing API Invoker onboarding

### 6.6.1	Solution description

This solution is related to Key issue #5: Enhancing support to API Invoker onboarding, proposing to enhance the API invoker onboarding procedure specified in 3GPP TS 23.222. The enhanced procedure enables the CCF to onboard an API Invoker to CAPIF, based on meeting the criteria information from the API invoker. The API invoker may include the criteria information in the API Invoker onboarding request message, to inform that it wishes to onboard to CCF only if the criteria information in the request is met by CCF.

Figure 6.6.1-1 illustrates the procedure for enhanced onboarding of the API Invoker to the CAPIF.

Figure 6.6.1-1: Enhanced API Invoker onboarding procedure

1.	The API Invoker sends the onboard API invoker request message to the CCF, to onboard itself to the CAPIF, as specified in 3GPP TS 23.222, along with additional onboarding criteria information. The onboarding criteria information may include the information about AEFs serving certain set of service API(s), availability of AEFs serving a set of service APIs, support of security methods for certain AEFs / AEF types/ service APIs/ Service API categories, interconnection with a given set of CCFs etc.

Table 6.6.3-1: On-board API Invoker request

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding information | M | The information of the API invoker including enrolment details, required for on boarding |
| APIs for enrolment | O | List of APIs being enrolled for. |
| Proposed expiration time | O | Proposed expiration time for the onboarding. |
| Onboarding criteria | O | API Invoker indicates to the CCF that the API Invoker wishes to on board itself to the CCF only if the CCF supports the features in Criteria information (For example, AEFs serving certain set of service API(s), availability of AEFs serving a set of service APIs, support of certain security methods, support of certain security methods for certain AEFs / AEF types/ service APIs/ Service API categories, interconnection with a given set of CCFs and like so). |

2.	On receipt of onboard API invoker request message, the CCF begins the onboarding process by verifying whether all the necessary information has been provided to onboard the API invoker. If the onboard API invoker request message includes criteria information, then CCF verifies whether the CCF can support the criteria information. The CCF further proceeds with API invoker onboarding procedure, only if the CCF supports the criteria information from the API Invoker.

3.	If the API invoker is granted permission to be onboarded, the CCF responds with the onboard API invoker with success message, including information about the provisioned API invoker profile/API Invoker enrolment details, which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs.

Figure 6.6.1-2 illustrates the new procedure for notifying the API invoker from CCF, once the onboarding criteria information is no longer supported by the CCF.

Pre-condition:  CCF decides to notify the API invoker  due to certain conditions like, onboarding criteria information not met.

Figure 6.6.1-2: notification procedure

1.	 CCF sends onboarding notification request to API Invoker, including the notification reason (onboarding condition not met).

2.	The API invoker acknowledges the onboarding notification request message.

3.	The API invoker, based on the notification reason from CCF in step 1, may renew the API invoker’s enrolment to CCF using the procedures in clause 8.1 of 3GPP TS 23.222 with new criteria information or cancel the API invoker’s enrolment to CCF using the procedures in clause 8.2 of 3GPP TS 23.222.

* * * End of Changes * * * *