3GPP TSG-SA WG6 Meeting #61	S6-242597

Jeju Island, South Korea, 20th – 24th May 2024	(revision of S6-242083)

Source:	Huawei, HiSilicon

Title:	pCR on solution of providing application layer caller information to callee

Spec:	3GPP TR 23.700-92 v0.5.0

Agenda item:	8.2

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes solution of providing application layer caller information to callee which solves the Key Issue#4.

2. Reason for Change

As part of basic communication, the originating UE may insert a P-Preferred-Identity header field in any initial SIP request for a dialog or in any SIP request for a standalone transaction as a hint for creation of a public user identity as described in 3GPP TS 24.229 [x], 3GPP TS 24.607 [x] specified the OIP service using IP Multimedia (IM) Core Network (CN) subsystem. But the information carried in the P-Preferred-Identity only includes the public user identity related information (i.e. text) in the sip header.

As the gap analysis addressed in the key isse#4, provide application layer caller information (data channel media) to callee can not be supported by Rel-18 work and is not covered by Rel-19 FS_NG_RTC_Ph2 WID.

This solution proposes to present enhanced identity information (e.g. data channel media) to the terminating user in order to identify the originating user.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.5.0.

* * * First Change * * * *

## 8.X	Solution #X: Providing application layer caller information to callee

### 8.X.1	Description

This clause provides solution against Key Issue #4 Support of providing application layer caller information to callee.

This solution uses the architecture of eMMTel Enabler as specified clause 8.1 as the basis and has no requirement to enhance the architecture.

The Pre-call Multimedia Information Presentation (PMIP) as specified in GSMA NG 129 [11] are supported in this solution:

-	The Pre-call Multimedia Information Presentation (PMIP) feature allows users to send the additional call related content during the ringing process. This pre-call information might be static or provided on per-call basis. It can communicate the call intent, provide the enhanced caller identification with the caller’s job title, picture, and location. All the information is transmitted over an IMS data channel;

In this procedure, the eMMTel Enabler Server acts as a DC AS based on SA2’s IMS exposure capability architecture as specified in TR 23.700-77 [19].

### 8.X.2	Procedures

#### 8.X.2.1	Establish a Call including caller information

Figure 8.X.2.1-1 below illustrates how to send application layer caller information to the callee via ADC data channel.

Pre-conditions:

1).	Both the Application layer Caller and eMMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.x.2.1-1: Establish a Call including caller information

1.	Establish the application call between the application layer caller and callee as specified in solution#2, the application data channel between eMMTel Enabler Server and Callee is established via IMS Core (MF/MRF), it’s assumed that the Callee does not answer the call before the application data channel negotiation is complete.

2.	The application layer caller sends an application layer caller information request to the eMMTel Enabler Server, the application layer caller can also upload the application layer information to the eMMTel Enabler Server in advance and only include the indicator of the information in this step. The request message includes information elements as specified in Table 8.x.2.1-1.

Table 8.x.2.1-1: Information elements in application layer caller information request

| Information element | Status | Description |
| --- | --- | --- |
| Caller Identity (see NOTE) | M | The identity of the application layer caller. |
| Security credentials | O | Security information required by the eMMTel Enabler Server. |
| application layer caller information | O | A list of application layer caller information configured in this request. The detailed information may be specific DC media information or a indicator of the DC media information. |
| NOTE: The Caller Identity may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain, If Caller Identity is not MSISDN based identifier, the eMMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Caller Identity and the routable ID. |  |  |

3.	The eMMTel Enabler Server verifies the content of Call related information in the request.

4.	The eMMTel Enabler Server sends an application layer caller information response to the application layer caller.

Table 8.x.2.1-2: Information elements in application layer caller information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the verification of this application layer caller information. |
| Reason | O | Indicates the reason for failure. |

5.	The eMMTel Enabler Server sends the application layer caller information to the Callee via IMS Core (MF/MRF). The IMS Core (MF/MRF) parses the HTTP packet and sends the information to Callee via application data channel.

NOTE:	The information may be somewhat delayed compared to called party alert, if the Called Party answers the call immediately after the called party alert, the pre-call information can not be presented to the Called Party.

6.	The Callee answers tha call.

### 8.X.3	Impacts on Existing Nodes and Functionality

In this solution, the procedures use the basic capability exposure by IMS core which is under studied by SA2.

* * * End of Changes * * * *