3GPP TSG-SA WG6 Meeting #66	S6-251055

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-25xxxx)

Source:	Nokia, Kontron Transportation France

Title:	Clarification on support of multi-talker control in adhoc group calls

Agenda Item:	10.3

Contact:	Martin Oettl, martin.oettl@nokia.com

Abstract: The discussion paper describes how multi-talker control is enabled for adhoc group calls in current specification and discusses an alternative solution. Finally, it provides a recommendation on how to proceed.

## Introduction

The new WID on Stage 2 for FRMCS Phase 6 (SP-250390) also addresses enhancements for ad hoc group communications and specifically highlights multi-talker control:

- Specify enhancements for ad hoc group communications (e.g., call forwarding, combining of group calls and emergency alerts, informing group call participants about important events, multi-talker control).

The reason for adding multi-talker control to the list of objectives is that there were doubts that the support of multi-talker control in ad hoc group calls lacks on proper configuration, i.e. it is unclear whether a specific ad hoc group support multi-talker control.

This discussion paper describes the current solution, an alternative approach and provides a proposal on how to proceed.

## Discussion

Current specification TS 23.379 Annex A.5 configures the support for multi-talker control within the MCPTT service configuration data (see below), i.e. either all ad hoc groups within an MC system support the multi-talker feature or no ad hoc group does. Current specification does not allow a per ad hoc group specific usage either pre-configured or controlled by the MCPTT server per implementation specific means.

Table A.5-2: MCPTT service configuration data (on-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Configuration management server |
| --- | --- | --- | --- | --- |
|  | Ad hoc group call configurations |  |  |  |
| Subclause 6.2.3.7.13 of 3GPP TS 22.179 [2] | > Multi-talker floor control supported | Y | Y | Y |
| Subclause 7.4.2.3.5 and subclause 7.4.2.3.6 | > Audio mixing is performed in the UE or in the network to support multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | > Maximum number of simultaneous talkers applicable for multi-talker control | N | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [19]. NOTE 2: The usage of this parameter by the MCPTT server is up to implementation. NOTE 3: If the support for ad hoc group call is disabled by the MC system then all other configurations related to ad hoc group call are not applicable. |  |  |  |  |

An ad hoc group specific multi-talker support control per pre-configuration raises the question how to configure the usage as ad hoc groups are created when used and do not exist before. One option is to use the MCPTT group configuration from the MCPTT group which is associated with the ad hoc group (see below).

Table 10.19.2.1-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID (see NOTE 1) | O | The MCPTT group ID to be associated with the ad hoc group communication |

The problem with that approach is that all users would be required to be pre-configured to be users of that associated MCPTT group, which is a significant configuration overhead. Also Stage 3 behaviour would need to be modified as the associated MCPTT group is only used to proper keys when end-to-end media security is requested.

Another alternative is that the MCPTT server controls the use of the multi-talker feature for a specific MCPTT ad hoc group on a “left to implementation” bases, i.e. the MCPTT server decides during the ad hoc group call setup that multi-talker control is used and informs the group participants accordingly. This requires a few changes in current procedures including modification of the following two information flows:

10.19.2.2 Ad hoc group call request return (MCPTT server – MCPTT client) -> to inform the requester

10.19.2.4 Ad hoc group call request (MCPTT server – MCPTT client) -> to informs all other group participants

#### 10.19.2.2	Ad hoc group call request return (MCPTT server – MCPTT client)

Table 10.19.2.2-1 describes the information flow ad hoc group call request return from the MCPTT server to the MCPTT client. This response could be intermediate response to provide the server assigned MCPTT ad hoc group ID.

Table 10.19.2.2-1: Ad hoc group call request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ad hoc group ID (see NOTE) | O | The MCPTT group ID to be associated with the ad hoc group call which is either provided by the group call initiator or assigned by the MCPTT server. This information element shall be present if the authorization result is success. |
| Pre-configured group ID (see NOTE) | O | The preconfigured MCPTT group ID whose group configuration is to be used for the ad hoc group call (e.g. information for end-to-end security). |
| Multi-talker support | M | Indicates whether multi-talker control is applied (yes/no) |

#### 10.19.2.4	Ad hoc group call request (MCPTT server – MCPTT client)

Table 10.19.2.4-1 describes the information flow ad hoc group call request from the MCPTT server to the MCPTT client.

Table 10.19.2.4-1: Ad hoc group call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The identity of the MCPTT user towards which the request is sent |
| Functional alias | O | The functional alias of the calling party |
| MCPTT ad hoc group ID | M | The MCPTT group ID to be associated with the ad hoc group call |
| Multi-talker support | M | Indicates whether multi-talker control is applied (yes/no) |

The solution that the MCPTT server controls the use of the multi-talker feature for a specific MCPTT ad hoc group is feasible, the specification effort is limited, and the solution does not break any stage 3 principles.

## Proposal

We believe that the current solution where the support of multi-talker control is part of the MCPTT service configuration data is sufficient and no per ad hoc group solution is needed. If a group specific solution is needed than the MCPTT server should control the use of multi-talker and the configuration is „left to implementation“.