3GPP TSG-SA WG6 Meeting #67	S6-252057

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0038 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor group creation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - S6-251464 has general description about Spatial anchor group management - S6-252056 Discussion on support for Spatial anchor group |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the procedure for creation of spatial anchor group. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of managing spatial anchors as group will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.x.2, 8.3.x.2.1, 8.3.x.3, 8.3.x.3.1, 8.3.x.3.2, 8.3.x.3.3, 8.3.x.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.x	Spatial anchor group management

8.3.x.2	Procedures

8.3.x.2.1	Spatial anchor group creation

Figure 8.3.x.2.1-1 illustrates the spatial anchor group creation procedure through which the requestor SAn client or the VAL server can request the SAn server through SEAL GM server to create a spatial anchor group associating or linking a list of spatial anchors with the group. SEAL GM server server creates and stores the spatial anchor group and assigns a unique identifier (Spatial anchor group ID) to enable the requestor to perform spatial anchor group related operations

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.3.x.2.1-1: Spatial anchor group creation

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor group create request to the SAn server. The request includes a requestor identifier, security credentials, list of spatial anchors identified by the spatial anchor IDs to be associated or linked as part of the spatial anchor group, human readable description about the spatial anchor group and service area where the spatial anchor group is accessible as described in clause 8.3.x.3.1.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the creation of spatial anchor group. If the requestor is authorized, the SAn server sends a group creation request as described in clause 8.3.x.3.3 to the SEAL GM server.

3.	The SEAL GM server creates the spatial anchor group and assigns a unique identifier to it and stores the spatial anchor group related information provided in the step 1. The spatial anchor group related information contains spatial anchor group ID, list of spatial anchor IDs associated with the group, service area etc. The SEAL GM server provides a group creation response as described in clause 8.3.x.3.4 to the SAn server.

4.	The SAn server sends a spatial anchor group create response as described in clause 8.3.x.3.2 to the requestor. If the spatial anchor group is created, the response includes an indication of success and spatial anchor group identifier of newly created spatial anchor group and list of spatial anchor IDs associated with the created group. Otherwise, the response includes an indication of failure and can include a reason for failure.

# * * * * Next change * * * *

8.3.x.3	Information flows

8.3.x.3.1	Spatial anchor group create request

Table 8.3.x.3.1-1 describes information elements for the spatial anchor group create request from the SAn client or VAL server to the SAn server.

Table 8.3.x.3.1-1: Spatial anchor group create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Group description | O | Human readable description about the spatial anchor group (e.g. List of restaurants) |
| Service area | O | Service area where the spatial anchor group is visible or accessible. |
| VAL service information | M | Information about the application service associated with the spatial anchor group. |
| Access control rules | O | Access control rules defining which entities are permitted to discover the spatial anchor group. |
| Service ID | M | The identifier of the application service. |
| List of spatial anchors IDs | M | List of spatial anchors IDs to be associated with the group. |

8.3.x.3.2	Spatial anchor group create response

Table 8.3.x.3.2-1 describes information elements for the spatial anchor group create response sent by the SAn server.

Table 8.3.x.3.2-1: Spatial anchor group create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the spatial anchor group create request was successful. |
| >Spatial anchor group ID | M | The unique identifier of the created spatial anchor group. |
| >List of Spatial anchor IDs | M | List of spatial anchor IDs associated with the created spatial anchor group. |
| Failure response (NOTE) | O | Indicates that the spatial anchor group create request has failed. |
| >Cause | O | The cause for the request failure. |
| NOTE: Only one of these IE shall be present. |  |  |

8.3.x.3.3	Group creation request

Table 8.3.x.3.3-1 describes the information flow group creation request from the SAn server to the SEAL GM server.

Table 8.3.x.3.3-1: Group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the SAn server performing the request. |
| Identity list | M | List of spatial anchors IDs that are part of the group to be created. |
| VAL service ID | O | The identifier of the application service. |
| VAL service specific information | O | Placeholder for VAL service specific information, which include group description, service area, VAL service information |

8.3.x.3.4	Group creation response

Table 8.3.x.3.4-1 describes the information flow group creation response from the SEAL GM server to the SAn server.

Table 8.3.x.3.4-1: Group creation response

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor group ID | M (see NOTE) | The unique identifier of the created spatial anchor group. |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of Spatial anchor group ID information element has no meaning. |  |  |

# * * * * End of changes * * * *