3GPP TSG-SA WG6 Meeting #67	S6-252298

Fukuoka, Japan, 19 - 23 May 2025	(revision of S6-252114)

Source:	China Mobile

Title:	Sol for KI#4 data transmission quality guarantee in complex tethered device scenarios based on PIN

Spec:	3GPP TR 23.700-51

Agenda item:	9.5

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This contribution propose to address the following open issues in KI#4,

-	How to enhance to support for SEALDD connection management among tethering device and multiple tethered devices, e,g, based on PINAPP solution, device identifier mechanism.

-	How could the application enabling layer support for QoS differentiation in complex tethered device scenarios, such as when multiple tethered devices are connected to the same 3GPP device.

2. Reason for Change

To support complex tethered device scenarios, for example, multiple tethered devices are connected to the 3GPP UE to use XR service together. Other several tethered devices are connected to the same 3GPP UE to use another application service. Under the circumstances, SEALDD layer should be able to know the mapping relationship of XR tethered devices and 3GPP UE, to provide accurate transmission service and QoS guarantee service. It is proposed that the 3GPP UE could act as PEGC and PEMC, the tethered XR device can act as a PINE, and the PIN server will store tethering mapping relationship as a tethering group information in PIN profile. Based on that, SEALDD enabled XR data transmission establishment should be enhanced to support for each tethered devices or tethering device applicaiton traffic transmission use multiple SEALDD-UU flow respectively.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[7]	3GPP TR 26.926: "Traffic Models and Quality Evaluation Methods for Media and XR Services in 5G Systems".

[x]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[y]	3GPP TS 23.433: "Data Delivery enabler for vertical applications".

* * * Next Change * * * *

5	Solutions

5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #x |  |  |  | X |
| Sol #2 |  |  |  |  |

5.x	Solution #x: data transmission quality guarantee in complex tethered device scenarios based on PINAPP

5.x.1	Architecture Impacts

This solution is based on architecture of PINAPP as described in 3GPP TS 23.542 [x], and architecture of SEALDD as described in 3GPP TS 23.433 [y].

The solution improves the PINAPP layer defined in 3GPP TS 23.542, i.e.:

- Multiple tethered device as PINE can be registered in one 3GPP UE as PEGC and PEMC

- multiple flows can be requested in one SEALDD enabled regular data transmission connection establishment.

2.	The SEALDD server uses 5GC capabilities to configure XR traffic flows.

5.x.2	Solution description

This solution addresses the improvement of the SEALDD layer to support complex tethered device scenarios. For example, multiple tethered devices are connected to the 3GPP UE to use XR service together. Other several tethered devices are connected to the same 3GPP UE to use another application service. Under the circumstances, SEALDD layer should be able to know the mapping relationship of XR tethered devices and 3GPP UE, to provide accurate transmission service and QoS guarantee service. It is proposed that the 3GPP UE could act as PEGC and PEMC, the tethered XR device can act as a PINE, and the PIN server will store tethering mapping relationship as a tethering group information in PIN profile. Based on that, SEALDD enabled XR data transmission establishment should be enhanced to support for each tethered devices or tethering device applicaiton traffic transmission use multiple SEALDD-UU flow respectively.

5.x.2.1	Impact to existing PIN procedures

The PIN procedures and information flows in 3GPP TS 23.542 [x] can be enhanced (highlighted in bold italics) as follows.

8.4.2.2.1 PINE registration directly to PIN server Figure 8.4.2.2.1-1 illustrates PIN registration procedure based on request/response model. Pre-conditions: 1. The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server; 2. The UE Identifier is available; 3. The PINE has been authorized to communicate with the PIN server; Figure 8.4.2.2.1-1: PINE registration directly to PIN server 1. The PINE (including PINE/PEMC/PEGC) sends a PINE Registration Request to the PIN server. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PINE Address, optionally PINE type(tethered device/tethering device), VAL service ID, tethering device ID(if PINE is a tethered device). If the PEMC trigger the request, this registration request carries the PIN ID of the PIN for which it is intending to register as PEMC and the PIN element may indicate whether it is to be assigned with primary or secondary PEMC role. - If PINE type is tethering device, the PIN server checks whether the UE identified by the GPSI has subscribed to be a PEMC. Also, the PIN server can check the whether the UE identified by the GPSI has subscribed to be a PEMC itself. If subscribed, it checks the subscription of PINE whether the requesting PIN element can be assigned with primary or secondary role and authorize the PIN element accordingly. In case if the PIN has already been created, the requesting PIN element is assigned with the secondary PEMC role irrespective of the role requested by the PIN element. PIN server allocated a tethering group ID to this group. If PINE type is tethered device, the PIN server identify tethering group based on tethering device ID and VAL service ID, then store tethering mapping information in PIN profile. 3. The PIN server responds to the PINE with PINE Registration Response with allocated PIN client ID and tethering group ID(optionally) in successful response. If the registration procedure fails, the PIN server should give the failure response that indicates the cause of registration request failure. 8.4.2.2.2 PINE registration indirectly to PIN server The following procedure defines the PEMC can substitute the PINE/PEGC to register into PIN server. Pre-conditions: 1. The PEMC has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server; 2. The UE Identifier or PIN client Identifier is available; 3. The PEMC has been authorized to communicate with the PIN server; 4. The PINE/PEGC has already received the IP address of PEMC. Figure 8.4.2.2.2-1: PINE registration indirectly to PIN server 1. The PINE/PEGC sends PINE Registration Request to the PEMC. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PINE Address, tethering type(tethered device). 2. If the PEMC receives the PINE Registration Request from PINE, the PEMC may represent the PINE to register into PIN server by sending the PINE registration request to PIN server. If the PEMC represents the PINEs to perform registration procedure, it should send a PINE Registration Request to PIN server including an Indication of representation registration and Lists of PINEs/PEGCs. If the tethering type of PINE/PEGC is tethered device, PEMC checks whether and which tethering group the PINE is belong to. PEMC should send a PINE Registration Request to PIN server including an Indication of representation registration, Lists of PINEs/PEGCs and tethering group ID(if need). 3. The PIN server responds to the PEMC with PINE Registration Response with allocated PIN client ID in successful response. When the PINE Registration Request that containing Indication of representation registration, the PIN server shall verify whether PINE(s) and PEGC(s) included in the List of PINEs/PEGCs are allowed to fullfill the requested role or not. The PIN server responses to PEMC with a successful response and may include a list of accepted registration information and a list of rejected registration information (if any). If all the PINEs in the Lists of PINEs/PEGCs are rejected, the PIN server sends a failure response including the failure cause. If PINE/PEGC is tethered device, PIN server add PINE/PEGC into tethering group information. 4. The PEMC sends the response received in step 3 to the PINE/PEGC. When the PEMC receiving the PINE Registration response from PIN server, the PEMC should extract the accepted registration information and rejected registration information (if any) for all the PINEs, and send the result to each PINE in the Lists of PINEs/PEGCs.

8.2.2.1 PIN Profile in a PIN PIN profile includes information about the static data needed for configuration for the Personal IoT networks. Table 8.2.2.1-1: PIN Profile Parameter Name; Parameter Description; PIN Server; PEMC; PEGC. PIN ID; The identifier of the PIN; Y; Y; Y. PIN Description; Human-readable description of the PIN, for example, the company name, location or the type of service.; Y; Y; Y. Duration; Specifies the time period of how long the PIN can be active; Y; Y; Y. Maximum number of PIN elements; Maximum number of PINEs allowed to join the PIN; Y; Y; N. Allowed PIN service; List of service that can be offered within a PIN: > PIN service Provider Identifier > PIN service type > PIN service Feature; Y; Y; N. Allowed PEMC list; The list of PINE static information for PINE(s) that can be allowed to take the role of PEMC: > PINE ID > GPSI > Role (e.g., primary, secondary or both) > Port number (see NOTE1); Y; Y; Y. Allowed PEGC list; The list of PINE static information for PINE(s) that can be allowed to take the role as PEGC: > PINE identifier > GPSI > Role (e.g., primary, backup or both) > Port number (see NOTE1); Y; Y; Y. PIN Server ID; The identifier of the PIN server that serves the PIN; N; Y; Y. PIN server Endpoint; The endpoint information (e.g., URI, FQDN) or a static IP address used to communicate with the PIN server.; N; Y; Y. Allowed PIN elements list; List of PINEs which can be allowed to join the PIN > PINE ID; Y; Y; Y. Allowed tethering group list; List of tethering mapping which can be allowed to join the tethering group(s) (see NOTE2); Y; Y; Y. NOTE1: NAT aspects are not specified in the current release. NOTE2: Allowed tethering group list is defined in table 8.2.2.1-2.. Table 8.2.2.1-2: Allowed tethering group list Information element; Status; Description. tethering group ID; M; Identify the tethering group.. VAL service ID; M; Identity of the VAL service of this tethering group. tethering device identifier; O; Identifier of a VAL UE acting as a tethering device in this group, e.g. VAL UE ID or UE IP address. tethered device identifier list; O; Identifier list of tethered devices, e.g.MAC address.

5.x.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.542 [x] can be enhanced (highlighted in bold italics) as follows.

9.12.2.1 SEALDD enabled multi-modal data transmission service for multi-modal application 9.12.2.1.1 General The following clauses specify procedures, information flows and APIs about SEALDD enabled data transmission for multi-modal application, including the SEALDD facilate PDU set handling. 9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing multi-modal data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. - The VAL server has got tethering group ID from VAL client. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, tethering group ID, list of requested flows, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for multi-modal application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1. 2. Upon receiving the request, if tethering group ID is included, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server sends tethering group information request to PIN server, including tethering group ID, if tethering group ID is included in request. 3. PIN server sends response to SEALDD server, including tethering device identifier(e.g. VAL UE ID or UE IP address), tethered device identifier list(e.g.MAC address). 4. SEALDD server allocates SEALDD-S connection information of the SEALDD server to receive the application data traffic from the VAL server and PIN server for application data transfer of tethered devices and tethering devices as SEALDD-S connection information of the SEALDD server side. The SEALDD server allocates a specific SEALDD-S connection information used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S connection information of the SEALDD server side). The VAL server and SEALDD server can use SEALDD-S connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the SEALDD-S connection information allocated by SEALDD server, and the QoS information for application traffic. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID. 5. Same as step 3 of clause 9.2.2.2. 6. The VAL client sends a SEALDD service request to SEALDD client, optionally including tethering group ID, tethered device information derive from PIN server. The VAL client receives a SEALDD service response to the SEALDD client. If the data transmission session information contains the SEALDD address information, then the VAL client sends the selected SEALDD address information to SEALDD client in service request message. The response indicates that whether the SEALDD service request is successful or not. 7. Same as step 5 of clause 9.2.2.2. 8. SEALDD client maps each MAC address of tethered device into UE IP address+port number, and store the mapping information. SEALDD client allocates SEALDD-UU flow ID(s) for tethering device (VAL UE) and tethered device(s) respectively. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, MAC address, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates. 9-10. Same as step 7-8 of clause 9.2.2.2. NOTE : If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

9.9.2.1 SEALDD enabled data transmission quality guarantee by switching SEALDD server Figure 9.9.2.1-1 illustrate the procedure for data transmission quality guarantee based on transmission quality report from SEALDD server and QoS monitoring from 5GS. The procedure is applicable to the scenario where there is a single path between UE and SEALDD server. Pre-conditions: 1. The VAL server discovers and selects the SEALDD server by CAPIF functions. 2. The VAL server has requested the transmission quality measurement to the SEALDD server by invoking the Sdd_TransmissionQualityMeasurement_subscription API in clause 9.7.4.2. 3. A SEALDD service policy (i.e., the necessary SEALDD layer actions for meeting the service policy requirements) has been configured in SEALDD server and shared with the SEALDD client. Figure 9.9.2.1-1: SEALDD enabled data transmission quality guarantee procedure by switching SEALDD server 1. The VAL server sends a Sdd_RegularTransmission request to the SEALDD server, as specified in clause 9.2.2.2. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), and optionally, the QoS information for the application traffic, e.g. QoS requirements, tethering group ID. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server. The QoS information may be allocated by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. 3. The regular data transmission connection is established according to clause 9.2.2.2. 4. The S-SEALDD server (i.e., SEALDD server#1) can generate the transmission quality measurement report according to the SEALDD enabled transmission quality measurement procedure in clause 9.7.2.1, and detect whether the current transmission quality can satisfy the QoS requirements of VAL application. 5. The S-SEALDD server subscribes to 5GC for QoS monitoring of the specific UE related to the VAL user, as defined in clause 5.2.6.9 in 3GPP TS 23.502 6]. If the S-SEALDD server diagnoses that QoS deterioration is caused by N6/SEALDD overload (i.e., based on QoS monitoring between UE and UPF, and the E2E transmission quality measurement). The S-SEALDD server can determine to trigger the data transmission quality guarantee procedure (i.e., switching the connected SEALDD server according to SEALDD service policy) based on the QoS monitoring and E2E transmission quality measurement. NOTE 1: The QoS monitoring subscription request may be triggered after step 4 when the current transmission quality cannot satisfy the QoS requirements of VAL application. NOTE 2: This procedure cannot solve the QoS deterioration issue caused by NG-RAN (e.g. RAN congestion). 6. The S-SEALDD server performs target SEALDD server discovery procedure by using EEL, as specified in clause 9.4. 7. If tethering group ID is included in Sdd_RegularTransmission request. The S-SEALDD server sends tethering group information request to PIN server. The request includes tethering group ID. 8. PIN server sends response to S-SEALDD server. It includes tethering group information, tethering device identifier(e.g. VAL UE ID or UE IP address), tethered device identifier list(e.g.MAC address). 9. The S-SEALDD server can select the T-SEALDD server (i.e., SEALDD server #2) based on N6 traffic and/or SEALDD server load from performance of the available target SEALDD servers in step 6. 10. The SEALDD relocation procedure is performed for the switched same SEALDD servers and the switched same VAL servers for each application traffic in this tethering group respectively based on tethering group information, as specified in clause 9.6.2.2. After the SEALDD relocation procedure, the SEALDD client can connect to the selected T-SEALDD server to obtain the data transmission quality guarantee service (i.e. the QoS requirements of VAL application can be satisfied).

5.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End Change * * * *

<Proposed change in revision marks>