3GPP TSG-SA WG6 Meeting #68	S6-253046

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0384 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction of clause 7.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enh4MCPTT |  |  |  |  |  | Date: |  |  | 06.08.2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Short data communication is always one of three possible communication types: - Standalone data communication over signalling plane where a single SDS message is transferred via SIP MESSAGE over signalling plane. - Standalone session data communication over media plane where an single SDS message doesn’t fit into SIP MESSAGE and requires media plane for transmission. - Session data communication where several mutual SDS messages between MCData clients are transferred in an established session over media plane. Each data communication above is requested by dedicated information flows. Several information flows in clause 7.4.2.1 includes mandatory information element ‘Transaction type’ which is either for ‘Standalone transaction’ or for ‘Session based transaction’. Because of the information flow itself specify precisely the requested communication type this information element is not needed and can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information element ‘Transaction type’ shall be removed from information flows below: - 7.4.2.1.3 MCData standalone session data request - 7.4.2.1.5 MCData session data request - 7.4.2.1.10 & 11 MCData group session standalone data request - 7.4.2.1.13 & 14 MCData group data request |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause confusion or misinterpretation if ‘Transaction type’ is not removed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2.1.3, 7.4.2.1.5, 7.4.2.1.10, 7.4.2.1.11, 7.4.2.1.13, 7.4.2.1.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 7.4.2.1.3	MCData standalone session data request

Table 7.4.2.1.3-1 describes the information flow for the MCData standalone session data request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.3-1: MCData standalone session data request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID (see NOTE 1) | O | The identity of the MCData user towards which the data is sent |
| Functional alias (see NOTE 1) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 2) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Requested Priority | O | Application priority level requested for this communication. |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| NOTE 1: At least one identity shall be present. If both are present the MCData ID shall be used to route the request and the functional alias is just for information. NOTE 2: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

Table 7.4.2.1.3-2: MCData standalone session data request (MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

* * * Second Change * * * *

##### 7.4.2.1.5	MCData session data request

Table 7.4.2.1.5-1 describes the information flow for the MCData session data request sent from the MCData client to the MCData server and from the MCData server to another MCData client.

Table 7.4.2.1.5-1: MCData session data request (MCData client to MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ID (see NOTE 1) | O | The identity of the MCData user towards which the data is sent |
| Functional alias (see NOTE 1) | O | The associated functional alias of the MCData user identity towards which the data is sent. |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 2) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: At least one identity shall be present. If both are present the MCData ID shall be used to route the request and the functional alias is just for information. NOTE 2: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

Table 7.4.2.1.5-2: MCData session data request (MCData server to MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| MCData ID | O | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator | O | Indicates that the data request is for MCData emergency communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Application identifier (see NOTE) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

* * * Third Change * * * *

##### 7.4.2.1.10	MCData group session standalone data request (MCData client – MCData server)

Table 7.4.2.1.10-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData client to the MCData server.

Table 7.4.2.1.10-1: MCData group session standalone data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI, attached data hosts) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption or IP data in IP connectivity sessions are for data host consumption. |  |  |

* * * Fourth Change * * * *

##### 7.4.2.1.11	MCData group session standalone data request (MCData server – MCData client)

Table 7.4.2.1.11-1 describes the information flow for the MCData group session standalone data request (in subclause 7.4.2.6.2) sent from the MCData server to the MCData client.

Table 7.4.2.1.11-1: MCData group session standalone data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the MCData user towards which the data is sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI, attached data hosts) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption or IP data in IP connectivity sessions are for data host consumption. |  |  |

* * * Fifth Change * * * *

##### 7.4.2.1.13	MCData group data request (MCData client – MCData server)

Table 7.4.2.1.13-1 describes the information flow for the MCData group data request sent from the MCData client to the MCData server.

Table 7.4.2.1.13-1: MCData group data request (MCData client – MCData server)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| Requested priority | O | Application priority level requested for this communication session |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

* * * Sixth Change * * * *

##### 7.4.2.1.14	MCData group data request (MCData server – MCData client)

Table 7.4.2.1.14-1 describes the information flow for the MCData group data request sent from the MCData server to the MCData client.

Table 7.4.2.1.14-1: MCData group data request (MCData server – MCData client)

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData group ID | M | The MCData group ID to which the data is to be sent |
| MCData ID | M | The identity of the recipient MCData user |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Emergency indicator (see NOTE 1) | O | Indicates that the data request is for MCData emergency communication |
| Alert indicator (see NOTE 2) | O | Indicates whether an emergency alert is to be sent |
| Imminent peril indicator (see NOTE 1) | O | Indicates that the data request is for MCData imminent peril communication |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered or read or both) |
| Payload Destination Type | M | Indicates whether the SDS payload is for application consumption or MCData user consumption |
| Location | O | Location of the Originating MCData user sending the SDS message |
| Application identifier (see NOTE 3) | O | Identifies the application for which the payload is intended (e.g. text string, port address, URI) |
| Application metadata container | O | Implementation specific information that is communicated to the recipient |
| SDP offer | M | Media parameters offered |
| NOTE 1: If used, only one of these information elements shall be present. NOTE 2: This information element may be present only when Emergency indicator is present. NOTE 3: The application identifier shall be included only if the payload destination type indicates that the SDS message is for application consumption. |  |  |

* * * End of Change * * * *