3GPP TSG-SA WG6 Meeting #68	S6-253056

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-251xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0386 | rev | - | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Location information for ad hoc group data session request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Information Flow for Ad Hoc Group Data Session Request already define the Server→Client and Server→Server flow directions. However, unlike other data requests types in the table, which already include Location Information as optional, these flows do not include the optional Location Information. This gap leads to inconsistencies between data types. Adding Location Information ensures consistent availability across all data types and communication directions, thereby improving operational capability in situation-critical scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the Ad Hoc Group Data Session Request for Information Flow to include the optional Location Information attribute for: Ad Hoc Group Data Session Request: Server→Server and Server→Client |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not added, there will be an inconsistent with other data types regarding the availability of Location Information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.2.3 , 7.17.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.17.2.3	Ad hoc group data session request (MCData server – MCData server)

Table 7.17.2.3-1 describes the information flow ad hoc group data session request between the MCData servers.

Table 7.17.2.3-1 Ad hoc group data session request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID | M | The MCData group ID to be associated with the ad hoc group data communication |
| MCData ID list (see NOTE 1, NOTE 3) | O | MCData IDs of the participants being invited for the ad hoc group data communication |
| SDP offer | M | Media parameters of MCData server |
| Broadcast indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for a broadcast ad hoc group data communication |
| Imminent peril indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for ad hoc group imminent peril data communication |
| Emergency Indicator (see NOTE 2) | O | Indicates that the ad hoc group data communication request is for ad hoc group emergency data communication |
| Preconfigured MCData group ID | O | Group identity whose configuration is to be applied for this ad hoc group data communication. |
| Criteria for determining the participants (see NOTE 3) | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Requested priority | O | Application priority level requested for this group data communication |
| Location information | O | Location of the calling party |
| NOTE 1: This element is included only when the data communication initiating client sends the list of participants. NOTE 2: If used, only one of these information elements is present. NOTE 3: Only one of these information elements is present. |  |  |

Editor's Note: It is FFS if the server to server message is needed in a call request or response message.

# * * * * Second change * * * *

#### 7.17.2.4	Ad hoc group data session request (MCData server – MCData client)

Table 7.17.2.4-1 describes the information flow ad hoc group data session request from the MCData server to the MCData client.

Table 7.17.2.4-1 Ad hoc group data session request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending data |
| MCData ID | M | The identity of the MCData user towards which the request is sent |
| Functional alias | O | The associated functional alias of the MCData user sending data. |
| MCData ad hoc group ID | M | The MCData group ID associated with the ad hoc group data communication |
| SDP offer | M | Media parameters of MCData server |
| Broadcast indicator (see NOTE) | O | Indicates that the ad hoc group data communication request is for a broadcast ad hoc group data communication |
| Imminent peril indicator (see NOTE) | O | Indicates that the ad hoc group data communication request is for ad hoc group imminent peril data communication |
| Emergency Indicator (see NOTE) | O | Indicates that the ad hoc group data communication request is for ad hoc group emergency data communication |
| Preconfigured MCData group ID | O | Group identity whose configuration is to be applied for this ad hoc group data communication. |
| Call resulting criteria for determining the participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both that the MCData server used for determining the participants e.g., it can be a location based criteria to invite participants in a particular area |
| Location information | O | Location of the calling party |
| NOTE: If used, only one of these information elements is present. |  |  |

# * * * * End of changes * * * *