3GPP TSG-SA WG6 Meeting #68	S6-253109

Gothenburg, Sweden, 25th – 29th August 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0057 | rev |  | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor group membership update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current clause 8.3.7 Spatial anchor group management have only Spatial anchor group creation and Spatial anchor group deletion |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR add Spatial anchor group membership update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality of managing spatial anchors as group will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.7.2.x, 8.3.7.3.x, 8.3.7.3.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

8.3.7	Spatial anchor group management

8.3.7.2	Procedures

8.3.7.2.x	Spatial anchor group membership update

Figure 8.3.7.2.x-1 illustrates the spatial anchor group membership update procedure through which the requestor SAn client or the VAL server can request the SAn server to change the membership of a spatial anchor group (e.g. to add or delete spatial anchors).

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server; and

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

3.	For SAn client to trigger this procedure, the SAn client has received the required information from the VAL client over SAn-C interface.

Figure 8.3.7.2.x-1: Spatial anchor group membership update

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor group membership update request to the SAn server. The request as includes a requestor identifier, security credentials, identifier of the spatial anchor group, list of spatial anchors IDs affected by the add or delete operation.

2.	Upon receiving the request, the SAn server validates if the spatial anchor group exists and if the requestor is authorized to operate the spatial anchor group membership. If the requestor is authorized, the SAn server updates the spatial anchor group membership.

3.	The SAn server sends a spatial anchor group membership update response to the requestor.

# * * * * Next change * * * *

8.3.7.3	Information flows

8.3.7.3.x	Spatial anchor group membership update request

Table 8.3.7.3.x-1 describes information elements for the spatial anchor group membership update request from the SAn client or VAL server to the SAn server.

Table 8.3.7.3.x-1: Spatial anchor group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| Spatial anchor group ID | M | Identify of the spatial anchor group |
| List of spatial anchors IDs | M | List of spatial anchors IDs affected by this operation. |
| Operations | M | Add to or delete from the spatial anchor group |

8.3.7.3.y	Spatial anchor group membership update response

Table 8.3.7.3.y-1 describes information elements for the spatial anchor group membership update response sent by the SAn server.

Table 8.3.7.3.y-1: Spatial anchor group membership update response

| Information element | Status | Description |
| --- | --- | --- |
| Spatial anchor group ID | M | Identify of the spatial anchor group |
| Result | M | Indicates the success or failure for the operation |

# * * * * End of changes * * * *