3GPP TSG-SA WG6 Meeting #68	S6-253114

Gothenburg, Sweden 25th – 29th August 2025

Source:	Huawei, Hisilicon

Title:	Key issues: how to enhance DAA by utilizing sensing results

Spec:	3GPP TR 23.700-15 v0.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	Lei Chen (chenlei79@huawei.com)

1. Introduction

This contribution proposes a key issue for how to enhance DAA by utilizing sensing results.

2. Reason for Change

This contribution is to disscuss potential key issues of utilizing sensing results for UAV.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15.

* * * First Change * * * *

## 4.x Key issue #x: how to enhance DAA by utilizing sensing results

Detect And Avoid (DAA), which is to detect the potential collicsion between UAVs, is a key service of UAS to accomplish the safe and effient UAV management. Accorring to TS 23.255,  the detection of potential collicsion mainly denpends on the direct UAV to UAV communciaiton over PC5 and Local DAA Ground Station (LDGS) assitance in the current UAS. In addtion, the identification of UAVs depends on the PC5 communicaiton between UAV and UAV or LDGS. This causes that the DAA service must depends on the PC5 communication. However, the current situtation is that the PC5 communication is not large-scale commercially used in UAV industry and even some UAVs are not equiped with 3GPP UE module.

In 5G-Advanced, integrated sensing and communication will be supported by the network and it could provide sensing results to trusted 3rd party by network exposure. The sensing results includes objects detection, object recognition (e.g., vehicle, human, animal, UAV), high accuracy localication, tracking and activety recognition, accoring to TS 23.137. Some of sensing results could be used to enhance DAA to enable a more safe and efficient UAV management system.

However, the radio sensing only can detect an UAV as an object and it is hard to identify the UAV. So, it would make sense to study how to enhance DAA by utilizing sensing result. For this, at least the following aspects should be considered:

- To enhance UAE layer to support DAA service with considering sensing results

- UAE layer how to obtain sensing results to support DAA service

Editor’s note: UAE layer how to oabtain sensing results needs to wait  for SA2 progress

* * * END of CHANGE * * * *