3GPP TSG-SA WG6 Meeting #68	S6-253116

Gothenburg, Sweden 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.438 | CR | 0014 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update digital assets authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-App |  |  |  |  |  | Date: |  |  | 2025.08.12 |
| Category: | F |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A digital asset of user may be generated from various business parties. These parties can endorse the authenticity of the asset, so they need to issue the asset with a digitally signed signature which will be used by the asset's consumers to verify its authenticity. However, current standards only specify how assets are obtained, but do not provide digital asset authenticity verification service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add additional security parameters in DA profile and corresponding authentication procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authenticity and validity of digital assets cannot be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 7.2, 8.2.3.1, 8.3.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***  Start of 1st CHANGES ***

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Digital asset profile: It is a digital asset specific configuration and parameters (e.g. allowed locations) applicable to one or more application(s). The digital asset profile may be associated with one or more VAL user(s). In this release, avatar is considered as a digital asset.

Digital asset identifier: A digital asset identifier is used to uniquely identify a digital asset across different mobile metaverse services.

For the purposes of the present document, the following terms given in 3GPP TS 22.156 [2] apply:

Avatar

digital asset

Digital assert issuer：An entity, e.g. bussiniess party, which is responsible for endorsing the authenticity of a digital assert, the endorsement can be achieved through the issuer cryptographically signing the user's digital assets with a signature.

***  End of 1st CHANGES ***

***  Start of 2nd CHANGES***

## 7.2	Digital asset profile

The digital asset profile includes the information elements about the attributes of a single digital asset. Table 7.2-1 shows the information elements for the digital asset profile.

NOTE 1: In this release, the digital asset profile is specified considering Avatar as a digital asset of the user.

Table 7.2-1: Digital asset profile

| Information element | Status | Description |
| --- | --- | --- |
| Digital asset ID | M | Identifier of the digital asset profile |
| Digital asset type | M | Type of the digital asset. (For this release, it contains "Avatar"). |
| List of owners | M | List of identity of the owner user(s) who created the digital asset. |
| Issuer ID | O | Identifier of the digital asset issuer |
| List of Metaverse service provider identifiers | O | List of service providers for the digital asset. |
| access control list | O | List of tuples indicating the requestors allowed to perform operations with this digital asset and the allowed operation types. |
| > allowed user list | O | List of users allowed to access this digital asset |
| > allowed operations | O | List of operations allowed by this user. Possible operations are (one or more of) : Discovery, Get, Update, Delete |
| > list of predictive models | O | For each allowed user, list of one or more predictive model for the user, in order to predict user's behaviour. |
| current status | O | Current status (in use or not) of the profile |
| spatial conditions | O | List of spatial conditions (e.g. locations) where this digital asset profile is allowed to be accessed |
| > current location | O | Current location where the profile is being used |
| allowed application list | O | List of application IDs which are allowed to use this digital asset profile |
| associated accessories | O | List of accessories (e.g. Hat, watch, shoes) which are purchased for this digital asset profile |
| expiry time | O | Time until this digital asset profile is valid |
| History information | O | Usage history for this digital asset profile |
| > list of locations | O | List of locations where the digital asset profile is used. |
| > time of use | O | Time information when the digital asset profile is used |
| > list of applications | O | List of the application IDs which used this digital asset profile. |
| Digital asset media information | O | Information related to digital asset media (e.g. URI where the media is stored) |
| Issuer’s signature | O | The signature cryptographically signed by the digital asset issuer on the asset, such as the signature of the operator or the copyright issuer. |

***  End of 2nd CHANGES ***

***  Start of 3rd CHANGES***

#### 8.2.3.1	Digital asset profile create request

Table 8.2.3.1-1 describes the information elements for a digital asset profile create request sent from a requestor (e.g. DA client) to a DA server.

Table 8.2.3.1-1: Digital asset profile create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset profile parameters | M | Parameters to create digital asset profile |

Table 8.2.3.1-2: Parameters for digital asset profile

| Information element | Status | Description |
| --- | --- | --- |
| Digital asset owner identifier(s) | M | A list of identifier(s) for the digital asset owner. |
| Digital asset type | M | The type of digital asset, e.g. avatar, wallet, etc. |
| Issuer ID | O | Identifier of the digital asset issuer |
| List of Metaverse service provider identifiers | O | List of service providers for the digital asset. |
| access control list | O | List of tuples indicating the requestors allowed to perform operations with this digital asset and the allowed operation types. |
| > allowed user list | O | List of users allowed to access this digital asset |
| > allowed operations | O | List of operations allowed by this user. Possible operations are (one or more of) : Discovery, Get, Update, Delete |
| > list of predictive models | O | For each allowed user, list of one or more predictive model for the user, in order to predict user's behaviour. |
| spatial conditions | O | List of spatial conditions (e.g. locations) where this digital asset profile is allowed to be accessed |
| > current location | O | Current location where the profile is being used |
| allowed application list | O | List of application IDs which are allowed to use this digital asset profile |
| associated accessories | O | List of accessories (e.g. Hat, watch, shoes) which are purchased for this digital asset profile |
| expiry time | O | Time until this digital asset profile is valid |
| Digital asset media information | O | Information related to digital asset media (e.g. URI where the media is stored) |
| Issuer’s signature | O | The signature cryptographically signed by the digital asset issuer on the asset, such as the signature of the operator or the copyright issuer. |

***  End of 3rd CHANGES ***

***  Start of 4th CHANGES***

### 8.3.2	Procedure

Figure 8.3.2.1-1: Digital asset discovery

1.	A DA client sends a digital asset discovery request to the DA server. The request includes information listed in Table 8.3.3.1-1.

2.	The DA server authenticates and authorizes the request. If authorized, the DA server applies the discovery criteria to discover digital assets from the local repository.

3.	The DA server generates and sends a response to the requestor. The response includes information listed in Table 8.3.3.2-1.

4.  The DA client receives the response and authenticates the issuer's signature within the digital asset profile using the issuer's public key.

***  End of 4th CHANGES***