3GPP TSG-SA WG6 Meeting #68	S6-253133

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-252233)

Source:	Fogus, Apple, Telefonica, UMA

Title:	Pseudo-CR CAPIF_Ph4 Key Issue Realising the GSMA OP CER

Spec:	3GPP TR 23.700-43

Agenda item:	9.11

Document for:	Agreement

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

The CAPIF phase 4 SID includes an objective: Support for new CAPIF enhancements identified from SDOs/industry forums such as GSMA, ETSI OpenCAPIF, etc.

2. Reason for Change

This discussion paper proposes a Key Issue based on findings from ETSI OpenCAPIF, specifically relating to providing support for API providers beyond those from the 3GPP network.

3. Conclusions

n/a.

4. Proposal

It is proposed to agree the following key issue for inclusion in 3GPP TR 23.700-43.

* * * First Change * * * *

## X.x	Key issue #X: Realising the GSMA OP CER

### X.x.1	Description

The CAPIF Core Function (CCF) as described in 3GPP TS 23.222 [X] supports exposure of service APIs. This can be leveraged by the Network Exposure Function (NEF) as an example of 3GPP specified northbound API offered functionality. The CCF service API exposure can also include APIs not specified by 3GPP and supports such extensibility through vendor-specific extensions, noting in this case that such extensions should not be intended to replace any existing 3GPP northbound API functionality.

Leveraging the 3GPP CAPIF specifications, the ETSI Software Development Group OpenCAPIF (SDG OCF) has been established to develop an open-source implementation of CAPIF, providing developers with a platform to expose and consume APIs in a secure and consistent way.

In addition, GSMA Operator Platform Group (OPG) has defined the Operator Platform (OP), which is a platform for exposing operator services/capabilities and edge compute resources to customers/developers in a standardized way. Deployment of the OP via CAPIF has been considered in 3GPP TS 23.946 [Y]. The OP describes a Capabilities Exposure Role (CER) [OPG.02 Operator Platform: Requirements and Architecture, Version 7.0, clause 3.2.2] which could be foreseen as offering functionality comparable to that offered by the CCF.

### X.x.2	Open issues

This key issue proposes to study potential enhancements to CAPIF to enhance support for API providers and invokers that refer to vertical specific API services, specifically focusing on potential CCF enhancement to address the Service Management requirements of the NBI that the GSMA OP foresees and defined in CAMARA API: Capabilities And Runtime Restrictions https://github.com/camaraproject/CapabilitiesAndRuntimeRestrictions (listed there as: Discover Capabilities and Constraints; Stay updated on changes; and Adapt your requests Automatically).

* * * End of Changes * * * *