3GPP TSG-SA6 Meeting #68	S6-253147

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0045 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modifications to data management assistance service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-08-16 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The client data processing trigger in AIML data management assistance is currently in request response form. After receieving the request from AIMLE server, the AIMLE client has to perform data operations which involve data preparation and data analysis. These process involves applying some functions on data to generate output. The operation may take good amount of time on UE if data is large. Under current architecture, an AIMLE server may need to keep connection open untill the client has processed the request, generate the output and sent it in response. Moreover, client data processing may get futile if connection between AIMLE server and client goes down during data processing. Here, its being proposed that client data processing trigger request/ response shall support notification feature in order to send client output. This change will not hamper message flows between AIMLE server and VAL server as their interaction for data management assistence is already following subscription model. The same pattern is used in ML model training request given in clause 8.3. - Adding “Notification target address” in table 8.15.3.1 on AIMLE data management assistance subscription request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modification to client data processing trigger request/response to support notification and notification target address in data management subscription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Connection will remain open on AIMLE server untill client has processed the request. In case of network error before client response, the processed output will not reach AIMLE server from client as connection was broken. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 8.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

## 8.15	AIMLE data management assistance

### 8.15.1	General

AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation and data analysis operations and send the outputs to the AIMLE server for aggregation.

The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance.

### 8.15.2	Procedure

Pre-conditions:

1.	AIMLE clients have registered with AIMLE server.

2.	UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10].

Figure 8.15.2-1: AIMLE data management assistance

1.	An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.15.3.1-1.

2.	The AIMLE server authenticates and authorizes the request. If authorized, the AMILE server assigns an identifier for the subscription.

3.	The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.3.2-1.

4.	The AIMLE server sends Client data processing trigger request to AIMLE clients. The request can be for data preparation or data analysis. The request includes information as described in Table 8.15.3.4-1.

5. The AIMLE client sends client data processing response to the AIMLE server with information as described in table 8.x.x.x.-1

6.	Each AIMLE client sends the requirements to trigger data management for the VAL client to perform the requested data operation. The VAL client performs the operation locally.

NOTE:	The data preparation and data analysis functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data as part of data preparation or data analysis in a similar manner as ML training.

7.	After the data operation completes, each AIMLE client sends an AIMLE client data processing trigger notification to the AIMLE server with information as described in Table 8.15.x.x.x.

8.	The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations.

9.	The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.15.3.3-1.

### 8.15.3	Information flows

#### 8.15.3.1	AIMLE data management assistance subscription request

Table 8.15.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure.

Table 8.15.3.1-1: AIMLE data management assistance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management operations | M | An indicator showing what data management type is being requested: data preparation, data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| >Data preparation requirements | O (NOTE 1) | Data preparation requirements as detailed in Table 8.15.3.1-2. |
| >Data analysis requirements | O (NOTE 1) | Data analysis requirements as detailed in Table 8.15.3.1-3. |
| AIMLE clients list | O (NOTE 2) | A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier. |
| AIMLE client selection criteria | O (NOTE 2) | Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided. |  |  |

Table 8.15.3.1-2: Data preparation requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Data preparation requirements | M | Requirements for data preparation. |
| > Dataset ID | M | The identifier for the dataset. |
| > Dataset feature ID | M | Identifier or name of dataset feature to process. |
| > Data preparation function | M | This indicates the function which prepares the data, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. |

Table 8.15.3.1-3: Data analysis requirements

| Information element | Status | Description |
| --- | --- | --- |
| Dataset identifier | M | An identifier for the dataset. |
| Dataset analysis requirements | M | Requirements for data analysis. |
| > Dataset ID | M | The identifier for the dataset. |
| > Dataset feature ID | M | Identifier or name of dataset feature. |
| > Data analysis function | M | This indicates the function which performs the data analysis, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request. |

#### 8.15.3.2	AIMLE data management assistance subscription response

Table 8.15.3.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure.

Table 8.15.3.2-1: AIMLE data management assistance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Subscription identifier | M | An identifier for the subscription. |
| Expiration time | O | Expiration time for the subscription |

#### 8.15.3.3	AIMLE data management assistance notify

Table 8.15.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure.

Table 8.15.3.3-1: AIMLE data management assistance notify

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Aggregated data preparation outputs | O (NOTE 1) (NOTE 2) | Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output. |
| Aggregated data analysis outputs | O (NOTE 1) (NOTE 2) | Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomaly values, and feature correlation information. |
| Timestamp | O | Timestamp of the data management notification |
| NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. |  |  |

#### 8.15.3.4	Client data processing trigger request

Table 8.15.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data processing trigger procedure.

Table 8.15.3.4-1: Client data processing trigger request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Data management type | M | An indicator showing what data management type is being requested: data preparation, data analysis. |
| Data management requirements | M | Requirements for the data management request: |
| >Data preparation requirements | O (NOTE) | Data collection requirements as detailed in Table 8.15.3.1-2. |
| >Data analysis requirements | O (NOTE) | Data collection requirements as detailed in Table 8.15.3.1-3. |
| Operational schedule | O | A schedule to perform the requested data management operation. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| NOTE: At least one of the information elements shall be provided. |  |  |

#### 8.15.3.5	Client data processing trigger response

Table 8.15.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure.

Table 8.15.3.5-1: Client data processing trigger response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training request was successful. |
| Failure response | O (NOTE) | Indicates that the ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present |  |  |

#### 8.15.3.6	Client data processing trigger notification

Table 8.15.3.6-1 shows the notification sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure.

Table 8.15.3.6-1: Client data processing trigger notification

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the data management operation |
| Data preparation outputs | O (NOTE 1) | The output data after performing data preparation. One output data is generated for each requirement. (NOTE 2) |
| Data analysis outputs | O (NOTE 1) | The output data generated by data analysis. One output data is generated for each requirement. (NOTE 2) |
| Timestamp | O | Timestamp of the data management operation |
| NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data. |  |  |

* * * End of Changes * * * *