3GPP TSG-SA WG6 Meeting #68	S6-253159

Goteborg, Sweden 25th – 29th August 2025	(revision of S6-252122)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0459 | rev | 2 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for combining multiple ad hoc group call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause introduces a new procedure to combine multiple ongoing ad hoc group calls upon the request of an authorised MCPTT user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new procedure is included to combine multiple ad hoc group calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requirements are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.3.1.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.19.3.1.7	Combining multiple ad hoc group calls

This clause describes the procedure to combine multiple ad hoc group calls. Multiple ad hoc group calls may be combined upon the request of an authorised MC service user.

NOTE:	The upper limit of the number of ad hoc group calls to be combined is up to implementation.

Figure 10.19.3.1.7-1 describes the procedure for combining multiple ad hoc group calls. The procedure is applicable to more than two ongoing ad hoc group calls to be combined.

The procedure is applicable to ad hoc group calls irrespective how they are set up, i.e., whether the initiated (authorized) MC service user selects the list of participants as in clause 10.19.3.1.1, or the MC service server selects the list of participants based on criteria as in clause 10.19.3.1.3. The resulting, i.e., the combined ad hoc group MCPTT call is then a new ad hoc group based on the individual ad hoc groups.

Pre-conditions:

- The MCPTT user 1 (authorised user) is participating in an ongoing ad hoc group call #1 with MCPTT user 2, …, MCPTT user n.

- The MCPTT user 1 is participating in an ongoing ad hoc group call #2 with MCPTT user n+1, …, MCPTT user m.

Figure 10.19.3.1.7-1: Combining multiple ad hoc group call procedure

1.	The MCPTT user at MCPTT client 1 wishes to combine multiple going ad hoc group MCPTT calls. MCPTT client 1 sends an ad hoc group call combine request towards the MCPTT server. The request includes among others the identities of the ad hoc group calls to be combined.

2.	The MCPTT server checks whether the MCPTT client 1 is authorised to combine multiple ongoing ad hoc group calls. If the MCPTT client 1 is authorised, the MCPTT server proceeds with the next step.

3.	The MCPTT server combines the MCPTT users of the individual ad hoc groups into one by creating a new ad hoc group call that consists of the multiple ongoing ad hoc groups included in step 1. Furthermore, it assigns a new ad hoc group ID associated with the combined, i.e., new ad hoc group.

4.	The MCPTT server sends the MCPTT client 1 an ad hoc group call combine response including the identity of the ad hoc group associated with the combined ad hoc groups.

5.	The MCPTT client 1 sends an ad hoc group call request towards the MCPTT server. The request includes the identity of the ad hoc group associated with the combined ad hoc groups.

6.	The MCPTT server checks whether MCPTT client 1 is authorised to send ad hoc group call request. If the authorisation is successful, the MCPTT server proceeds.

7.	The MCPTT server sends an ad hoc group call request return to the MCPTT client 1

8a.-8b.	The MCPTT server sends ad hoc group call requests to the MCPTT clients which were determined in step 3 to participate in the combined ad hoc group call.

9a.-9b.	The target MCPTT clients notify the corresponding MCPTT users about the ad hoc group call.

10a.10b.	The MCPTT clients send the MCPTT server an ad hoc group call response.

11.	The MCPTT server sends ad hoc group call response to MCPTT client 1.

12.	The MCPTT server may send ad hoc group call notify to MCPTT client 1.

13.	Media plane and floor control associated with the combined ad hoc group call is established.

14.	MCPTT server initiates ad hoc group call release procedure for the ad hoc group call #1 and 2 according to clause 10.19.3.1.2.

Any changes to the participating MCPTT users in each individual ad hoc group will have an impact on the participating MCPTT users in the combined ad hoc group, i.e., if an MCPTT user is added to (or removed from) one individual ad hoc group, the MCPTT user will be added to (or removed from) the combined ad hoc group.

# * * * * End of changes * * * *