3GPP TSG-SA WG6 Meeting #68	S6-253174

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

Source:	Jio Platforms

Title:	Enhancements to AIMLE client registration and selection

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

The AIMLE server assigns an AIML task to UE which include model and operations. The client registers with its availability schedule. If a task is assigned to an AIML client, the client may update its registration with new availability schedule. In case of multiple task configuration on client, the client may update its operational schedule only if the scheduled AIMLE task can consume all available resources on UE. However, for small models with little computation and growing compute at UE, an AIMLE client may be capable of processing multiple AIMLE tasks at same time. Under such circumstances, a client must indicate about its partial availability related to percentage of resource available on AIMLE client to accept a new AIMLE task during a time window. The client can achieve this by updating its registration once it accepts participation in a particular AIMLE task. Percentage of resource available on UE/AIMLE client during desired time window will also allow AIMLE server to select an AIMLE client to run a desired model i.e. a low compute intensive model may consume less resources on AIMLE client, leaving room for other AIMLE tasks.

2. Reason for Change

To enhance client selection procedure in case multiple AIMLE tasks are required to be sceduled on a UE. Currently, there is no mechanism where UE can indicate its percentage availability so that it can be selected for multiple AIMLE task at same time. The registeration request only contains static max capabilites of client not the real time capability. Hence, standadized IE is required to define incremental remaining resource availability on AIMLE client.

3. Conclusions

This paper proposes solution to allow a client to update its percentage resource availability during a time window. This enhances AIMLE client selection by AIMLE server as multiple tasks can be scheduled on AIMLE clients based on resources available with them in real-time.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0.

# *** First Change ***

## 7.XX	Enhancements to AIMLE client registration and selection

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

AIMLE clients being capable of handeling multiple tasks at same time may inform about their percentage availability whenever they accept a new task. This feature lets and AIMLE server decide if an AIMLE client can be seleceted for an AIMLE task based on percentage of resource it can dedicate for any new AIMLE task. If the resources are not sufficient, AIMLE server can choose a different AIMLE client. Without this feature, the UE may or may not update its availability schedule depending upon the allocated task (e.g. a less compute intensive task may consume less resource on AIMLE client and it may not update its availability. However, the AIMLE server may perceive the UE as 100% available for AIMLE task which is incorrect.)

#### 7.XX.1.2	Impact to existing AIML client registration

Table 8.7.3.2-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week. |
| AIMLE client percentage availability schedule | O | Indicates client’s real-time availability or percentage availability in terms of percentage of resource available for new AIML task during multiple time schedule. E.g. (time: 13:00 – 15:00 available: 80%). |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

#### 7.XX.1.2	Impact to existing AIML client selection

## 8.9	AIMLE client selection

### 8.9.1	General

The AIMLE client selection procedure has two modes of operation: VAL server selection and AIMLE server selection. In VAL server selection, a List of AIMLE client IDs is provided to form an AIMLE client set, which can be used for AI/ML operations (e.g., training). In AIMLE server selection, a VAL server provides AIMLE client selection criteria and number of the required AIMLE clients for the AIMLE server to select the AIMLE client set.

The following clauses specify procedures, information flows, and APIs for AIMLE client selection.

### 8.9.2	Procedure

#### 8.9.2.1	AIMLE client selection

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with the AIMLE clients.

Figure 8.9.2.1-1: AIMLE client selection

1.	A VAL server sends an AIMLE client selection request to an AIMLE server to select AIMLE clients available for participation in AI/ML operations (e.g., is available and have required data to train an ML model). The AIMLE client selection request includes information as described in Table 8.9.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to select AIMLE clients.

3.	If the requestor is authorized, the AIMLE server performs AIMLE client selection based on the provided inputs as described below.

For VAL server selection, the AIMLE server receives a list of AIMLE client IDs in the request and selects the AIMLE clients in the list as candidate AIMLE clients.

For AIMLE server selection, the AIMLE server retrieves a list of clients with AI/ML capabilities from the ML repository. From the list of clients, the AIMLE server selects a list of candidate AIMLE clients, whose client profiles fulfil the AIMLE client selection criteria. The AIMLE server also assess ML model compute requirements and percentage of free resources available in AIMLE client as per its profile for AIMLE client selection. The AIMLE server may use SEAL (LM service), NEF (e.g. MonitoringEvent API), and NWDAF (e.g. UE mobility analytics as defined in clause 6.7.2 of 3GPP TS 23.288 [2]) capabilities to assist the AIMLE client selection.

The AIMLE server determines UEs by using their identifiers to determine their location and then selects only those that fulfil the location requirements specified in the AIMLE client selection criteria. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

The AIMLE server may then determine the QoS parameters for the AIML traffic session between the requestor and the candidate AIMLE client(s) and configure the AI/ML traffic session(s) via SEALDD (Sdd_RegularTransmission API) or NEF services (AfSessionWithQoS API). 	The AIMLE server determines the application QoS parameters based on the VAL Service ID.

NOTE 1:	To determine the application QoS parameters based on the VAL service ID, the AIMLE server can use the VAL service ID associated AIMLE client QoS requirements in the AIMLE client selection criteria provided in step 1.

NOTE 2:	If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server can discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the AIMLE client selection response message.

NOTE 3:	The AIMLE server can reuse SEAL group management for any necessary group management.

4.	The AIMLE server performs AIMLE client participation procedure with each candidate AIMLE client as described in clause 8.10.

For all candidate AIMLE clients that agreed to participate in AI/ML operations, the AIMLE server selects the AIMLE clients and assigns an AIMLE client set identifier for the selected clients. The AIMLE client set may then be used for training a ML model.

NOTE 4:	If a required minimum number of AIMLE clients for AI/ML operation is provided in the AIMLE client selection request and the number of AIMLE clients that agreed to participate in AI/ML operations is less than this number, an AIMLE client set identifier will not be assigned and the status will be set to fail in the AIMLE client selection response in step 5.

5.	The AIMLE server sends an AIMLE client selection response that includes information in Table 8.9.3.2-1.

# *** End of Changes ***