3GPP TSG-SA WG6 Meeting #68	S6-253176

Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

Source:	Jio Platforms

Title:	Enhancements to AIML training by inclusion of training mode

Spec:	3GPP TR 23.700-83

Agenda item:	9.4

Document for:	Approval

Contact:	Raghvendra Bhushan <Raghvendra.Bhushan@ril.com>

1. Introduction

The Key Issue#6, on the study aspects on support enhancements related to diverse AIMLE deployments scenarios includes:

- Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

The AIMLE server aggregates training data from multiple AIMLE clients. This aggregation can be in two modes i.e. synchronous and asynchronous. In synchronous mode, all AIMLE clients sends data in same time duration and AIMLE server wits for all device to send their data and then performs aggregation of all device training data before proceeding to next global aggregation. While, in case of asynchronous mode of training, the AIMLE client send their updates to server whenever they complete their training or AIML task iterations. The server merges the updates to AIML model as soon as the client update arrives and do not wait for all client to send their data for merging to global model.

Both aggregation mechanisms have their own Pros and Cons. Like a slow client may delay model training in case of synchronous training mode (straggler problem). While, in case of asynchronous training mode, the updates of slow client may be merged to global model with low weight and treating the late updates as stale updates. On the other hand, these training modes extends different flexibility to AIMLE clients.

The AIMLE client can dedicate all its resources for synchronous training in order to complete it on time. While, the client can perform asynchronous training in relaxed mode and performing multiple AIML tasks along with Asynchronous training.

2. Reason for Change

ML models can be trained in two modes i.e. synchronos and asynchronous. Each mechanism has its own benefits. Certain models require training in either modes strictly. But, the training mode declaration by a VAL server is not present in current version of TS 23.482. This information will let AIMLE server to merger training data to ML model in required manner and it can help AIMLE client too by allowing them to manage their resource allocation for a AIML task and availability schedule.

3. Conclusions

This paper proposes enhancements to ML training by introducing training modes i.e. synchronous and asynchronous modes of aggregating training parameters sent by AIMLE client to ML model.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.2.0 and include it in TS 23.482.

* * * First Change * * * *

## 7.XX	Introduction of AIMLE Training Mode in ML Model Profile and Client participation Request

### 7.XX.1	Solution Description

#### 7.XX.1.1	General

As described in 3GPP TS 23.482, the VAL server requests the AIMLE server for the ML model training. VAL server requests the AIMLE server to train a ML model by specifying the type of learning, details about the ML model to be trained or requirements to select a ML model for training and criteria for selecting the members who can participate in the ML model training

However, the training mode is not defined for this request. The training mode can be synchronous or asynchronous on basis of multiple parameters like past experience on accuracy, stability, data distribution, device availability, network availability, number of AIMLE clients, AIMLE server capability, Application cruciality (emergency services) or task urgency etc. the AIMLE server needs to explore its capability to support such training and respond to VAL client accordingly. The AIMLE server shall also inform AIMLE clients about training mode so that the client can schedule multiple tasks accordingly.

#### 7.XX.1.2	Impact to existing ML Model training Request

### 8.3.2	Procedure for ML model training

Figure 8.3.2-1 illustrates the procedure for AIMLE server to support ML model training based on the request from the VAL server.

Figure 8.3.2-1: ML model training

1.	The VAL server sends an ML model training request to AIMLE server, requesting to assist in its ML model training. This request consists of ML model information or ML model requirement information, etc.

2.	The AIMLE server checks whether the VAL server is authorized to perform the ML model training request. If no model information is provided but only the model requirement information is provided in step 1, the AIMLE server identifies and selects the appropriate ML model for training based on the ML model requirement information using the procedure as specified in clause 8.11.3.

3.	If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indicating the reason for failure.

4.	The AIMLE server also checks if it can support the training mode for aggregating output of multiple clients as per requested training mode. If AIMLE server supports training mode then it will return success response, otherwise a failure response indicating the reason for failure.

4.	The AIMLE server notifies the VAL server to update the list FL/ML clients selected or de-selected for the ML model training or to share the training output or any errors during the training process.

### 8.3.3	Information flows

#### 8.3.3.1	ML model training request

Table 8.3.3.1-1 describes the information flow from the VAL server to the AIMLE server for requesting the ML model training.

Table 8.3.3.1-1: ML model training request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE 1) | List of member clients to be utilized for training the ML model. |
| Member selection criteria | O (NOTE 1) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| ML model information | O (NOTE 2) | Identifies the ML model that has to be distributed to the selected member clients for training. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model requirement information | O (NOTE 2) | Identifies the requirement for selecting a model to be trained and this information contains the filtering criteria for selecting the model as specified in Table 8.11.4.1-2. |
| ML Model Training Mode | O | Identifies the training mode and aggregation of training data to be done in synchronous manner or asynchronous manner. |
| Members update notification | O | Indicates whether requestor needs to be notified whenever there is update related to new member clients selected or de-selected. |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| NOTE 1: At least one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

#### 8.3.3.2	ML model training response

Table 8.3.3.2-1 describes the information flow of ML model training response from the AIMLE server to the VAL server.

Table 8.3.3.2-1: ML model training response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE) | Indicates that the ML model training request was successful. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE server for training. |
| Failure response | O (NOTE) | Indicates that the ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: At least one of these information elements shall be present |  |  |

Note: In case, AIMLE server do not supports requested training mode then the failure response shall indicate VAL server about unsupported training mode.

* * * Second Change * * * *

#### 7.XX.1.3	Impact to existing AIML Client Participation request

### 8.10.3	Information flows

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| ML model ID | M | The ML model ID to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| ML model training mode | O | Identifies the training mode and aggregation of training data to be done in synchronous manner or asynchronous manner. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

#### 8.10.3.2	AIMLE client participation response

Table 8.10.3.2-1 shows the response sent by the AIMLE clients to the AIMLE server for the AIMLE client participation procedure.

Table 8.10.3.2-1: AIMLE client participation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| Status | M | The status for the request: success or failure. |
| Cause | O | The cause for the request failure. |

Note: The client can validate if it is available to perform training in synchronous or asynchronous mode during the requested time duration.

* * * Third Change * * * *

#### 7.XX.1.3	Impact to existing ML Model Information

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| ADAE Analytics ID | O | Represents ADAE analytics ID for which the model can be used. |
| ML model size | O | Indicates the size of the ML model. |
| ML model source identifier | O | The identifier of ML model source (e.g., VAL server ID, VAL client ID) that stored the model in the ML repository. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location prediction, etc.). |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to use the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, input/output parameters of the ML model, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed performance | O (NOTE 2) | Provides information on the performance of the model e.g. accuracy, or application-specific performance metrics (if ML model is in trained or deployment phase). |
| > Training information | O (NOTE 2) | If the ML model is in trained or deployed phase: Information on the data that has been used to train the model (e.g. data sources, volume, freshness), and the base model ID in case of Transfer Learning. |
| > Indication of continuous model training | O | Indicates whether the model can be continuously trained or not. |
| > Continuous model training parameter | O | Parameters required for continuous model training. |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| ML Model Training Mode | O | Identifies the training mode and aggregation of training data to be done in synchronous manner or asynchronous manner. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Security and access requirements | O | Represents the information on security requirements for storing the ML model information and the ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs or VAL client IDs, time period and location access). If the access requirement is private use only, then the model is not discoverable by other consumers. |
| ML model usage requirements | O | Represents the requirements for using the ML model (e.g. for inference or for training). The requirements are used by the AIMLE server to determine whether an AIMLE client is capable of using the model based on comparing the requirements with information in the AIMLE client profile in Table 8.7.3.1-2. |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

# *** End of Changes ***