3GPP TSG-SA WG6 Meeting #68	S6-253180

Stor-Göteborg, Sweden 25th – 29th August 2025

Source:	Nokia

Title:	Pseudo-CR on updating Adoption Solution 5

Spec:	3GPP TR 23.700-35 v1.0.0

Agenda item:	9.3

Document for:	Approval

Contact:	Rajesh Babu Natarajan rajesh_babu.natarajan@nokia.com

- Introduction

Adoption solution #5 was proposed to address adoption gap #4, to highlight the target application use case of SEAL services.

- Reason for Change

Table in the clause 8.2.5.1.2 is not complete and needs to be updated with more information.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v1.0.0.

* * * First Change * * * *

##### 8.2.5.1.2	Overall introduction of SEAL services

This solution is to address the overall summary of the values and beneficial use case(s) of each SEAL service.

The supporting of the following are the key services that make SEAL layer distinct with other non-3GPP basis developer API Platform:

-	NSCE/NRM/SEAL-DD which are specific to the usage of 5G/4G network capabilities.

-	E-MMTel enabler which is specific the usage of IMS system capabilities.

-	LMS utilize both 3GPP based position and non-3GPP based position.

-	AI/ML enabler could support AI/ML service related to mobile UE, leveraging on the 3GPP network-for-AI capabilities

| Service type | Service description | Benefits to application provides |
| --- | --- | --- |
| NRM | - Provide value added APIs by abstrating the capabilties served by core network layer entities MB-SMF/PCF/NEF APIs/SCEF/BMSC, to enable third party to use 3GPP QoS service in a simple way | Simply the usage of App providers (APP deveolper, APP owner) of 3GPP network capabilities to: - Improve qualitiy of application layer communication by using NRM API to tigger the 3GPP network connection with QoS supporting, compared to application layer communication over best effort 3GPP network connection. |
| SEAL DD | - Provide Caching and Distribution between application content server and mobile UE by using various 3GPP network capabilities designed for different type of application data to optimization purpose. For example reliable transmission mechanism specific to URLLC data. | Futher improve QoE of the applications when using Caching and Distribution service, by supporting the usage of 3GPP network capabilities(including control plane and user plane) for QoS optimization |
| NSCE | - Provide the API services to enable third party to retrive the slice SLA requirement automaticly, order slice product from MNO system, mornitor/change slice SLA, by aggregating the API services from OAM and Core network, SEAL layer services (service KQI information) | Provide the required functions(e.g deriving and recomending slice SLA to let a slice customer e.g., enterprise knows how much resource is required for there services) to enable third party to perform self-management operation on their dedicated slice, when NaaS is provided by operator OAM. |
| AI/ML enabler | AIMLE | AIMLE provides APIs for enabling and managing AI/ML services. It leverages the network capabilities of 5G systems to support secure and efficient collaboration, model lifecycle management, and distributed intelligence. |
| LMS | LMS provides location management related functionality and location based services to the 3rd party applications. It utilizes the core network related location capabilities and value add on top of it in-order to provide a enriched location services. | Availability of enriched Location information from core network along with the GPS co-ordinates. provides readily available supplementary location based services such as geofencing, location deviation monitoring etc., apart from reporting and fetching of location information. |
| Group management | Group management service provides the group management related operations considering the needs of multiple vertical applications. Apart from regular CRUD operations it provides the value add services like location based group creation, group regrouping, group member registration etc. | Re-usability : Single group management framework catering to multiple vertical needs easing the development time. |
| Configuration management | TBD | TBD |
| Digital asset | Provides application enablement to Metaverse application by supporting following functionalities: - Digital asset profile management: which includes creating, updating, retrieving and deletign the digital asset profile - Digital asset discovery - Digital asset media management – which includes uploading and downloading the digital asset media. | Capabilities to re-use the digital asset across multiple Metaverse applications. |
| Spatial Anhor enabler and spitial mapping enabler | Offers the platform for managing the spatial anchors and spatial maps for metaverse services. Enables the re-use of spatial anchors and spatial maps across multiple verticals and thus improving the metaverse experience for the end users. | Capability to re-use the spatial anchors and maps across multiple verticals. Provides the insight on the spatial anchor usage to improve the effeciency |

* * * End of Change * * * *