3GPP TSG-SA WG6 Meeting #68	S6-253234

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0155 | rev | - | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to L4S Feedback capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.433 defines the L4S feedback capability as “Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request” which is not clear about what the IE actually represents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the definition of the L4S Feedback capability IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous definition leads to misinterpretations and implementation mistakes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.2.3.3	SEALDD regular transmission connection establishment request (Client to Server)

Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the regular SEALDD connection establishment.

Table 9.2.3.3-1: SEALDD regular transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side used to establish SEALDD connection. |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| Capability for BAT and periodicity adaptation | O (See NOTE 2) | Indicates BAT and periodicity adaptation capability of the SEALDD client side (for client side initiated request) per SEALDD traffic descriptor. |
| Transmission assistance info | O (See NOTE 2, NOTE 4) | Indicates transmission assistance information for uplink SEALDD traffic of the SEALDD client side (for client side initiated request). It includes BAT, BAT window, periodicity, and periodicity range per SEALDD traffic descriptor. |
| L4S feedback capability | O (See NOTE 3) | Identifies the capability of ECN code point marking for L4S support for client side initiated request. It indicates the capability of the client to adjust the congestion control algorithm based on the ECN Congestion Experience feedback received in the incoming packets. |
| VAL UE client access capability | O | Indicates the access technologies supported by VAL UE client. E.g.,3GPP, Non-3GPP(WLAN) |
| XR Application device capability information | O (See NOTE 5) | Indicates XR Application device capability information (e.g., encoding capabilities) |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: If provided, BAT window and periodicity range are mutually exclusive with capability for BAT and periodicity adaptation. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. NOTE 4: The periodicity range may only be present together with the periodicity when BAT and BAT window are present. The BAT window may only be present together with the BAT. NOTE 5: This IE is used for the SEALDD enabled XR traffic data delivery. |  |  |

* * * End of Changes * * * *