3GPP TSG-SA WG6 Meeting #68	S6-253xxx

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253xxx)

Source:	Telefónica, Apple

Title:	TR 23.947 Intro

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	david.artunedoguillen@telefonica.com

1. Introduction

This contribution provides the scope text for the Guidelines for 3GPP Application Enablement usage (AppEnable_EXT) technical report.

2. Reason for Change

This contribution proposes text for the Introduction clause of 3GPP TR 23.947.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947.

* * * First Change * * * *

# Introduction

Emerging service verticals such as connected vehicles, unmanned aerial vehicles (UAVs), and smart factories require low-latency, high-bandwidth, and context-aware services delivered close to the end-user. 3GPP has developed several frameworks to support these needs:

- CAPIF (TS 23.222) defines a common and secure way to expose and consume network and service APIs across multiple domains and stakeholders.

- EDGEAPP (TS 23.558) specifies the architecture for enabling and orchestrating applications at the network edge, including discovery, context management, and mobility support.

- SEAL (TS 23.434) provides reusable service enablers that can be consumed by vertical applications via standardised APIs.

While these frameworks are specified separately, many advanced services will require their combined use. For example:

- A UAV management platform may rely on EDGEAPP for low-latency data processing, SEAL for geolocation and group communication, and CAPIF for secure API exposure to third-party control applications.

- A V2X service may integrate CAPIF for secure discovery of roadside assistance APIs, EDGEAPP for edge-hosted data processing, and SEAL for reliable multicast messaging.

This TR provides integration guidelines, reference deployment models, and developer-facing recommendations for building such combined deployments. While TR 23.946 provides guidelines for the use of CAPIF in 3GPP environments, this document focuses on its integration with EDGEAPP and SEAL to enable API-driven service exposure and orchestration across both edge and core domains.

* * * End of Changes * * * *